3GPP TSG-RAN WG3 #126								R3-247643

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	16.3

Source:	China Telecom

Title: 	Discussion on Signalling Procedure Based on Topology 1

Document for:	Discussion

Introduction

The study item on solutions for Ambient IoT (Internet of Things) in NR was agreed in RAN#102 meeting [1]. In the last RAN3 #125bis meeting, the signalling and procedures for Topology 1 were agreed and captured into TR 38.769. This contribution aims to discuss the content information in inventory and command.

Discussion

Content in Inventory for Topology 1

In RAN3#124 meeting, whether the following information for topology 1 need to be contained in inventory message was discussed:

- [M] AIoT Device ID MASK (to find a single device, a group of devices, or all devices)

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

We prefer to not introduce an additional device ID mask to find a group of devices as device ID may comprise of multiple bits like gNB-ID or cell ID. If the CN decide to find a group of devices, it can only send part of device ID bits to NG-RAN. The A-IoT device with the same part of device ID bits need to treat this inventory message. Therefore, it is need to specify a variable length of A-IoT device ID in inventory message. Regarding the inventory session ID, we support to introduce this session ID as specifications need to allow multiple Inventory procedure instances to run in parallel over RAN-CN interface. Considering the A-IoT RAN system may cover multiple regions in practical deployment, the inventory area list needs to be contained in Inventory message. The estimated number of devices can be used to enable CFRA in case of single device inventory. With this information, the RAN node can configure an appropriate configuration for CFRA and timing configuration. Additionally, A-IoT system may also perform periodic inventory to allow the newly coming AIoT devices to be discovered. The AF may further provide the inventory strategy information (e.g. inventory period) to perform periodic inventory without further explicit requests from the AF. The inventory strategy information can be configured via Inventory message or Reconfigure/Modified via other NGAP message.

In conclusion, the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Proposal 1: the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Content in Command message

In addition, since the Commands refers to an instruction (e.g., Read,Write, Disable) sent by an AF to an Ambient IoT Device, are end-to-end protected between AF and Ambient IoT Device, the AMF may include one or more Commands information in Paging message. Therefore, the Command Information with String type can be also included in Command message. Additionally, the usage of Command (e.g., Read, Write, Disable) is also need to be contained in Command Message.

Proposal 2: the Command Information with String type and the usage of Command are contained in Command Message.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Proposal 2: the Command Information with String type and the usage of Command are contained in Command Message.

Reference

- RP-234058,New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair)