3GPP TSG-RAN WG3 Meeting #126	R3-247651

Orlando, USA, 18-22 Nov 2024

Agenda Item:	11.3

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Coverage and Capacity Optimization

Document for:	Discussions & Approval

- Introduction

The following agreements have been made in RAN3#125bis:

- Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

- The Coverage Modification Cause reflects the predicted CCO issue.

- WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

- Introducing separate time information for future coverage state and predicted CCO issue.

- Time for future coverage state: The point in time when the future coverage state will be applied.

- Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

Based on that and the objectives [1] of Rel-19 WI on 'Enhancements for AI/ML in NG-RAN', we present our analysis of the normative work for the Coverage and Capacity Optimization. Additionally, the corresponding TP is included.

- Discussion

In [2], the predicted information exchange over interfaces to support AI/ML assisted CCO involves predicted CCO issues and future CCO state. Following is the potential impacts captured in the TR:

Xn interface:

Enhance existing procedure to collect information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

F1 interface:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU

-	Timing Information

2.1 Non-split architecture

For the legacy way, the NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. Therefore, over Xn interface, it is proposed to introduce Future CCO State IE and Associated coverage modification cause IE in the NG-RAN NODE CONFIGURATION UPDATE message.

Moreover, RAN3 has made the agreement that Future Coverage State and the associated Coverage Modification Cause can be transmitted via the XnAP NG-RAN NODE CONFIGURATION UPDATE message. In this meeting, we prefer to moving one step forward based on the existing agreement that: Future CCO State, generated by AI/ML function, can involve cell-level coverage state and SSB-level coverage state.

- The newly introduced Future Coverage Modification List IE can involve future cell coverage state and future SSB coverage state.

In addition, for the Predicted Coverage Modification Cause IE, it is related to the predicted CCO issues detected in NG-RAN node 1. In the legacy Coverage Modification Cause IE, it includes coverage, cell edge, and network energy saving. Since network energy saving is not related to CCO, for the current case, the predicted coverage modification cause IE only needs to include coverage and cell edge to indicate the reason for the coverage modification.

- The newly introduced Predicted Coverage Modification Cause IE is an enumerated type that includes coverage and cell edge.

Possible implementation:

9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| //Skipped omitted text |  |  |  |  |  |  |
| Future Coverage Modification List |  | 0..1 |  |  | YES | ignore |
| >Future Coverage Modification Item |  | 0..<maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be impacted. | - |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values Indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | - |  |
| >>Future SSB Modification List |  | 0..1 |  | List of SSB beams to be modified coverage. | - |  |
| >>>Future SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB to be modified. | - |  |
| >>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. | - |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reasons for the predicted coverage modification in NG-RAN node1. | - |  |
| >> Time for Future Coverage State | O |  | INTEGER (0..60) | Indicate the time when will this future coverage modification happen. | - |  |
| >> Time for Predicted CCO Issue | O |  | INTEGER (0..60) | Indicate the time when will this predicted CCO issue may happen. | - |  |

2.2 Split architecture

As described in TS 38.401 [3], following is the details of CCO in the case of split architecture.

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.

In the legacy approach, the gNB-CU detects CCO issues with affected cells and beams, signalling this to the gNB-DU via GNB-CU CONFIGURATION UPDATE. The gNB-DU then generates the new coverage states for the gNB-CU via GNB-CU CONFIGURATION UPDATE. With the introduction of AI/ML, the gNB-CU detects predicted CCO issues, allowing the gNB-DU to generate future CCO states and inform the gNB-CU accordingly as shown in the Figure 1.

Figure 1. CCO procedures in the case of split architecture

- Introduce the Predicted CCO Assistance Information IE in the gNB-CU CONFIGURATION UPDATE message from CU to DU, which includes the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE.

- Introduce the Future Coverage Modification List IE in the gNB-DU CONFIGURATION UPDATE message from DU to CU, which includes future cell coverage state and future SSB coverage state.

2.4 Suggested CCO state

The discussion is left to normative phase whether the gNB-CU generates the suggested future CCO state and sends it to the gNB-DU as assistance information. The deployment scenarios captured in the TR to support AI/ML based CCO:

The following solutions can be considered for supporting AI/ML-based CCO:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Therefore, the gNB-CU is the appropriate location for model inference. From our perspective, it is feasible for the gNB-CU to generate the recommended future CCO state and send it to the gNB-DU as assistance information. The gNB-CU can predict CCO issues and utilize the trained AI/ML model to forecast potential CCO states based on predicted CCO issues and its current and past coverage states. While the potential CCO states generated by the gNB-CU may not be as accurate as those produced by the gNB-DU due to the latter having more measurement data, this information serves as valuable assistance from the gNB-DU's perspective to help improve CCO state generation.

- gNB-CU can generate the suggested future CCO state and sends it to the gNB-DU as assistance information. The suggested future CCO state can transferred within the Predicted CCO Assistance Information IE in the gNB-CU CONFIGURATION UPDATE message from CU to DU.

2.3 Timing information

In our previous meeting, companies agreed to introduce two separate timing info for both Future CCO state and Predicted CCO issue. Hence,based on the legacy mechanism and our previous agreement, it is naturally to deduce that the time information for the Future CCO state and Predicted CCO issue may be carried by the same message carry their timing information accordingly in both split and non-split architecture.

- The separate time information related to Future CCO state and Predicted CCO issue may be carried and transmitted together in the messages that convey these two CCO information for both split and non-split architecture.

In Rel-18, it was agreed that the requested prediction time includes a specific point in time for which the prediction information is being requested, assuming that this point is in the reasonable future. Therefore, the timing information for the future CCO state and the predicted CCO issue should also fall within this reasonable future timeframe, in accordance with this principle.

- The timing information for the future CCO state and the predicted CCO issue should also fall within the reasonable future timeframe of 0 to 60 seconds.

2.4 Performance Feedback

Some companies provided views on whether NW needs to evaluate the Predicted CCO issue and/or Future CCO state by using performance feedback. Before RAN3 further discusses this issue, we  prefer to firstly check companies view on how to evaluate the performance of a Predicted CCO issue. More specifically, from our point of view, there is a conflict between a well predicted CCO issue and the performance evaluation.Lets use a critical example to further explain our view: gNB may preform the future coverage modification accordingly to avoid any negative performance impact if this gNB can ideally get the related predicted CCO issue. In other word, A perfect Predicted CCO issue can be used by gNBs to avoid this issue happen. But how to evaluate an issue if this issue can not happen? In this meeting, we prefer to make some clarification on how to evaluate a Predicted CCO issue by using the performance feedback, then

- It is proposed for RAN3 to clarify how to evaluate Predicted CCO issue by using performance feedback.

The corresponding TP to 38.423, 38.300 and 38.401 are provided in our other contributions this meeting.

- Conclusion

Following is our proposals:

Proposal 1The newly introduced Future Coverage Modification List IE can involve future cell coverage state and future SSB coverage state.

Proposal 2The newly introduced Predicted Coverage Modification Cause IE is an enumerated type that includes coverage and cell edge.

Proposal 3Introduce the Predicted CCO Assistance Information IE in the gNB-CU CONFIGURATION UPDATE message from CU to DU, which includes the Predicted CCO Issue IE and the Predicted Affected Cells and Beams IE.

Proposal 4Introduce the Future Coverage Modification List IE in the gNB-DU CONFIGURATION UPDATE message from DU to CU, which includes future cell coverage state and future SSB coverage state.

Proposal 5gNB-CU can generate the suggested future CCO state and sends it to the gNB-DU as assistance information. The suggested future CCO state can transferred within the Predicted CCO Assistance Information IE in the gNB-CU CONFIGURATION UPDATE message from CU to DU.

Proposal 6The separate time information related to Future CCO state and Predicted CCO issue may be carried and transmitted together in the messages that convey these two CCO information for both split and non-split architecture.

Proposal 7The timing information for the future CCO state and the predicted CCO issue should also fall within the reasonable future timeframe of 0 to 60 seconds.

Proposal 8It is proposed for RAN3 to clarify how to evaluate Predicted CCO issue by using performance feedback.

- Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 38.401