3GPP TSG-RAN WG3 #126	              R3-247669

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	18.2

Source:	CMCC

Title:	Discussion on supporting LP-WUS Indicating Paging Monitoring

Document for:	Discussion

- Introduction

Rel-19 NR LP-WUS/WUR WI was approved in [1], which states that in RRC IDLE/INACTIVE modes, it's observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI). We will discuss on the following objectives in this contribution.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In RAN2#125bis, RAN2 agreed that the PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In RAN3#125bis, We agreed the following agreements:

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

The last used cell and extra bits issues still need to be discussed.

- Discussion

2.1 LP-WUS subgrouping capability information

First of all, we want to make it clear that for CN assigned subgrouping, whether it is CN paging or RAN paging, the CN assigned subgroup ID for LP-WUS will be directly carried in the F1 paging message. This is itself a capability indication, that is, when the DU does not receive the CN assigned subgroup ID in the F1 paging message, it will not wake up the UE in this way. Therefore, CN assigned subgrouping does not require additional capability indication.

Proposal 1: CN assigned subgrouping does not require additional capability indication in F1 paging message.

The functions and operating logic of PEI and LP-WUS are very similar, therefore, for the question of whether UE_ID based subgrouping should be per UE or per cell, PEI can be used as a reference. In PEI, as far as we know, there are two levels of LP-WUS subgrouping capability information: first is UE level, that is, the UEID Subgrouping Support Indication IE included in UE Paging Capability IE in the PAGING message; second is cell level, that is, the PEI Subgrouping Support Indication IE included in the Paging Cell Item IEs IE in the PAGING message. It is the same UE capability for supporting PEI and UE_ID based PEI subgrouping.

Therefore, it is reasonable to introduce the same indication in LP-WUS, because the gNB-DU not only needs to know whether the UE supports LP-WUS, but also needs to know whether a cell supports LP-WUS function.

Proposal 2: Both UE level and cell level LP-WUS subgrouping capability information are required in F1 paging message.

2.2 Entry/exit condition for LP-WUS monitoring

RAN2#127bis reached the following conclusions:

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

It could be observed that the entry/exit conditions only include the measurement related threshold, which is more related to the lower layer, and the LP-SS measurements are only used by the UE for switching between MR and LR in the serving cell and not for cell selection/reselection. So it is reasonable for gNB-DU to decide the entry/exit condition.

Proposal 3: It is reasonable for gNB-DU to decide the entry/exit condition.

2.3 Last used cell issue

In last meeting we discussed whether the anchor gNB provides a new IE for CN assigned subgroup ID into the Xn paging message for RAN paging, and decided to postpone this agreement until RAN2 reaches a conclusion about that UE can use the LP-WUS outside the last serving cell.

But as far as we know, RAN2 has not started discussing this issue. And referring to PEI, the lastUsedCellOnly flag in SIB1 is configurable, and there is no mandatory constraint that PEI can only be used for the last used cell.

In case LP-WUS is used in any cell, it is beneficial for the UE because LP-WUS can always be used. If the network finds that there are too many LP-WUS transmissions when the TA/RNA consists of a large number of cells, it can configure LP-WUS to be used only for the last used cell. In summary, this should be a configurable feature, and the new IE for CN assigned subgroup ID should be introduced into the Xn paging message for RAN paging.

Proposal 4: A new IE for CN assigned subgroup ID should be introduced into the Xn paging message for RAN paging.

2.4 UE_ID based subgrouping

According to the agreement in RAN2#127 meeting:

- In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately (if UE is configured to use both PEI and LP-WUS and if UE support both) in order to reduce the false paging rate. FFS on detailed configurations.

Moreover, RAN1 also made a working assumption that the number of LP-WUS subgroups is 32, which is different from 8 in PEI.

So, for UE_ID based LP-WUS subgrouping, the UE_ID for the calculation also need to be extended just like what we do in PEI.

Proposal 5: A new IE to include extra bits for UE_ID based LP-WUS subgrouping should be introduced into F1 paging.

- Conclusions

In this contribution, we discussed supporting LP-WUS indicating paging monitoring in RRC_IDLE/INACTIVE mode, and following are the proposals.

Proposals:

Proposal 1: CN assigned subgrouping does not require additional capability indication in F1 paging message.

Proposal 2: Both UE level and cell level LP-WUS subgrouping capability information are required in F1 paging message.

Proposal 3: It is reasonable for gNB-DU to decide the entry/exit condition.

Proposal 4: A new IE for CN assigned subgroup ID should be introduced into the Xn paging message for RAN paging.

Proposal 5: A new IE to include extra bits for UE_ID based LP-WUS subgrouping should be introduced into F1 paging.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)