3GPP TSG-RAN WG3 Meeting #126														R3-247677

Orlando, US, Nov 18th - Nov 22th, 2024

Agenda item:			11.2

Source:					CMCC

Title:						Discussion on AI-based network slice

Document Type:		Discussion

Introduction

During RAN3#125bis, the following agreements have been captured for AI/ML enabled slicing use case.

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

In this paper, we provide our consideration of AI/ML based Network Slicing.

Discussion

Xn interface enhancement

In RAN3#125bis, RAN3 has agreed to enhance the Data Collection Reporting Initiation and Data Collection Reporting procedure for slice use case, but how to support the new introduced input/feedback information was not discussed.

In SI phase, slice use case introduced the predicted radio resource status per slice and predicted slice available capacity as input information. As the Data Collection Reporting Initiation procedure already included the ‘Predicted Radio Resource Status’, and the ‘Predicted Radio Resource Status’ IE is referred to the ‘Radio Resource Status’ IE, which already including the radio resource status per slice, only the semantics description is needed to be updated to include the per slice level radio resource status.

Proposal 1: Update the semantics description of ‘Predicted Radio Resource Status’ IE in Data Collection Reporting procedure to ‘The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and Slice Radio Resource Status Item IE.’.

For the predicted slice available capacity, we propose to introduce a new IE in Data Collection Reporting procedure, and introduce a new bit in ‘Report Characteristics’ IE in Data Collection Reporting Initiation procedure to support this information.

Proposal 2: Introduce a new IE in Data Collection Reporting procedure to include predicted slice available capacity information.

Proposal 3: Introduce a new bit in ‘Report Characteristics’ IE of Data Collection Reporting Initiation procedure to include predicted slice available capacity information.

Then, RAN3 already agreed to introduce Slice To Report List for Data Collection IE in Data Collection Request message to indicate which slices' predicted information are required by the local node. Considering that when a local node requests prediction information from a neighbour node, the neighbour nodes may not support to provide the requested prediction information temporarily. Same as R18, it’s necessary to indicate which slice’s prediction information cannot be provided in the Data Collection Response message.

Proposal 4: Indicate which slice’s prediction information cannot be provided in Data Collection Response message. Same as the R18 procedure.

Currently, the Data Collection Response message including the ‘Node Measurement Initiation Result List’ and ‘Cell Measurement Initiation Result List’. For how to implement the slice level prediction information, two options can be discussed:

Opt 1: Introduce a separate IE for the failed slice level prediction information, different from the Node Measurement Initiation Result List IE and Cell Measurement Initiation Result List IE.

Opt 2: Introduce a new IE under the Cell Measurement Initiation Result List IE.

As Slice To Report List for Data Collection IE introduced in Data Collection Request message is under the Cell To Report List for Data Collection IE, it’s better to follow the same principle in Data Collection Response message that introduce a new IE under the Cell Measurement Initiation Result List IE. Thus, we slightly prefer Opt 2.

Proposal 5: Introduce a new IE under the Cell Measurement Initiation Result List IE in Data Collection Response message to indicate which slice’s prediction information cannot be provided.

Finer granularity of UE performance feedback

In RAN3#125bis, RAN3 has discussed 3 options of UE performance feedback:

Opt1: per S-NSSAI per PDU Session

Opt2: per S-NSSAI

Opt3: per-QoS flow groups

And a WA of the granularity of UE performance feedback was captured.

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

From our perspective, the per S-NSSAI per PDU session level can be used to evaluate the service performance after handover. Since the each slice of the UE may contain multiple PDU sessions, if one slice contain more than one PDU sessions and each PDU session represents a service, it’s difficult to monitor the service performance after handover with slice level UE performance feedback. Also, the slice level UE performance feedback can be derived by the per S-NSSAI per PDU Session level UE performance. So, we propose to turn the WA into agreement.

Proposal 6: Turn the “WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node” into agreement.

Based on the above discussion, we provide a TP for TR38.423 in R3-247678.

Proposal 7: Agree the TP in R3-247678 for 38.423.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: Update the semantics description of ‘Predicted Radio Resource Status’ IE in Data Collection Reporting procedure to ‘The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and Slice Radio Resource Status Item IE.’.

Proposal 2: Introduce a new IE in Data Collection Reporting procedure to include predicted slice available capacity information.

Proposal 3: Introduce a new bit in ‘Report Characteristics’ IE of Data Collection Reporting Initiation procedure to include predicted slice available capacity information.

Proposal 4: Indicate which slice’s prediction information cannot be provided in Data Collection Response message. Same as the R18 procedure.

Proposal 5: Introduce a new IE under the Cell Measurement Initiation Result List IE in Data Collection Response message to indicate which slice’s prediction information cannot be provided.

Proposal 6: Turn the “WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node” into agreement.

Proposal 7: Agree the TP in R3-247678 for 38.423.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC