3GPP TSG-RAN WG3 Meeting #126	             R3-247686

Orlando, US, Nov 18- Nov 22, 2024

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on RAN Architecture for Ambient IoT

Document for:	Discussion

1	Introduction

In this contribution, we will continue to discuss the RAN architecture aspects for Ambient IoT.

2	Discussion

During last meeting, SA2 made some conclusions regarding to the architecture and protocol stack for topology1 and topology2, and those conclusions will have impact on our RAN3 discussion. Based on SA2 outcome, we can add some supplements to our architecture and protocol stack for topology1 and topology2.

2.1	Topology1

Regarding to topology 1, The following contents excerpted from TR 23.700-13-110 can be used as reference:

### 8.1.2 Architecture to Support Topology 1 The principles and aspects in this clause are agreed to support Topology 1: - The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1: - Communicate with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request). NOTE 1: There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not. …… When a BS Reader and the AIoTF communicate directly: - The AIOTF communicates with a BS Reader via a direct interface Nx. - Figure 8.1.2-2 below shows the aspects related to Topology 1 (direct path) in reference point representation with other NFs removed. Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path) NOTE 4: The protocol used over Nx will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2. NOTE 5: The protocol stack used between the AIoTF and the BS Reader will be concluded by RAN3. - Figure 8.1.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the BS Reader and AIoTF. Figure 8.1.2-3: Example Protocol Stack between AIoTF and AIoT Device for Topology 1 (Direct Path) Editor’s Note: The details of the protocol stack are FFS. When a BS Reader and the AIoTF communicate indirectly via an AMF: - The AIoTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages. NOTE 6: If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader. - NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services. NOTE 7: The details of the enhancements will be concluded by RAN3. - The AMF shall be enhanced to support Services which are used by an AIoTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIoTF) are introduced in 5GC. - The AMF passes information between BS Readers (e.g. operation requests and responses) and the AIOTF. Editor’s Note: The AMF enhancements are FFS. Editor’s Note: Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS). …… - Figure 8.1.2-5 below shows the aspects related to Topology 1 (indirect Path via AMF) in reference point representation with other NFs removed. Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF) - Figure 8.1.2-6 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the BS Reader and AIoTF. Figure 8.1.2-6: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF) Editor’s Note: The details of the protocol stack are FFS. Editor’s Note: How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF is FFS.

It is noted that the AIOTF is introduced to support Ambient IoT functionality communicates with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request. From definition in RAN3, the AIoT CN may include the AIoTF and AMF. Current logical system architecture for topology1 in RAN3 can already fit for two kinds of SA2 architecture. Likewise, it also applies to the current protocol stack designed for topology1 in RAN3. The protocol stack of the AIoTF can be well presented by the current protocol stack for AIoT CN in RAN3. As we already agreed, it is unnecessary to illustrate how A-IoT upper layer information is terminated between AIoTF and device in RAN3 spec, and the end-to-end protocol design is under SA2 scope which is now clearly depicted in their latest spec.

Observation 1: As the AIoT CN includes the AIoTF and AMF, current logical system architecture and protocol stack for topology1 in RAN3 can already fit for two kinds of SA2 topology1 design.

Proposal 1: There is no need to update the current logical system architecture and protocol stack for Topology1.

2.2	Topology 2

During previous meeting, three solutions are captured in RAN3 for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

In SA2 TR, it is clearly stated that only User-plane option and RRC-based option will be specified for Topology2. Meanwhile, in the last RAN3 meeting, companies spent a large amount of time on how to construct the protocol stack for NAS/UP based solution of topology2, finally without consensus. Regard to RRC-based solution, it is efficient for A-IoT enabled gNB allocating A-IoT resources to UE reader. What’s more, it brings benefits for A-IoT enabled gNB involvement to select more suitable intermediate UEs since there is more real-time information. For example, A-IoT enabled gNB can select the UE reader based on the beam information where UE is located. For UP-based solution, it is obviously that the solution is quite simple which supports end-to-end IP connectivity between UE reader and AIoTF. While for NAS-based solution, it is still being discussed by SA2 whether it is needed and we personally do not find many obvious advantages compared with the other two solutions. However, to keep align with the discussion progress from RAN3 and SA2, we may not rule out three solutions in the last meeting of SI.

Proposal 2: To keep align with the discussion progress from RAN3 and SA2, RAN3 is not suggested to rule out three solutions for topology 2 in the last meeting of SI.

Concerning topology2, the following contents excerpted from TR 23.700-13-110 can be used as reference:

#### 8.1.3.2 User-plane option The following principles apply: - As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2. Figure 8.1.3.2-1: User-plane architecture for Topology 2 Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2 NOTE 1: Which transport protocol to use for AIoT-AP can be decided by CT1 in coordination with RAN3. NOTE 2: Security for AIoT-AP is assumed to be defined by SA3. Editor’s note: Further details are FFS. Editor’s note: The terminology, e.g. AIoT RAN is FFS. #### 8.1.3.3 RRC option The following principles apply: - Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2. Editor’s note: Which option to use, i.e., whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1 ) or via AMF (Option B in Figure 8.1.3.3-Y )) is FFS. Editor’s note: How addressing works for UL for Option B is FFS. Editor's note: Further details are FFS. Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A) Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

RRC based solution

From SA2 interim conclusion, it is noted that the RRC based solution also has two potential architectures, the AIoTF either connects A-IoT-enabled gNB directly or via an AMF, similar to the architectures depicted in topology 1. As it is not RAN3 scope to specify each function node in A-IoT CN, the current protocol stack for RRC based solution in RAN3 can well reflect the two possible protocol stacks by only appearing A-IoT CN protocol stack.

Figure 1: RRC based solution of Topology 2

Observation 2: The current protocol stack for RRC based solution in RAN3 can well reflect the two possible protocol stacks by only appearing A-IoT CN protocol stack.

Proposal 3: There is no need to update the current logical system architecture and protocol stack for RRC solution for Topology 2.

UP based solution

Figure 2: UP based solution of Topology 2

From SA2 interim conclusion, the A-IoT related signalling/data between the UPF and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU Session. The A-IoT-enabled gNB handles the A-IoT-enabled UE’s user plane data as legacy, i.e., using NG-U GTP-U tunnels. Note that for UP based solution, A-IoT radio resources allocation should be controlled by A-IoT-enabled gNB, so some necessary signalling transmission is required to trigger the resource allocation. In addition, the ways to trigger A-IoT radio resource allocation can be upon CN request or upon UE request, the details solution can be discussed in WI. The A-IoT radio resources can be requested in advance to the UP based communication with the A-IoT device or can be requested along with the UP based communication.

Proposal 4: For UP based solution, A-IoT radio resources allocation should be controlled by A-IoT-enabled gNB, so some necessary signalling request mechanisms, i.e., UE based or CN based are required to trigger the resource allocation.

NAS based solution

Figure 3: NAS based solution of Topology 2

In this case, A-IoT CN communicates with A-IoT-enabled UE using NAS message directly. The A-IoT related signalling/data between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets. The A-IoT enabled gNB is transparent to the NAS packet which is possibly transferred via DL NAS Transport and UL NAS Transport procedures. The resource allocation criteria used for UP based solution can also be applied to NAS based solution.

Proposal 5: The resource allocation criteria used for UP based solution can also be applied to NAS based solution.

A-IoT enabled UE authorization

In topology 2, if a UE wants to act as UE reader, the precondition is this UE should be authorized by the network. Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB. To support UE authorization, UE can report its capability to the CN in the UE Registration procedure. Once receiving the UE capability information, the UDM will send the UE subscription information to the AMF. If the UE is authorized, then the AMF informs to the NG-RAN node UE is authorized to perform A-IoT service. Hence, the NGAP signalling procedure supporting the A-IoT-enabled UE authorization should be discussed in WI.

Observation 3: Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB.

Proposal 6: The NGAP signalling procedure supporting the A-IoT-enabled UE authorization should be discussed in WI.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Observation 1: As the AIoT CN includes the AIoTF and AMF, current logical system architecture and protocol stack for topology1 in RAN3 can already fit for two kinds of SA2 topology1 design.

Observation 2: The current protocol stack for RRC based solution in RAN3 can well reflect the two possible protocol stacks by only appearing A-IoT CN protocol stack.

Observation 3: Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB.

Proposal 1: There is no need to update the current logical system architecture and protocol stack for Topology1.

Proposal 2: To keep align with the discussion progress from RAN3 and SA2, RAN3 is not suggested to rule out three solutions for topology 2 in the last meeting of SI.

Proposal 3: There is no need to update the current logical system architecture and protocol stack for RRC solution for Topology 2.

Proposal 4: For UP based solution, A-IoT radio resources allocation should be controlled by A-IoT-enabled gNB, so some necessary signalling request mechanisms, i.e., UE based or CN based are required to trigger the resource allocation.

Proposal 5: The resource allocation criteria used for UP based solution can also be applied to NAS based solution.

Proposal 6: The NGAP signalling procedure supporting the A-IoT-enabled UE authorization should be discussed in WI.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN;