3GPP TSG-RAN WG3 Meeting #126	              R3-247687

Orlando, US, Nov 18- Nov 22, 2024

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on coordination between CN and RAN

Document for:	Discussion

1	Introduction

In this contribution, we would like to provide our views on some essential coordination between A-IoT CN and A-IoT RAN.

2	Discussion

As RAN2 and RAN3 are discussing paging over A-IoT radio and over XX interface separately, RAN3 can take following RAN2 agreements about paging over A-IoT radio interface as reference.

RAN2#127:

1	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the A-IOT interface as upper layer data.

Based on RAN2 agreements, they think some information are considered useful to be visible to the reader from CN. From RAN3 perspective, it is our responsibility to check whether above information is worth to be visible from CN-RAN interface.

Regard to the service type of A-IoT, no matter for topology 1 or topology 2, it is beneficial for A-IoT RAN node and gNB be aware of the service type to allocate resources efficiently for different types of services. Regard to different kinds of service type, different sizes of resource are allocated. For example, if the A-IoT paging is inventory, A-IoT RAN node or gNB will allocate the D2R resource for A-IoT devices up to 96bits according to the inventory type. If the A-IoT paging is read, A-IoT RAN node or gNB will reserve 1000bits at most to retrieve the content from A-IoT device. If the A-IoT paging is write, A-IoT RAN node or gNB will only allocate limited resource to receive the feedback on whether the result of write is successful or not. Therefore, it is necessary to include service type (e.g., inventory, read, write, etc.) in the A-IoT paging message.

Observation 1: It is beneficial for A-IoT RAN node or gNB be aware of the service type to allocate resources efficiently for different types of services.

Proposal 1: Service type (e.g., inventory, read, write, etc.) should be included in the A-IoT paging message.

As RAN2 mentions the information about targeted for one or more than one device is useful to be visible to the reader from CN, this agreement is made with the consideration that the information may help A-IoT RAN node allocate radio resources and RA type selection. If it is targeted for single device, the A-IoT RAN node can enable the dedicated device to perform CFRA. If it is targeted for more than one device, the A-IoT RAN node enable all devices to perform CBRA.

Observation 2: The information about targeted for one or more than one device helps A-IoT RAN node allocate radio resources and RA type selection.

Proposal 2: Targeted for one or more than one device should be included in the A-IoT paging message.

For the estimated number of A-IoT devices, if the number is available, it can be informed from CN to A-IoT RAN node. It is not mandatory to include the approximate number in A-IoT paging message. The A-IoT RAN node can always estimates the device number by itself. Therefore, the estimated number of target A-IoT devices is not mandated in A-IoT paging message.

Proposal 3: The estimated number of target A-IoT devices is not mandated in A-IoT paging message.

RAN2#127:

Support mechanism for reader to be able to trigger multiple/subsequent A-IoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

RAN2#127bis:

The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase.

During RAN2 discussion, reader should support the mechanism on triggering multiple A-IoT paging messages which are associated with the same request from A-IoT CN. However, each paged device is expected to respond only once for one request from A-IoT CN, otherwise, duplicated response may case more collisions and greatly decrease the efficiency of the A-IoT system. To avoid the duplicated response from devices, an identifier representing the A-IoT CN request, such as session ID or transaction ID, can be included in A-IoT paging message over both A-IoT radio and XXAP to identify separate request from different A-IoT CNs or one A-IoT CN. With that, if a device is accessed successfully after being paged by an A-IoT paging, it should save the identifier reflecting the A-IoT CN request contained in the A-IoT paging message. After that, if another A-IoT paging message which contains the same identifier as the saved one is received, the device will not respond to it.

Observation 3: To avoid the duplicated response from devices, an identifier representing the A-IoT CN request, such as session ID or transaction ID, can identify separate request from different A-IoT CNs or one A-IoT CN.

Proposal 4: The session ID or transaction ID should be included in A-IoT paging message.

Furthermore, when reviewing the legacy paging procedure, the paging triggered by A-IoT CN is broadcasted among all cells within the corresponding TAI list, while the paging triggered by RAN is broadcasted among all cells within the RAN Notification Area (RNA). For A-IoT, it has been assumed that the device will not support tracking/RAN area update procedure. To improve the efficiency of A-IoT paging, it is beneficial for A-IoT CN to maintain an association relationship between A-IoT devices and associated area. To support the association, something like A-IoT paging area composed of A-IoT readers can be included in A-IoT paging message. The A-IoT CN can inventory the A-IoT device(s) within this specific area instead of always inventorying all A-IoT devices.

Observation 4: To improve the efficiency of A-IoT paging, it is beneficial for A-IoT CN to maintain an association relationship between A-IoT devices and associated area.

Proposal 5: A-IoT paging area should be included in A-IoT paging message.

Moreover, sometimes the inventory requirements from the server are periodic. Naturally, the CN will also provide the RAN with inventory periodicity, indicating how often inventory or command should be performed.

Proposal 6: The inventory periodicity may be included in A-IoT paging message.

In addition, there is a remaining Editor’s Note in pCR stating that it is FFS whether A-IoT RAN needs to interpret/store/process A-IoT Device Identification. Since we already agreed that the A-IoT RAN node may send one or multiple Inventory response message to the A-IoT CN. The A-IoT RAN node have the requirements to store the A-IoT Device Identification. However, considering the security risk, A-IoT RAN node is not required to process or interpret the A-IoT Device Identification, the final decision is subject to SA2 and SA3.

Proposal 7: The A-IoT RAN node needs to store the A-IoT Device Identification, but is not required to process or interpret.

Regarding the coordination, we also need to consider the reader selection which may need the coordination between A-IoT CN and A-IoT RAN. During last meeting, companies show different views on reader selection.

For topology 1, we already agreed the coordination mechanism may needed between A-IoT RAN and A-IoT CN. The A-IoT paging area consist of the A-IoT RAN node may pre-configured in A-IoT CN, the A-IoT CN can select the reader based on the pre-reconfiguration information by OAM and the reader ID included in Inventory Report/Command Response message reported by A-IoT RAN in previous A-IoT inventory/command procedure.

Proposal 8: For topology 1, the A-IoT CN can select the reader based on the pre-reconfiguration information by OAM and the reader ID included in Inventory Report/Command Response message reported by A-IoT RAN in previous A-IoT inventory/command procedure.

For topology 2, concerning A-IoT enabled UE reader selection, for RRC based solution, both A-IoT CN and A-IoT enabled gNB can perform A-IoT-enabled UE selection. Note that gNB selection and CN selection are not contradictory, they may exist simultaneously. The A-IoT CN maintains the mapping between A-IoT enabled gNB and A-IoT enabled UE, as well as mapping between A-IoT enabled UE and device. In addition, the authorization information is stored in the A-IoT CN, so it knows which UE is authorized as the UE reader. However, A-IoT CN does not know the UE RRC states and A-IoT CN may select two UEs which are both close to the device, resulting the duplicate inventory or command by UE reader. Thus, the UE reader selection needs the involvement with A-IoT enabled gNB.  For NAS/UP based solution, the A-IoT data transmission is transparent to the A-IoT enabled gNB, only A-IoT CN selects the UE reader. In a word, the UE reader selection mechanism for topology 1 and topology 2 will be specified in WI.

Proposal 9: For RRC based solution of topology 2, both A-IoT CN and A-IoT enabled gNB can perform A-IoT-enabled UE selection.

Proposal 10: For NAS based solution of topology 2, only A-IoT CN selects the A-IoT enabled UE.

Proposal 11: The UE reader selection mechanism for topology 1 and topology 2 will be specified in WI.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on coordination between CN and RAN:

Observation 1: It is beneficial for A-IoT RAN node or gNB be aware of the service type to allocate resources efficiently for different types of services.

Observation 2: The information about targeted for one or more than one device helps A-IoT RAN node allocate radio resources and RA type selection.

Observation 3: To avoid the duplicated response from devices, an identifier representing the A-IoT CN request, such as session ID or transaction ID, can identify separate request from different A-IoT CNs or one A-IoT CN.

Observation 4: To improve the efficiency of A-IoT paging, it is beneficial for A-IoT CN to maintain an association relationship between A-IoT devices and associated area.

Proposal 1: Service type (e.g., inventory, read, write, etc.) should be included in the A-IoT paging message.
Proposal 2: Targeted for one or more than one device should be included in the A-IoT paging message.

Proposal 3: The estimated number of target A-IoT devices is not mandated in A-IoT paging message.

Proposal 4: The session ID or transaction ID should be included in A-IoT paging message.

Proposal 5: A-IoT paging area should be included in A-IoT paging message.

Proposal 6: The inventory periodicity may be included in A-IoT paging message.

Proposal 7: The A-IoT RAN node needs to store the A-IoT Device Identification, but is not required to process or interpret.

Proposal 8: For topology 1, the A-IoT CN can select the reader based on the pre-reconfiguration information by OAM and the reader ID included in Inventory Report/Command Response message reported by A-IoT RAN in previous A-IoT inventory/command procedure.

Proposal 9: For RRC based solution of topology 2, both A-IoT CN and A-IoT enabled gNB can perform A-IoT-enabled UE selection.

Proposal 10: For NAS based solution of topology 2, only A-IoT CN selects the A-IoT enabled UE.

Proposal 11: The UE reader selection mechanism for topology 1 and topology 2 will be specified in WI.

4	Reference

[1] 	RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2]	RP-234058, Time_budget_request_R19_Ambient_IoT_SID