3GPP TSG-RAN WG3 Meeting #126	             R3-247688

Orlando, US, Nov 18- Nov 22, 2024

Agenda Item:	16.4

Source:	CMCC

Title:	Discussion on locating Ambient IoT device

Document for:	Discussion

1	Introduction

During last meeting, we agreed the following agreements on the topic of locating an Ambient IoT device.

The following principle applies to Topology1 and Topology2: For both purpose a) and b), identifier of the reader may be reported to A-IoT CN in the Inventory Report message. While for purpose b), whether the reader ID based granularity of the A-IoT device location is sufficient can be further checked.

Additionally, the current TR related to locating Ambient IoT device part is shown as follows:

### 6.9.x General Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network. A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g. by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device. Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS. Editor’s Note 2: Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study. Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study. Editor’s Note 4: Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

Since the upcoming meeting will be the last meeting for Rel-19 A-IoT SI, in this contribution, we will solve all the questions shown in the Editor’s Notes in TR.

2	Discussion

The Editor’s Note 1 is asking how does A-IoT CN know the “reader” location. For topology 1, an A-IoT RAN node may serve one or more readers, the reader location is considered as stable and not moving. The possible way for A-IoT CN obtaining the reader location is based on OAM configuration or coordination between A-IoT RAN and A-IoT CN. For topology 2, A-IoT enabled UE served as reader may be stationary or moving, so the A-IoT CN can trigger LCS to obtain the UE’s location using existing positioning method or OAM configuration.

Proposal 1: For topology 1, the way for A-IoT CN obtaining the reader location is based on OAM configuration or coordination between A-IoT RAN and A-IoT CN.

Proposal 2: For topology 2, the A-IoT CN can trigger LCS to obtain the UE’s location using existing positioning method or OAM configuration.

Editor’s Note 1 also states that whether to use more than one “reader” within one A-IoT RAN for location purposes. From our view, RAN3 should avoid to utilize more readers for locating an A-IoT device in this release. The utilization of more than one “reader” for location purposes will lead to new positioning method, which needs additional work for other working groups to evaluate the performance of A-IoT device and check the feasibility of new positioning method.

Observation 1: The utilization of more than one “reader” for location purposes will bring new positioning method, which needs additional work for other working groups to evaluate the performance of A-IoT device and check the feasibility of new positioning method.

For example, as some companies proposed, the new positioning method with multiple TRPs or multiple readers to measure the power of the backscattered signal or the output power of the carrier wave is introduced, which violates the objective of the Rel-19 SI. As a reference, the SID is shown as follows:

E. Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). …. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). …. - RAN3-led: - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

From the objective, it is noted that the traffic types for device will be restricted on indoor inventory and indoor command. There is no description about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by A-IoT devices is not in the scope of Rel-19 A-IoT discussion.

Observation 2: From the objective, the traffic types for device will be restricted on indoor inventory and indoor command and no description about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by A-IoT devices is not in the scope of this release discussion.

Proposal 3: RAN3 should avoid to utilize more readers for locating an A-IoT device in this release.

For the Editor’s Note 2, on behalf of the device location information, it is already agreed that support locating the A-IoT device at reader granularity. It means that the device location is represented as the reader ID which needs to be perceived by the network. The first thing waiting to be resolved is how to provide the reader ID of the A-IoT device to the A-IoT CN.

In topology 1, the A-IoT RAN node plays the role of the reader and the A-IoT RAN node ID can be regarded as the reader ID. During the XX Interface Setup procedure, the A-IoT RAN node can inform its A-IoT RAN node ID to A-IoT CN. Then, the A-IoT RAN node does not need to report its ID in the future inventory or command procedure as the A-IoT CN already knows where the report message is originated from. If the A-IoT RAN node serves more than one reader, the A-IoT RAN node reports reader IDs in the Inventory Report message and/or Command Response message to the A-IoT CN.

Observation 3: The A-IoT RAN node does not need to report its ID during the inventory or command procedure as the A-IoT CN already knows where the report message is originated from during the XX interface Setup procedure.

Proposal 4: In topology 1, the A-IoT RAN node can inform its A-IoT RAN node ID to A-IoT CN during the XX Interface Setup procedure.

Proposal 5: In topology 1, if the A-IoT RAN node serves more than one reader, the A-IoT RAN node reports reader IDs in the Inventory Report message and/or Command Response message to the A-IoT CN.

In topology 2, UE plays the role of the reader and the UE ID is already agreed to consider as the reader ID. Currently, three solutions are under discussion for topology 2, the difference between these three solutions lies in whether the gNB perceives the device location information.

Regard to the RRC-based solution, the UE is known by gNB since the UE needs to collect the device ID and to report them by RRC message. It is assumed that the UE ID along with the collected device ID are reported from A-IoT-enabled gNB to A-IoT CN via the Inventory Report message and/or Command Response message. Based on the mapping information between UE and A-IoT device, the A-IoT CN is aware that the device is under the control of which UE reader, and the UE is under the control of which gNB.

Regard to the NAS-based solution and UP-based solution, as the transmission of signalling or data is terminated in A-IoT CN and transparent to the gNB, the gNB does not know the device location information. The A-IoT CN collects the NAS packets or the PDU sessions reported from UE via the Inventory Report procedure. After receiving the NAS or user plane message from the UE, the A-IoT CN knows by default from which A-IoT-enabled UE the A-IoT device responded.

Observation 4: For RRC-based solution, the A-IoT CN is aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Proposal 6: Regard to the RRC-based solution for topology 2, the UE ID along with the collected device ID are reported from A-IoT-enabled gNB to A-IoT CN in the Inventory Report message and/or Command Response message.

Proposal 7: Regard to the NAS-based solution and UP-based solution for topology 2, the A-IoT CN knows by default from which A-IoT-enabled UE the A-IoT device responded.

Regard to the Editor’s Note 3, majority of companies reach a common understanding in last meeting that one A-IoT RAN node may serve multiple TRPs as multiple readers within one A-IoT RAN node in topology1, and it can be naturally associated with TRP ID can be considered as a location of A-IoT device. However, no related agreements were made in last meeting and RAN3 needs to confirm the TRP ID can be considered as a finer “reader ID” granularity for locating an Ambient device in this meeting. Hence, it is not necessary to introduce finer granularity for locating an Ambient device besides the “reader ID granularity”.

Proposal 8: RAN3 needs to confirm the TRP ID can be considered as a finer “reader ID” granularity for locating an Ambient device.

Proposal 9: It is not necessary to introduce finer granularity for locating an Ambient device besides the “reader ID granularity”.

Regard to the Editor’s Note 4, the SID points out RAN3 should identify potential solutions for locating an Ambient IoT device with no specification impact by reusing existing user location report, or minimal specification impact to convey location information to CN. From current NGAP, the User Location Information IE is included in multiple procedures from gNB to AMF for UE location reporting. The location information in User Location Information is represented by cell ID and TAI. Likewise, considered that the agreed case a and case b, the reader granularity, i.e., A-IoT RAN node ID, or TRP ID, or UE id, can be included in User Location Information. From our view, the reader ID may not support an extremely accurate A-IoT device location information towards CN, but it is enough to handle the positioning requirement for the case a and case b. With the consideration that the discussion in this release only focuses on the indoor case, the reported reader ID can be used to improve the A-IoT system operating more efficiently, and narrow the search scope within a small range which is simple for consumer to find an A-IoT device location. Therefore, it is nothing found on gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

Observation 5: With the consideration that the discussion in this release only focuses on the indoor case, the reported reader ID can be used to improve the A-IoT system operating more efficiently, and narrow the search scope within a small range which is simple for consumer to find an A-IoT device location.

Proposal 10: There is no gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

3	Conclusion

Based on the discussions mentioned above, we provide some observations and proposals on locating Ambient IoT device:

Observation 1: The utilization of more than one “reader” for location purposes will bring new positioning method, which needs additional work for other working groups to evaluate the performance of A-IoT device and check the feasibility of new positioning method.

Observation 2: From the objective, the traffic types for device will be restricted on indoor inventory and indoor command and no description about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by A-IoT devices is not in the scope of this release discussion.

Observation 3: The A-IoT RAN node does not need to report its ID during the inventory or command procedure as the A-IoT CN already knows where the report message is originated from during the XX interface Setup procedure.

Observation 4: For RRC-based solution, the A-IoT CN is aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Observation 5: With the consideration that the discussion in this release only focuses on the indoor case, the reported reader ID can be used to improve the A-IoT system operating more efficiently, and narrow the search scope within a small range which is simple for consumer to find an A-IoT device location.

Proposal 1: For topology 1, the way for A-IoT CN obtaining the reader location is based on OAM configuration or coordination between A-IoT RAN and A-IoT CN.

Proposal 2: For topology 2, the A-IoT CN can trigger LCS to obtain the UE’s location using existing positioning method.

Proposal 3: RAN3 should avoid to utilize more readers for locating an A-IoT device in this release.

Proposal 4: In topology 1, the A-IoT RAN node can inform its A-IoT RAN node ID to A-IoT CN during the XX Interface Setup procedure.

Proposal 5: In topology 1, if the A-IoT RAN node serves more than one reader, the A-IoT RAN node reports reader IDs in the Inventory Report message and/or Command Response message to the A-IoT CN.

Proposal 6: Regard to the RRC-based solution for topology 2, the UE ID along with the collected device ID are reported from A-IoT-enabled gNB to A-IoT CN in the Inventory Report message and/or Command Response message.

Proposal 7: Regard to the NAS-based solution and UP-based solution for topology 2, the A-IoT CN knows by default from which A-IoT-enabled UE the A-IoT device responded.

Proposal 8: RAN3 needs to confirm the TRP ID can be considered as a finer “reader ID” granularity for locating an Ambient device.

Proposal 9: It is not necessary to introduce finer granularity for locating an Ambient device besides the “reader ID granularity”.

Proposal 10: There is no gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)