3GPP TSG-RAN WG3 #126                                     R3-247716

Orlando, USA, 18th – 22th Nov, 2024

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses Evolution of Duplex Operation base on WID [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

- Discussion

  - Semi-static cell-specific SBFD time and frequency location configuration

The following agreements and WA are captured in RAN1:

Time domain:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

Frequency domain:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

The design of SBFD time and frequency location configuration should wait for RAN1 and RAN2. If RAN2 reuse container e.g., tdd-UL-DL-ConfigurationCommon for SBFD time configuration, RAN3 can simply reuse TDD UL-DL Configuration Common NR which has been captured in Served Cell Information already. For SBFD frequency location configuration, the NR Transmission Bandwidth and Carrier List already there which can be reused but it may need to introduce more information which can also wait for RAN1 and RAN2 decision.

For F1AP, the above SBFD time and frequency location configuration can be provided during F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE in Served Cell Information IE from DU to CU.

For XnAP, the above SBFD time and frequency location configuration can be provided during XN SETUP procedure and NG-RAN NODE CONFIGURATION UPDATE procedure in Served Cell Information IE between gNBs.

Proposal 1: RAN3 agrees to reuse Served Cell Information IE for transfer SBFD time and frequency location configuration in XnAP and F1AP. The IE design of SBFD time and frequency location configuration waits for RAN1 and RAN2.

  - Measurement resource configuration and Strongest DL beam information

LS [1] received from RAN1 as below

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Victim gNB measures NZP CSI-RS which is provided by neighbour aggressor cell. After measurement, victim gNB sends measurement result i.e., the strongest DL beam information back to neighbour aggressor cell. According to the strongest DL beam information, neighbour aggressor cell will try to reduce interference by implementation.

It can be found that there are two steps need to be supported

- Step 1: neighbour aggressor cell sends a set of one or more periodic NZP CSI-RS resources to victim cell

- Step 2: victim cell provides strongest DL beam information in the context of NZP-CSI-RS/SSB to neighbour aggressor cell

For step 1, NZP CSI-RS resources defined in TS38.331 is used to configure reference signal Non-Zero-Power (NZP) CSI-RS for measurement. It can be added in Served Cell Information NR.

For CU/DU split, neighbour aggressor gNB-DU sends NZP CSI-RS resources to neighbour aggressor gNB-CU for further transfer from neighbour aggressor gNB to victim gNB.

Proposal 2: Add a list of NZP CSI-RS resource in Served Cell Information NR in XnAP and F1AP.

For step 2, the strongest DL beam information is defined as CRI and SSB index.

The question is that if the reporting of the strongest DL beam is periodic or one-off or event trigger.

- In case of periodic response, the strongest DL beam information can be added in the RESOURCE STATUS UPDATE base on the request in Report Characteristics.

- In case of one-off response, the strongest DL beam information can be added in Xn SETUP RESPONSE and NG-RAN NODE CONFIGURATION UPDATE.

- In case of event trigger, victim gNB reports the strongest DL beam information when the interference over the threshold. The threshold is generated by victim gNB.

For CU/DU split, victim gNB-DU provide strongest DL beam to victim gNB-CU for further transfer from victim gNB to neighbour aggressor gNB. The same question as XnAP also exists in F1AP, whether the reporting of the strongest DL beam from victim gNB-DU is periodic or one-off or event trigger in DU.

Proposal 3: RAN3 discusses how to neighbour aggressor cell response the strongest DL beam information in XnAP (from victim gNB to aggressor gNB) and F1AP (from victim DU to victim CU) i.e., periodic response or one-off response or response base on event (threshold) trigger.

After neighbour aggressor gNB-CU receiving strongest DL beam information in the context of NZP-CSI-RS/SSB from victim gNB-CU, neighbour aggressor gNB-CU sends strongest DL beam information to neighbour aggressor gNB-DU e.g., for beam adjustment.

The strongest DL beam information can be added in Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE message.

Proposal 4: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE send from aggressor gNB-CU to aggressor gNB-DU.

  - CLI-mitigation request

- Note: RAN3 will not specify enhancements to network signaling to support inter-operator coordination for CLI handling

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

The victim gNB sends the CLI-mitigation handling request to neighbour aggressor gNB to expect neighbour aggressor gNB performs the CLI-mitigation. The NG-RAN NODE CONFIGURATION UPDATE can be reused.

The CLI-mitigation request can be considered as a trigger for neighbour aggressor gNB to send NZP CSI-RS to try to reduce CLI but we do not need to specify when and how to use CLI-mitigation request in specification.

Proposal 5: RAN3 agrees to introduce CLI-mitigation request in NG-RAN NODE CONFIGURATION UPDATE.

- Conclusion

Proposal 1: RAN3 agrees to reuse Served Cell Information IE for transfer SBFD time and frequency location configuration in XnAP and F1AP. The IE design of SBFD time and frequency location configuration waits for RAN1 and RAN2.

Proposal 2: Add a list of NZP CSI-RS resource in Served Cell Information NR in XnAP and F1AP.

Proposal 3: RAN3 discusses how to neighbour aggressor cell response the strongest DL beam information in XnAP (from victim gNB to aggressor gNB) and F1AP (from victim DU to victim CU) i.e., periodic response or one-off response or response base on event (threshold) trigger.

Proposal 4: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE send from aggressor gNB-CU to aggressor gNB-DU.

Proposal 5: RAN3 agrees to introduce CLI-mitigation request in NG-RAN NODE CONFIGURATION UPDATE.

- Reference

[1] R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, RAN1.