3GPP TSG-RAN WG3 #126	R3-24xxxx

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.1

Source:	CATT

Title: 	Discussion on LS for PDU Set Information Marking Support without QoS parameters

Document for:	Discussion and Approval

1	Introduction

A LS was received from SA2 that the PDU Set information marking on the GTP-U header can be supported while PDU Set QoS parameters are absent.

- If the PDU Set QoS parameters are absent, but the PDU Set Information marking is supported, the SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported.

- Based on the received PDU Set Information Marking Support Indicator from the CN, the NG-RAN sends information to the SMF to activate the PDU Set Information Marking for the PDU Set based handling (e.g., PSI based discarding due to congestion). Based on the NG-RAN provided information, the SMF instructs the PSA UPF to perform PDU Set information marking in the GTP-U header.

In this contribution we analyse the benefit on the PDU Set based handling without PDU set QoS parameters and provide feedback for NGAP signalling design.

2	Discussion

In case NG-RAN supports PDU set based handling, CN may only provide PDU Set information marking on the GTP-U header as agreed in SA2.

The PDU set information includes:

- PDU Set Sequence Number;

- Indication of End PDU of the PDU Set;

- PDU Sequence Number within a PDU Set;

- PDU Set Size in bytes;

- PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

The PDU Set Importance may help the PSI based discarding due to congestion and network scheduling even without PDU QoS parameter. The final feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters depends on RAN2.

Observation 1: PDU Set based handling without PDU set QoS parameters may be useful for PSI based discarding due to congestion and network scheduling. The final conclusion on whether it is useful depends on RAN2.

After receiving PDU Set Information Marking Support Indicator from CN, there are two options about how to NG-RAN response can be considered.

Option 1: NG-RAN responds once whether it supports PDU Set based handling without PDU set QoS parameters.

Option 2: NG-RAN responds whether it wants to activate PDU Set based handling without PDU set QoS parameters if NG-RAN supports this feature. Otherwise, NG-RAN does not response.

We prefer option 1 because it is capability whether NG-RAN can support PDU Set based handling without PDU set QoS parameters. There is no reason that NG-RAN supports it but does not want to perform i.e., “deactivate” in option 2, especially it will benefit from scheduling e.g., network can prioritize the PDU set of which 80% PDU already has been successful transmitted or the PSI is high.

Proposal 1: NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set information marking is supported.

Furthermore, there are two options that can be considered for option 1.

Option 1-a) Add a code point in PDU Set based Handling Indicator

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set based Handling Indicator | M |  | ENUMERATED (supported, supported only in UP…) |  |

Option 1-b) If the SMF sends PDU Set Information Marking Support Indicator to NG-RAN, the PDU Set based Handling Indicator means NG-RAN supports only PDU set based handling in UP. If SMF does not send PDU Set Information Marking Support Indicator to NG-RAN, the PDU Set based Handling Indicator means NG-RAN support PDU set based handling in both CP and UP.

The detail specification impact can be discussed when WI phase in R19 XR starts.

Proposal 2: RAN3 discuss whether to add a code point i.e., “supported only in UP” in PDU Set based Handling Indicator.

3 Conclusions

In this contribution the following observations and proposal were derived:

Observation 1: PDU Set based handling without PDU set QoS parameters may useful for PSI based discarding due to congestion and network scheduling. The final decision on whether usefulness or not is depend on RAN2.

Proposal 1: NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set based handling is supported the PDU Set information marking.

Proposal 2: RAN3 discuss whether to add a code point i.e., “supported only in UP” in PDU Set based Handling Indicator.

3 Annex

3GPP TSG-RAN WG2 #126	R3-24xxxxx

# Orlando, USA, 18th – 22nd 2024 November

Title:	[DRAFT] Reply LS on PDU Set Information Marking Support without QoS parameters

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	CATT (to be RAN3)

To:	RAN2

Cc:

Contact Person:

#### Name:	liu Ziqiao

Email address:	liuziqiao@catt.cn

Attachments:	None

1. Overall Description:

RAN3 has discussed on PDU Set Information Marking Support without QoS parameters and made the following conclusion.

Q2:SA2 would like RAN3 feedback which approach to support in Rel-19.

Answer: RAN3 thinks NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set information marking is supported. The signalling design will be discussed when R19 XR WI starts.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly asks SA2 to take the above information into account

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#127	                    17 – 21 February 2025		Athens, GR

TSG-RAN WG3#128	                    07 – 11 April 2025					China, CN