3GPP TSG-RAN WG3 Meeting #126	R3-247722

Orlando, USA, 18 – 22 November 2024

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			Further consideration on support of WAB

Document for:		Discussion and Decision

1.	Introduction

In R3-244019/S2-2407345 [1], the SA2 sent a LS on questions regarding FS_VMR_Ph2 as follows:

… Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06. …

In this LS, SA2 asks the RAN3 to provide any feedback on two issues related to the WAB support. In last RAN3 meeting, RAN3 had initial discussion on these issues, and captured some RAN-based solutions to avoid the multi-hop WAB [2]. In this contribution, we will discuss the open issues raised by the SA2 in the LS and provide our view on it.

2.	Discussion

Regarding two issues related to the additional ULI format and the access control to guarantee single-hop backhauling (to restrict WAB-MT accessing WAB-gNB cell), SA2 sent a LS to ask RAN3 to provide any feedback on these issues [1].

First issue in [1] is how to the access control (to restrict WAB-MT accessing WAB-gNB cell) for below two cases:

- WAB-MT accessing any WAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

- WAB-MT accessing WAB-gNB belonging to same WAB.

In RAN3#125bis meeting, RAN3 captured the following RAN based solutions to avoid multi-hop WAB in [2]:

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

- Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCSetupComplete message to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution 5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

In Solution 1, there is no Stage 3 specification impact. However, the operator should reserve dedicated frequencies and/or PCIs for WAB operation, thus causing some restrictions on the operator’s deployment.

Proposal 1: Solution 1 can avoid multi-hop WAB without Stage 3 specification impact.

Proposal 1a: Solution 1 causes some restrictions on the operator’s deployment to reserve dedicated frequencies and/or PCIs for WAB operation.

Solution 2 assumes that the dedicated slice is used for backhauling. The WAB-gNB does not support the dedicated slices for WAB, but these slices are supported by the BH-RAN node. Therefore, when the WAB-MT sends the Requested NSSAI which is consisted of the dedicated slices for WAB only, the WAB-gNB can decide to release the RRC connection with the WAB-MT.

According to the SA2 CR in [3], it is confirmed that the authorization of the WAB-MT is performed based on its subscription information which includes dedicated slice(s) and DNN for WAB operation. Therefore, the operator needs to configure the dedicated slice(s) for WAB operation in the legacy RAN node. In addition, for initial access, the WAB-MT should include the dedicated slice for WAB in the RRCSetupComplete message to indicate to the network whether the UE intends to operate as the WAB-MT. This means that it may be difficult to protect privacy for the NSSAI in the Access Stratum. If AMF instructs the UE to never include the slice information in the Access Stratum according to Clause 5.15.9 of TS 23.501, the WAB-gNB cannot initiate the RRCRelease message. In this case, the AMF can know that the UE is a WAB-MT based on the dedicated S-NSSAI(s)/DNN, and reject the registration request from the WAB-MT to prevent the multi-hop WAB. However, it causes additional latency because the initial access of WAB-MT is rejected by the AMF instead of the WAB-gNB. In addition, since the WAB-MT does not have a priori knowledge on WAB-gNBs, unnecessary access trials from WAB-MT may happen multiple times to different WAB-gNBs around.

During the mobility of the WAB-MT in CONNECTED mode, the BH-RAN node (i.e., source node) is difficult to know whether the UE is a WAB-MT and whether the target node is a WAB-gNB. However, since the WAB-gNB indicates to the BH-RAN node that it does not support the dedicated slices for WAB during the Xn Setup procedure, the BH-RAN node can avoid the selection of the WAB-gNB as a target node. Even if the BH-RAN node selects the WAB-gNB as a target node and the Xn handover for the WAB-MT is triggered, the target WAB-gNB can reject the handover based on the dedicated slices associated with the BH PDU Session in the Xn HANDOVER REQUEST message.

Proposal 2: Solution 2 can avoid multi-hop WAB without Stage 3 specification impact.

Proposal 2a: Solution 2 causes additional latency due to the reject for initial access by AMF and unnecessary access trials towards the neighbour WAB-gNBs.

In Solution 3, since the WAB-MT does not attempt to access to the WAB-gNB cell, unnecessary access trials from WAB-MT can be prevented. Therefore, our understanding is that Solution 3 can complement Solution 2. However, the WAB-gNB needs to be enhanced to broadcast a new indicator in SIB, thus resulting in Stage 3 specification impact. Our understanding is that since this solution is very similar to the approach in Rel-18 mobile IAB, foreseen specification impacts are already well known from Rel-18 mobile IAB.

It is possible that the BH-RAN node serving the WAB-MT is enhanced to support the WAB functionality (i.e., BH-RAN node can be aware of the WAB functionality). Therefore, the BH-RAN node can be aware of whether the UE operates as the WAB-MT based on the dedicated slices for WAB. Also, when the Xn interface is established between the stationary RAN nodes and WAB-gNB, the BH-RAN node can be aware of whether the neighbour RAN node is the WAB-gNB based on the WAB-gNB cell indication during the Xn Setup procedure. Therefore, the BH-RAN node does not consider the WAB-gNB as a candidate target node during the Xn handover.

Proposal 3: Solution 3 can be complementary to Solution 2 by preventing unnecessary access trials from WAB-MT.

Proposal 3a: Solution 3 has Stage 3 specification impact, which is already known from Rel-18 mobile IAB.

In Solution 5, the source node includes a new WAB-node indication in Xn HANDOVER REQUEST message. Then, the target BH-RAN node performs access control for this WAB-node. We think that Solution 5 has a drawback because the BH-RAN node needs to be enhanced to perform access control based on the new WAB-node indication. Considering the requirement on the support of the WAB, legacy BH-RAN node should be able to provide the BH PDU Session for the WAB node without any enhancement on the WAB. Therefore, it seems that the enhancement for the BH-RAN node is not preferable.

It is also possible to use the dedicated slices for WAB as the new WAB-node indication. However, this is already covered in Solution 2. As mentioned in Solution 2, since the dedicated slices for WAB is provided, the target BH-RAN node can perform slice-aware admission control without any enhancement.

Proposal 4: Solution 5 causes the enhancement on the BH-RAN node to perform access control during HO.

Based on the above observations, the following proposal is suggested to RAN3:

Proposal 5: Solution 2 and Solution 3 are selected to avoid the multi-hop WAB.

Proposal 5a: Proposal 5 needs to consult with RAN2.

For Case b) in SA2 LS [1], it seems that the WAB-MT accesses to the co-located WAB-gNB. For the WAB-MT in IDLE mode, it cannot access to the co-located WAB-gNB because the co-located WAB-gNB is not yet initialized. For the WAB-MT in CONNECTED mode, the BH-RAN node may determine to initiate the handover of the WAB-MT towards the co-located WAB-gNB. This can be avoided by e.g. WAB implementation. For example, the WAB-MT may not report to the BH-RAN node the measurement results for the co-located WAB-gNB. This is because the WAB-MT can obtain the information (e.g., gNB ID, TAC, PCI, NCGI, and so on) related to the co-located WAB-gNB by using internal interface between WAB-MT and WAB-gNB. In this case, the BH-RAN node does not consider the WAB-gNB as a candidate target node.

Another option to avoid the WAB-MT’s access to the co-located WAB-gNB is that the BH-RAN node explicitly discovers the co-location of the WAB-MT and the WAB-gNB. The easiest way to indicate the co-location of the WAB-MT and the WAB-gNB is that the WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure.

Proposal 6: The BH-RAN node should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 6a: The WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure.

Next question in [1] is how to define the Additional ULI for the WAB. In [2], SA2 considered the following options used for the Additional ULI in normative phase:

- Option 1: mapped TAC/Cell ID based on geo-location of the WAB based on input from OAM;

- Option 2: geo-location of the WAB;

- Option 3: TAC/Cell ID of the cell serving the WAB-MT in other PLMN

In last meeting, RAN3 had the discussion on this issue, and confirmed that when the WAB-gNB and the WAB-MT connect to the same PLMN, the mechanism in Rel-18 mobile IAB can be reused for the WAB.

However, for the case where the WAB-gNB and the WAB-MT connect to different PLMNs, RAN3 concluded in [2] that much more clarification on the purpose of the information referred to as “additional ULI” is needed. According to the SA2 agreed CRs in [4] and [5], the UE’s AMF considers the Additional ULI when it determines UE location (e.g., the UE presence in Area of Interest) and manages the UE location related functions (e.g. Mobility Restrictions, Warning Area List NG-RAN). In this case, from the UE serving PLMN point of view, the UE’s AMF should be able to determine the current location of the WAB-MT in BH PLMN based on the Additional ULI.

In Option 1, we think that how to determine mapped TAC/Cell ID based on geo-location of the WAB needs to be configured by OAM, thus may cause unnecessary complexity. For example, whenever the geo-location of the WAB node is changed, the WAB-gNB may contact the OAM server to obtain the mapped TAC/Cell ID related to the WAB’s current location. In order to support Option 2, it seems that a new IE needs to be defined to contain the geo-location of the WAB in Additional ULI. However, from the RAN3 point of view, this is not a critical specification impact. Also, Option 3 just follows the approach in Rel-18 mobile IAB. That is, the TAC/Cell ID related to the WAB’s current location is just provided to the UE’s AMF. Then, the UE’s AMF may translate/convert it into the location information in the UE serving PLMN based on local configuration or contact with the OAM server. Therefore, we think that this option has a minor RAN3 impact as well.

Proposal 7: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format.

The above proposals are captured into a draft reply LS in Appendix. Then, the following proposal is suggested to RAN3:

Proposal 8: It is proposed to send a replay LS in Appendix to SA2.

One of the objectives in this WID is to support the Xn interface by the WAB-gNB with the BH RAN node and with other neighbouring gNBs, including how to avoid setting up Xn between WAB-gNBs. However, we think that in some cases, the Xn connection between two WAB-gNBs is beneficial to support the UE mobility. For example, if one WAB-gNB is operated as a stationary gNB for a long time, the Xn interface can be established between this stationary WAB-gNB and the other moving WAB-gNB. Since the operator has additional information for WAB (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB node), it can decide whether to allow the establishment of Xn interface between the WAB-gNBs. Then, this information needs to be (pre-)configured to the WAB-gNB and/or the UE’s AMF by OAM.

Proposal 9: The Xn interface between WAB-gNBs can be established.

As shown in Figure X.1-1 of [6], after the WAB-gNB setup, the WAB-gNB can initiate the establishment of the Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s) by sending the Xn SETUP REQUEST message. As mentioned in Proposal 6a, the WAB-gNB includes the information related to the WAB-MT to the BH-RAN node to indicate the co-location of the WAB-MT and the WAB-gNB. Based on Solution 3, the WAB-gNB cell information may be also provided to other NG node(s). In order to support Proposal 9, we think that the WAB-gNB can contain additional assistance information (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB node, and so on) related to the WAB mobility in the Xn SETUP REQUEST message. If the receiving NG-RAN node is another WAB-gNB, it decides whether to establish the Xn interface with the WAB-gNB based on this assistance information. For example, if both WAB-gNBs have a similar moving characteristics/route, or either or both of WAB-gNBs are stationary, the receiving WAB-gNB can respond with the Xn SETUP RESPONSE message. In addition, it is possible for the receiving WAB-gNB to include additional assistance information related to the WAB mobility in the Xn SETUP RESPONSE message. The initiating WAB-gNB can take this information into account to manage the Xn interface between WAB-gNBs.

If the Xn setup between WAB-gNBs is not allowed or deemed not appropriate, the receiving WAB-gNB should reject the Xn setup with the initiating WAB-gNB and reply with the Xn SETUP FAILURE message. When the Xn SETUP FAILURE message contains appropriate information (e.g., WAB-gNB indication, new cause value “Target not allowed due to WAB”), the initiating node does not reinitiate the Xn Setup procedure towards the same receiving WAB-gNB. In addition, the receiving WAB-gNB may provide additional assistance information related to the WAB mobility to enable the initiating WAB-gNB to determine whether and when to reinitiate the Xn Setup procedure towards the same receiving node.

Proposal 10: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 11:, The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent reinitiation of the Xn Setup procedure towards the same receiving WAB-gNB.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 12: It is proposed to agree the corresponding TPs in [7].

3.	Conclusion

In this contribution, we focused on the open issues raised by the SA2 in the LS and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: Solution 1 can avoid multi-hop WAB without Stage 3 specification impact.

Proposal 1a: Solution 1 causes some restrictions on the operator’s deployment to reserve dedicated frequencies and/or PCIs for WAB operation.

Proposal 2: Solution 2 can avoid multi-hop WAB without Stage 3 specification impact.

Proposal 2a: Solution 2 causes additional latency due to the reject for initial access by AMF and unnecessary access trials towards the neighbour WAB-gNBs.

Proposal 3: Solution 3 can be complementary to Solution 2 by preventing unnecessary access trials from WAB-MT.

Proposal 3a: Solution 3 has Stage 3 specification impact, which is already known from Rel-18 mobile IAB.

Proposal 4: Solution 5 causes the enhancement on the BH-RAN node to perform access control during HO.

Proposal 5: Solution 2 and Solution 3 are selected to avoid the multi-hop WAB.

Proposal 5a: Proposal 5 needs to consult with RAN2.

Proposal 6: The BH-RAN node should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 6a: The WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure.

Proposal 7: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format.

Proposal 8: It is proposed to send a replay LS in Appendix to SA2.

Proposal 9: The Xn interface between WAB-gNBs can be established.

Proposal 10: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 11:, The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent reinitiation of the Xn Setup procedure towards the same receiving WAB-gNB.

Proposal 12: It is proposed to agree the corresponding TPs in [7].

4.	Reference

- R3-244019/S2-2407345, “LS on questions regarding FS_VMR_Ph2,” Samsung, May, 2024.

- Chair’s note on RAN3#125bis, October, 2024.

- S2-2411238, “Support of UE served by a MWAB: authorization aspects,” Nokia, LG Electronics, Huawei, ZTE, Samsung, Ericsson, Qualcomm Inc, October, 2024.

- S2-2411236, “Support of UE served by a MWAB: Additional ULI,” Nokia, Qualcomm Inc., Ericsson, Samsung, October, 2024.

- S2-2411114, “Procedure of supporting additional ULI,” ZTE, Ericsson, Samsung, October, 2024.

- R3-245840, “Support for Wireless Access Backhaul,” Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, October, 2024.

- R3-24xxxx, “(TP to TS 38.401 and 38.423) TP for WAB support,” LG Electronics, November, 2024.

5.	Appendix: Reply LS for S2-2407345

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #126 R3-24xxxx Orlando, USA, 18 – 22 November 2024 # Title: [Draft] Reply LS on questions regarding FS_VMR_Ph2 # Response to: R3-244019/S2-2407345 # Release: Rel-19 # Work Item: FS_VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2, RAN2 Cc: - Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: none # 1 Overall description RAN3 thanks SA2 for the LS on questions regarding FS_VMR_Ph2. RAN3 would like to provide the following feedback to SA2 regarding two open issues. Also, RAN3 has captured the conclusions on WAB in TR 38.799. The study item on WAB is now completed in RAN3. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. RAN3’s Answer for Case a): RAN3 has discussed the aspect on how to prevent multi-hop backhauling and then confirms that SA2 solution (i.e., combination of MWAB-gNB indication in NG SETUP REQUEST message and dedicated S-NSSAI(s)/DNN for MWAB operation) can be used to prevent multi-hop backhauling. To support this SA2 solution, it is also agreed to newly introduce a MWAB-gNB indication in NG SETUP REQUEST message. In order to prevent unnecessary access trials of the MWAB-UE towards the MWAB-gNB, RAN3 also considers that it is beneficial for the MWAB-gNB to broadcast a new indication that this is a MWAB cell in SIB. Hence, RAN3 would like to request RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. RAN3’s Answer for Case b): RAN3’s view is that this can be managed by e.g. MWAB implementation. That is, the MWAB-UE does not report the measurement results for the co-located MWAB-gNB to the BH-gNB. In addition, the MWAB-gNB provides the information about co-location of MWAB-gNB and MWAB-UE to BH-gNB during Xn Setup procedure. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06. RAN3’s Answer: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take the above into account. To RAN2: ACTION: RAN3 kindly asks RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#127 17 – 21 February 2025 Greece, Athens 3GPP TSG RAN3#127bis 07 – 11 April 2025 China, TBD

-----------------End of the First Change---------------