3GPP TSG-RAN WG3 Meeting #126	R3-247724

Orlando, USA, 18 – 22 November 2024

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:			(TP for TS 38.300) Functional split and Access control on 5G Femto

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on additional topological enhancements for NR was agreed at RAN #105 [1]. One of the objectives in this WID is to specify overall architecture and access control for NR Femto is necessary as follows:

… The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed. …

In this contribution, we will discuss the impacts on RAN architecture aspects for NR Femto including the access control and functional split, and provide our view on it.

2.	Discussion

In last meeting, RAN3 captured the NR Femto architecture with optional NR Femto GW in [2]. There are two Editor’s notes on protocol stacks for NG control plane in Clause 4.X.3.2. First FFS is whether the NG protocol stack for NR Femto without the NR Femto GW needs to be captured or not. Direct connection of the NR Femto node to the 5GC via the NG interface is already supported by current architecture without any enhancements. Therefore, Figure 4.3.1.2-1 of TS 38.300 can be reused for the NG-C protocol stack without the NR Femto GW.

Figure 1: NG-C Protocol Stack in TS 38.300

Therefore, it seems that there is no need to capture the duplicated figure for the NG-C protocol stack without the NR Femto GW in Clause 4.X.3.2.

Proposal 1: It is proposed to remove Figure 4.X.3.2-1.

Another editor’s note is how to capture the following sentences in detail in the functional split section.

- When the NR Femto GW is not present (Fig. 4.x.3.2-1), all the NGAP procedures are terminated at the NR Femto and the AMF.

- Any protocol function associated to a UE-associated procedure shall reside within the NR Femto and the AMF only.

For ST1, we think that it needs to be captured in the functional split section in order to show the difference between the functions hosted in the NR Femto node with and without the NR Femto GW.

Proposal 2: It is proposed to capture the following sentence as a NOTE in the functional split section.

- NOTE: When the NR Femto GW is not deployed, all the NGAP procedures are terminated at the NR Femto and the AMF.

According to the functional split in LTE, the detailed function description associated to a UE-associated procedure in HeNB GW is already captured in TS 36.300 as follows:

- Relaying UE-associated S1 application part messages between the MME serving the UE and the HeNB serving the UE.

The above sentence in TS 36.300 can be used to describe the function associated to a UE-associated procedure in the NR Femto node with the NR Femto GW. Also, for the NR Femto node without the NR Femto GW, this can be covered by ST1. Therefore, it seems that there is no need to capture ST2 in the functional split section.

Proposal 3: It is proposed to remove the following sentence in the interface section.

- NOTE: Any protocol function associated to a UE-associated procedure shall reside within the NR Femto and the AMF only.

As mentioned above, regarding the functional split, the current texts about HeNB GW in TS 36.300 can be used as a baseline for NR Femto GW. However, there are some stage 3 related descriptions in TS 36.300. In HeNB, the GUMMEI from the UE is used to select the MME during the Attach procedure. In the NR Femto, the GUAMI in the RRCSetupComplete message needs to be provided to the NR Femto GW for AMF selection. That is, the GUAMI is used instead of the GUMMEI. Also, the AMF UE NGAP IDs assigned by AMF and NR Femto GW needs to be included in NGAP signaling such as the INITIAL CONTEXT SETUP REQUEST message and HANDOVER REQUEST message. Then, we think that these texts can be further updated later after the Stage 3 discussion on NR Femto.

Proposal 4: It is proposed to capture the functional split descriptions for NR Femto based on the HeNB.

According to the objectives of NR Femto WID, the stage 2 descriptions on access control for NR Femto should be captured. For now, we think that current descriptions on access control for PNI-NPN in Clause 16.7.4 of TS 38.300 can be reused without any modifications. Therefore, in order to avoid unnecessary duplication, it is better to just add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300. In order to show that three access modes (i.e., open, hybrid, closed) equivalent to those in 4G can be also supported by existing CAG mechanism, we propose to capture the corresponding texts in Clause 5.3 of TR 38.799 into Appendix of stage 2 specification.

Proposal 5: For access control, it is proposed to add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300.

Proposal 6: For access control, it is proposed to capture the corresponding descriptions on three access modes (i.e., open, hybrid, closed) in TR 38.799 into stage 2 spec.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 7: It is proposed to agree the corresponding TPs in Appendix.

3.	Conclusion

In this contribution, we focused on the impacts on RAN architecture aspects for NR Femto including the access control and functional split, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to remove Figure 4.X.3.2-1.

Proposal 2: It is proposed to capture the following sentence as a NOTE in the functional split section.

- NOTE: When the NR Femto GW is not deployed, all the NGAP procedures are terminated at the NR Femto and the AMF.

Proposal 3: It is proposed to remove the following sentence in the interface section.

- NOTE: Any protocol function associated to a UE-associated procedure shall reside within the NR Femto and the AMF only.

Proposal 4: It is proposed to capture the functional split descriptions for NR Femto based on the HeNB.

Proposal 5: For access control, it is proposed to add a reference to access control for PNI-NPN in Clause 16.7.4 of TS 38.300.

Proposal 6: For access control, it is proposed to capture the corresponding descriptions on three access modes (i.e., open, hybrid, closed) in TR 38.799 into stage 2 spec.

Proposal 7: It is proposed to agree the corresponding TPs in Appendix.

4.	Reference

- RP-242395, “New WID on additional topological enhancements for NR,” NTT DOCOMO, INC., AT&T, September, 2024.

- R3-247030, “(BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects,” Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, October, 2024.

5.	Appendix: TP for TS 38.300

This appendix provides the Text proposal for TS 38.300 based on the proposals of this contribution.

----------------Start of the First Change---------------

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC.

Figure 4.X.1-1: NR Femto Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW.

For NR Femto, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto and the NR Femto GW;

-	Between the NR Femto and the Core Network;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW.

### 4.X.2	Functional Split

A NR Femto node hosts the same functions as an gNB as described in clause 4.X.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	A NR Femto node shall only connect to a single NR Femto GW at one time, namely no NG Flex function shall be used at the NR Femto node:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

-	The TAC and PLMN ID used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE registration is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUAMI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message; upon reception of the GUAMI Type from the UE, the NR Femto node shall also include it in the message if supported and supported by the NR Femto GW.

-	NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

-	Signalling the GUAMI of the Source AMF to the NR Femto GW in the NGAP PATH SWITCH REQUEST message.

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto node serving the UE, except the UE CONTEXT RELEASE REQUEST message received from the NR Femto node with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the NGAP UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by an NR Femto node attached to it. Otherwise it ignores the message.

-	In case of NGAP INITIAL CONTEXT SETUP REQUEST message and NGAP HANDOVER REQUEST message, informing the NR Femto node about any GUAMI corresponding to the serving AMF, the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE. In case of NGAP PATH SWITCH REQUEST ACKNOWLEDGE message, informing the NR Femto node about the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF.

-	Upon receiving an OVERLOAD START/STOP message, the NR Femto GW should send the OVERLOAD START/STOP message towards the NR Femto node(s) including in the message the identities of the affected AMF node. The NR Femto node uses this information received from the OVERLOAD START message to identify to which traffic the above defined rejections shall be applied. The NR Femto node shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto node receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

NOTE:	When the NR Femto GW is not deployed, all the NGAP procedures are terminated at the NR Femto node and the AMF.

-	Optionally terminating NG-U interface with the NR Femto node and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

-	Routing the NGAP PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto node.

A list of CAG IDs may be included in the PAGING message. If included, the NR Femto GW may use the list of CAG IDs for paging optimisation.

In addition to functions specified in clause 4.X.1, the AMF hosts the following functions:

-	Access control for UEs that are members of Closed Access Groups (CAG):

-	CAG information signalling to the NG-RAN in case of registration/handover and in case of the change of CAG information.

-	Supervising the NG-RAN action after the change in the CAG information of a UE.

### 4.X.3	Interfaces

#### 4.X.3.1	Protocol Stack for NG User Plane

Editor’s note: Description FFS.

#### 4.X.3.2	Protocol Stacks for NG Control Plane

When the NR Femto GW is not deployed, the NG-C protocol stack is shown on Figure 4.3.1.2-1.

The following figure shows the NG-C protocol stack with the NR Femto GW.

Figure 4.X.3.2-1: Control plane for NG Interface for NR Femto to AMF with the NR Femto GW

### 4.X.4	Access control

Existing access control mechanisms (e.g. CAG control) as described in clause 16.7.4 can be used to manage the UE(s)'s access to the NR Femto node. The open, hybrid and closed access mode can be also supported for the access control of the UEs served by NR Femto node.

<<<<<< NEXT CHANGE >>>>>>

Annex G (informative):
Components of Mobility Latency

Mobility interruption time for L1/L2 Triggered Mobility (LTM) is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

The latency of the mobility procedure for LTM is characterized by the terms illustrated in Figure G-1.

Figure G-1: Components of Mobility Latency for LTM

Each component of mobility latency is described in the table below, the values of which are specified in clause 6.3 of TS 38.133 [13]. The value of TRRC is specified in clause 12 of TS 38.331 [12].

Table G-1: Components of Mobility Latency

| Component | Meaning |
| --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) |
| TLTM-RRC-processing | Early ASN.1 decoding and validity/compliance check |
| TLTM-Processing | UE processing including applying target cell parameters and L1/L2 changes |
| Tfirst-RS | Time for fine tracking and acquiring full timing information |
| TRS-proc | Time for SSB processing |
| TLTM IU | Interruption uncertainty in acquiring the first UL transmission |
| TRAR | Time for RAR delay |
| Tfirst-data | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR |

The latency of the RACH-based and RACH-less LTM procedure is characterized by the terms illustrated in Figure G-2 and Figure G-3. In particular, Figure G-2 shows the LTM procedure where UE activates a TCI state of an LTM candidate cell before performing an LTM cell switch which involves a random access procedure. Further, Figure G-3 shows the LTM procedure when the UE performs the UL and DL synchronization before performing an LTM cell switch which does not involve any random access. In both figures, it is highlighted how the overall mobility latency of LTM can be reduced by early synchronization procedure. Further details on the UE processing timer of the delay components are specified in clause 6.3 of TS 38.133[13].

Figure G-2: Mobility Latency for RACH-based LTM

Figure G-3: Mobility Latency for RACH-less LTM

Annex H (informative):
Access modes in NR Femto node

In the NR Femto node, the open, hybrid and closed access mode which are equivalent to those in HeNB  can be supported for the access control of the UEs served by NR Femto node as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList IE in the SIB1, but without the cellReservedForOtherUse IE as specified in TS 38.331 [12]. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell as specified in TS 38.331 [12].

Annex I (informative):
Change history

-----------------End of the First Change---------------