3GPP TSG RAN WG3 125-bis	R3-247750

Orlando, USA, Nov 18th –22nd, 2024

Title: 	Data Collection Aspects for AIML support in CCO

Source: 	Jio

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During the recent RAN3#125-bis meeting, which focused in main aspects on the Study Item (SI) for AI/ML applications in RAN Release 19, key agreements were reached regarding a proactive approach to Coverage and Capacity Optimization (CCO), as detailed in TR 38.743 [1]. The proactive approach leverages AI/ML to predict potential coverage and capacity (CCO) issues and challenges, enabling future CCO states as preventive measures to be taken within a time range, before issues escalate into serious network failures.

Following the 3GPP TS 38.401[2] in sub-clause 7.9.3: “In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.” the CCO reactively and proactively detection functions are located in gNB CU. However there is no reference to the corresponding data collection and inputs for the future CCO state gNB DU local actions.

To effectively support AI/ML-driven CCO, extensive data collection from the network is essential. This data is then fed into AI/ML models, allowing them to learn, adapt, and predict potential issues. The quality and quantity of this data are critical for ensuring accurate predictions, making real-time monitoring and feedback loops vital. By having a rich dataset, AI/ML algorithms can identify early warning signs of network strain, enhancing the network’s ability to take proactive action and maintain optimal performance before any degradation affects users.

In this Discussion contribution, we emphasize and provide further discussion related to AI/ML based CCO data collection aspects.

# 2. Discussion

## Data collection for AI/ML based CCO

Following the discussions in RAN#125-bis meeting, the gNB-CU collects a range of UE measurements, which are primarily used to detect existing CCO issues and reactively act on them. We believe, however, that the coverage and the capacity (mainly due to interference issues) are two different scenarios with different signaling needs, data collection and feedback needs and actions.

For example, UE measurements taken over time at the edge of a cell can reveal trends that indicate an impending issue. If these measurements, as used for AI/ML gNB CU training, show a consistent increase in interference and decreasing rate in the SINR, might lead to a proper inference conclusion on an interference problem at the cell edge. gNB CU could signal that a CCO problem related to capacity is likely to be developped for specific cells or beams in a specific time frame. By identifying these trends early, AI/ML models can predict potential CCO capacity issues before they impact the network, triggering future CCO states and thus allowing for proactive adjustments to prevent network’s performance degradation. This expanded use of UE measurements offers a valuable tool in transitioning from reactive CCO solutions to a predictive, AI-driven proactive strategy.

Observation 1: Data collection strategy, as described in a reactive CCO issue in Release 18, could be reused for the AI/ML proactive CCO necessary actions related to CCO capacity (interference) issues.

However predicting CCO issues related to coverage can be particularly challenging, as it often requires more detailed and dynamic measurements than what is typically available in the reactive approach. One example involves UEs located close to the border coverage between two cells, for instance, between a cell served by gNB1 and a neighboring cell served by gNB2. In such cases, the cell selection and the corresponding connected mode service coverage of certain UEs depend on several different conditions which are out of the scope of this discussion, nevertheless we can assume that certain UEs connected to gNB2 will report coverage measurements (low RSRP or even Radio Link Failure (RLF) detection or unsynchronization) to gNB2 and gNB1 will or may be completely unaware of this problem and how those same UEs would ever perceive coverage from its own cell. This creates a blind spot for each gNB regarding the coverage conditions at the cell edges.

Consider then a scenario where several UEs are positioned near the border between gNB1 and gNB2 coverage. These UEs, assuming that are currently served by gNB2, might report to gNB2 a marginal coverage, even though they are technically within the coverage range of gNB1, which indicates a potential mobility issue (handover not triggered) and misconfiguration of mobility parameters (wrong settings of thresholds). On the other hand gNB1 remains unaware of this issue because it does not receive any data from UEs connected to gNB2. If these UEs experience a sudden degradation in coverage, such as due to interference or load imbalance, it might lead to a coverage hole or unnecessary handovers, impacting the user experience.

This unlike situation could potentially be resolved by two different approaches for both split and non-split architecture:

Approach 1: we assume there is not any prior signaling feedback between gNB2 and gNB1, but rather certain measurement and data collection cycle as described in the following steps:

- allow for the gNB2 UEs to report the predicted problem,

- gNB2 CU to make a proactive prediction based on UEs reports and collected own data

- gNB 2 CU to forward to gNB2 DU the CCO issue with the proper execution time reference

- gNB2 DU to generate the future CCO state and feedback gNB2 CU with a future CCO state time reference.

- gNB2 CU to inform via Xn interface the gNB1 CU with the future CCO and the corresponding timing info

- gNB1 CU to inform gNB1 DU for the future CCO and the timing info

- both gNBs execute the future CCO state on the same time info range

- gNB2 CU to monitor in another cycle the UEs report and if necessary start another similar action flow

Observation 2: With Approach 1 the network does not allow for feedback measurements between the proactive CCO actions, thus keeping the Xn interface load and the corresponding gNB CU processing load low.

Approach 2:  Assuming feedback signaling between gNB2 and gNB1 before any proactive and future CCO action is taken, this scenario allows the sharing measurements between the two gNBs over Xn interface on behalf of the AI/ML supported CCO functionality, following again certain measurement and data collection cycle as described in the following steps:

- allow for the gNB2 UEs to report the predicted problem,

- gNB2 CU to detect whether the UE reports are coming from cell edge (i.e. via TA)

- gNB2 CU to signal the detected problem to gNB1 CU

- gNB1 CU to prepare a reactive action and forward it to gNB1 DU

- gNB1 DU execute the reactive action and feedback the ack to gNB1 CU

- gNB1 CU to report a feedback to the gNB2 CU for the reactive actions execution

- gNB2 CU to continue collecting data from own UE reports in a new cycle

- If the problem is resolved no further actions are needed

- If the problem is not resolved then continue with the Approach 1 flow of future CCO states, as stated before.

Observation 3: Allowing serving gNB2 to signal to neighbor gNB1 some of the coverage data it receives from UEs at the cell coverage boundary, gNB1 would gain valuable insight into how its own coverage is perceived from the perspective of UEs connected to gNB2.

With this knowledge, gNB1 might be able to make small adjustments, such as optimizing the beamforming patterns, adjusting azimuth and elevation beam angles, changing SSB beam power etc, to slightly expanding the coverage area of its cell. Similarly, gNB2 could also adjust its coverage in response to measurements from UEs on its border with gNB1, thereby avoiding coverage gaps or overlaps.

Observation 4: Allowing serving gNB2 to signal to neighbor gNB1 some of the coverage data it receives from UEs at the cell coverage boundary, would give the opportunity for gNB1 to reactively act on the problem and potentially solve it before any proactive long solution would take place.

Such coordination between gNB CUs is critical because the channel conditions faced by UEs connected to one gNB might differ significantly from those connected to the neighboring gNB. There is no direct reciprocity between the channel conditions for UEs served by different gNBs, meaning that each gNB cannot rely solely on its own measurements to infer the experience of UEs served by the other gNB. By exchanging this cross-border measurement data, both gNB1 and gNB2 can more accurately predict and mitigate reactively potential CCO issues related to coverage, ensuring a more seamless user experience and reducing the likelihood of coverage problems at the cell edges and the proactively taken actions.

Proposal 1: For a reactive AI/ML approach, before any proactiveCCO state is declared and appropriate future CCO state is produced, approach 2 with proper feedback is proposed.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: Data collection strategy, as described in a reactive CCO issue in Release 18, could be reused for the AI/ML proactive CCO necessary actions related to CCO capacity (interference) issues.

Observation 2: With Approach 1 the network does not allow for feedback measurements between the proactive CCO actions, thus keeping the Xn interface load and the corresponding gNB CU processing load low.

Observation 3: Allowing serving gNB2 to signal to neighbor gNB1 some of the coverage data it receives from UEs at the cell coverage boundary, gNB1 would gain valuable insight into how its own coverage is perceived from the perspective of UEs connected to gNB2.

Observation 4: Allowing serving gNB2 to signal to neighbor gNB1 some of the coverage data it receives from UEs at the cell coverage boundary, would give the opportunity for gNB1 to reactively act on the problem and potentially solve it before any proactive long solution would take place.

Proposal 1: For a reactive AI/ML approach, before any proactiveCCO state is declared and appropriate future CCO state is produced, approach 2 with proper feedback is proposed.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)

[2] 3GPP TS 38.401