3GPP TSG RAN WG3 Meeting #126	R3-247752

Orlando, U.S.A., 18 - 22 November, 2024

Title: 	Response to R3-247597 and R3-247599

Source: 	ZTE Corporation

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this RAN3 meeting, RAN3 received a reply LS (R3-247010/ R2-2409280) from RAN2 as below, which is to reply the original LS (R3-245025/ S5-245138) from SA5.

R3-245025/ S5-245138 1 Overall description The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission. R3-247010/ R2-2409280 1. Overall Description: RAN2 would like to thank SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause. However, RAN2 would also like to inform SA5 that occasionally an SDT procedure may be unsuccessfully completed due to cell re-selection during SDT procedure, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold or integrity check failure while SDT procedure is ongoing. In these cases, the network would not be aware that the UEs SDT session is already terminated at the UE side, but would consider SDT session as ongoing until timer T319a expires. T319a is started when the SDT session starts and its maximum value is 4 seconds.

In the last RAN3 125bis meeting, RAN3 agreed to delay to discuss this issue pending to RAN2 progress.

In this meeting, two contributions (R3-247597/ R3-247599) are submitted to discuss this issue based on the RAN2’s reply LS (R3-247010/ R2-2409280).

R3-247597 2 Conclusion and Proposals This paper has analysed the proposal received in R3-247010 to count the number of RRC_INACTIVE UEs in a cell with SDT transmission and made the following observations: Observation 1: RAN3 is responsible for the O&M requirements and to decide the relevance of the request counting. Observation 2: it is not possible to know the number of RRC_INACTIVE UEs in a cell configured with SDT. Observation 3: The contribution of SDT traffic to cell load status is negligible compared to the overall traffic. The analysis leads to the following proposals: Proposal 1: RAN3 to conclude negatively on introducing a new counter for number of RRC_INACTIVE UEs in a cell with SDT transmission. Proposal 2: send RAN3 conclusion and guidance to SA5 as per the reply LS provided in [3]. R3-247599 1. Overall Description: RAN3 has studied the SA5 proposal for counting the number of RRC_INACTIVE UEs per cell with data transmission for the purpose of improving cell load status and would like to provide the following feedback as the responsible group for RAN O&M requirements: It is hardly possible to determine in which cell an SDT configured UE is located since an RNA in general includes many cells. Moreover, the data volume exchanged during SDT transmissions which last less than four seconds is negligible in comparison of overall cell load. RAN3 would prefer avoiding defining such counters.

This response paper further discusses this issue, considering the above contributions (R3-247597/ R3-247599).

- Discussion

R3-247597 Observation 1: RAN3 is responsible for the O&M requirements and to decide the relevance of the request counting. Proposal 2: send RAN3 conclusion and guidance to SA5 as per the reply LS provided in [3].

We have the same view with above in R3-247597, i.e., RAN3 group shall be involved and shall send reply to SA5 for this issue.

Proposal 1: RAN3 shall be involved into this issue and send RAN3’s view to the SA5.

R3-247597 Observation 2: it is not possible to know the number of RRC_INACTIVE UEs in a cell configured with SDT. Observation 3: The contribution of SDT traffic to cell load status is negligible compared to the overall traffic. The analysis leads to the following proposals: Proposal 1: RAN3 to conclude negatively on introducing a new counter for number of RRC_INACTIVE UEs in a cell with SDT transmission.

However, we have the different view from above in R3-247597.

The intention of the LS is to understand whether it is possible to define a mechanism for the network to count the number of UEs in RRC_INACTIVE state with data transmission (i.e. the number of UEs with SDT ongoing as clarified in RAN2 reply LS. It should be noted that this is not the same as the number of UEs configured with SDT) as one of the network KPIs. It means that SA5 wishes to define a mechanism for counting the number of UEs with ongoing SDT rather than number of UES with SDT configuration.

R3-245025/ S5-245138

1 Overall description

The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.

Proposal 2: RAN3 shall provide its view on SDT with data transmission, other than SDT configuration.

When SDT is initiated, the UE sends RRCResumeRequest to the network. SDT procedure is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signalling in RRCRelease). Thus, for MO-SDT, the network knows the SDT cause either by the RACH resource used for sending the RRCResumeRequest or by the reception of RRCResumeRequest via the type 1 CG resource configured for SDT.

R3-247010/ R2-2409280

1. Overall Description:

RAN2 would like to thank SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause. However, RAN2 would also like to inform SA5 that occasionally an SDT procedure may be unsuccessfully completed due to cell re-selection during SDT procedure, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold or integrity check failure while SDT procedure is ongoing. In these cases, the network would not be aware that the UEs SDT session is already terminated at the UE side, but would consider SDT session as ongoing until timer T319a expires. T319a is started when the SDT session starts and its maximum value is 4 seconds.

More, the following agreement is abstracted from RAN2’s Chair note.

RAN2 assumes that we are referring to “ongoing” SDT procedure and the network knows the number of UEs in a cell performing “ongoing” SDT.

When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest. Thus, the network can keep track of number of UEs in a given cell that have SDT ongoing at any given point in time in the cell, both for MT-SDT and MO-SDT cases.

Observation 1: RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session.

From RAN3 point view, no new mechanism is needed to keep count of this on the network side and no new UE procedure is needed either for this.

Proposal 3: From RAN3 point of view, it is possible for the network to provide the number of UEs in a given cell that have SDT with data transmission (both for MO-ST and MT-SDT) at any given point in time in the cell.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: RAN3 shall be involved into this issue and send RAN3’s view to the SA5.

Proposal 2: RAN3 shall provide its view on SDT with data transmission, other than SDT configuration.

Observation 1: RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session.

Proposal 3: From RAN3 point of view, it is possible for the network to provide the number of UEs in a given cell that have SDT with data transmission (both for MO-ST and MT-SDT) at any given point in time in the cell.

Based on the above observation and proposals, we further provide the corresponding reply LS to SA5.

- Draft reply LS

===================The Start of the reply LS===============================================

3GPP TSG RAN WG3 Meeting #126	R3-24xxxx

Orlando, U.S.A., 18 - 22 November, 2024

Title:	[DRAFT] Reply LS on Number of UEs in RRC_INACTIVE state with data transmission

Response to:	R3-245025 / S5-245138

Release:	Rel-19

Work Item:	PM_KPI_5G_Ph4

Source:	ZTE (to be RAN3)

To:	SA5

Cc:	RAN2

Contact Person:

Name:	Zijiang Ma

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	 3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission.

RAN3 observes that RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session.

From RAN3 point of view, it is possible for the network to provide the number of UEs in a given cell that have SDT with data transmission (both for MO-ST and MT-SDT).

2. Actions:

To SA5 group.

ACTION: 	RAN3 kindly asks SA5 to take the above feedback into consideration.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127		17 - 21 February 2025	Athens, GR

TSG-RAN3 Meeting #127bis	 07 - 11 April 2025		Wuhan, China

===============The End of the reply LS===============================================