3GPP TSG RAN WG3 Meeting #126	R3-247786

Orlando, U.S.A., 18 - 22 November, 2024

Title:	(TP to TR 38.769) Signaling and Procedures for NAS/UP based Topology 2

Agenda Item:	16.3

Source:	ZTE Corporation, Huawei, Xiaomi, NEC, Nokia, China Telecom, CATT, CMCC

Document for:	other

- Introduction

In the last RAN3 #125bis meeting, we have agreed to capture Signaling and Procedures for Topology 1 and for RRC based Topology 2 into TR 38.769 (R3-245849).

This TP is used to capture Signaling and Procedures for NAS/UP based Toplogy 2.

2	Text Proposal

-----------Start of the Change------------

6.5.3.1.2	NAS/UP solution

Editor’s Note 1: Future discussions on A-IoT Inventory will take place based on the following message flow, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1-2: Message flow for UE assistant A-IoT Inventory in Topology 2 - NAS/UP based solution is used

1.	The AIoT CN sends an Inventory request message to the selected AIoT-enabled UE.

1a. The AIoT enabled UE request AIoT radio resources.

1b. The AIoT enabled gNB allocates and coordinates AIoT radio resources.

1c. The AIoT enabled gNB responds the AIoT radio resources to the AIoT-enabled UE.

NOTE 1:	 In step 1c, the AIoT-enable gNB can reject the AIoT radio resource request.

2.	The AIoT-enabled UE sends an Inventory response message to the AIoT CN.

NOTE 2: In step 2, the AIoT enabled UE may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s), and the procedure ends.

3.	The AIoT-enabled UE(s) performs the inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4/4b.	After receiving inventory result reported from the AIoT device(s), the AIoT-enabled UE may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 3: Steps 4/4b may happen in parallel with Step 3 for different AIoT devices.

Figure 6.5.3.1-3: Message flow for CN assistant A-IoT Inventory in Topology 2 - NAS/UP based solution

0a.	The AIoT CN requests AIoT radio resources.

0b	The AIoT enabled gNB allocates and coordinates AIoT radio resources.

0c/d.The AIoT enabled UE is configured with AIoT resources it is allowed to use. Respective RRC signalling needs to carry information to allow association with information provided in step 1.

NOTE 1: The step 0c/0d can be skipped, if AIoT enable gNB sends the configured AIoT radio resources to the AIoT CN in step 0e.

0e.	The AIoT enabled gNB responds the AIoT radio resources to the AIoT CN.

NOTE 2: In step 0e, the AIoT-enable gNB can reject the AIoT radio resource request, and the procedure ends.

1.	The AIoT CN sends an Inventory request message to the AIoT-enabled UE, including the configured AIoT radio resources if step 0c/0d are skipped.

2.	The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE 3: In step 2, the AIoT-enabled UE may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s), and the procedure ends.

3.	The AIoT-enabled UE(s) trigger the inventory procedure towards the AIoT device(s).

4/4b.	After receiving inventory result reported from the AIoT device(s), the AIoT-enabled UE may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 4: Steps 4/4b may happen in parallel with Step 3 for different AIoT devices.

Editor’s Note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.