3GPP TSG-RAN WG3 Meeting #126	R3-247885

Orlando, USA, 18th – 22th Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1524 | rev | 2 | Current version: | 18.3.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Positioning SRS Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, ZTE,CATT, China Unicom, Ericsson, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Periodicity up to 20480ms was agreed to be supported with a hyperSFN offset equal to 0 or 1 in Rel-18. Thesrs-PosPeriodicConfigHyperSFN-IndexIE in TS38.331 indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms.The SRS Periodicity IE defined in clause 9.3.1.342 of TS38.473 is also support 20480ms in Rel-18. Therefore, it is needed to add srs-PosPeriodicConfigHyperSFN-Index in Position SRS Resource IE and Validity Area specific SRS Information IE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Toadd new IESRS PosPeriodicConfigHyperSFNIndexinPositioning SRS Resource IE - To add new IE SRS PosPeriodicConfigHyperSFN Index in Validity Area specific SRS Information IE - Update the corresponding ASN.1 part. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release).This CR only has an impact on Positioning function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SRS Postioning with 20480ms periodicity could not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.194, 9.3.1.339, 9.3.5,9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.455 CR0177 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Update WI code, semantics description, title and ASN.1 part. Addition of co-signers Rev 2: Update title, color of IE Type and ASN.1 part |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.3.1.194	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource ID | M |  | INTEGER (0..63) |  | - |  |
| CHOICE Transmission Comb Positioning | M |  |  |  | - |  |
| >Comb Two |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >Comb Four |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >Comb Eight |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  | - |  |
| Start Position | M |  | INTEGER(0..13) |  | - |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4,8,12) |  | - |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  | - |  |
| C-SRS | M |  | INTEGER(0..63) |  | - |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  | - |  |
| CHOICE Resource Type Positioning | M |  |  |  | - |  |
| >Periodic |  |  |  |  |  |  |
| >>Periodicity | M |  | SRS Periodicity 9.3.1.342 |  | - |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms. | YES | ignore |
| >Semi-persistent |  |  |  |  |  |  |
| >>Periodicity | M |  | SRS Periodicity 9.3.1.342 |  | - |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms. | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Slot offset | M |  | INTEGER(0..32) |  | - |  |
| Sequence ID | M |  | INTEGER(0..65535) |  | - |  |
| CHOICE Spatial Relation Positioning | O |  |  |  | - |  |
| >SSB |  |  |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>SSB index | O |  | INTEGER(0..63) |  | - |  |
| >PRS |  |  |  |  |  |  |
| >>PRS ID | M |  | INTEGER(0..255) |  | - |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  | - |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  | - |  |
| Tx Hopping Configuration | O |  | 9.3.1.343 |  | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.3.1.339	Validity Area specific SRS Information

This information element is used to indicate the SRS information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Transmission Comb | O |  |  |  | - |  |
| >Comb Two |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >Comb Four |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >Comb Eight |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  | - |  |
| Resource Mapping |  | 0..1 |  |  |  |  |
| >Start Position | M |  | INTEGER(0..13) |  | - |  |
| >Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12} |  | - |  |
| Frequency Domain Shift | O |  | INTEGER(0..268) |  | - |  |
| C-SRS | O |  | INTEGER(0..63) |  | - |  |
| CHOICE Resource Type Positioning | O |  |  |  | - |  |
| >periodic |  |  |  |  |  |  |
| >>Periodicity | M |  | 9.3.1.342 |  | - |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms. | YES | ignore |
| >semi-persistent |  |  |  |  |  |  |
| >>Periodicity | M |  | 9.3.1.342 |  | - |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms. | YES | ignore |
| >aperiodic |  |  |  | Not applicable if the Positioning Validity Area Cell List IE is included. |  |  |
| >>slot offset | M |  | INTEGER(0..32) |  | - |  |
| Sequence ID | O |  | INTEGER(0..65535) |  | - |  |

/////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-SRSPosPeriodicConfigHyperSFNIndex,

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### -- R

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

ResourceTypePos ::= CHOICE {

periodic			ResourceTypePeriodicPos,

semi-persistent		ResourceTypeSemi-persistentPos,

aperiodic			ResourceTypeAperiodicPos,

choice-extension	ProtocolIE-SingleContainer{{ ResourceTypePos-ExtIEs }}

}

ResourceTypePos-ExtIEsF1AP-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodicPos ::= SEQUENCE {

periodicity			SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer{ {ResourceTypePeriodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypePeriodicPos-ExtIEsF1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSPosPeriodicConfigHyperSFNIndex			CRITICALITY ignore	EXTENSION SRSPosPeriodicConfigHyperSFNIndex		PRESENCE optional	},

...

}

ResourceTypeSemi-persistentPos ::= SEQUENCE {

periodicity			SRS-Periodicity,

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistentPos-ExtIEs} }	OPTIONAL

}

ResourceTypeSemi-persistentPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSPosPeriodicConfigHyperSFNIndex			CRITICALITY ignore	EXTENSION SRSPosPeriodicConfigHyperSFNIndex		PRESENCE optional	},

...

}

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset			INTEGER (0..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### -- S

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# SRSConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

# SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions		ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL

}

# SRSConfiguration-ExtIEsF1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AggregatedPosSRSResourceSetList	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceSetList	PRESENCE optional},

...

}

# SrsFrequency ::=INTEGER(0..3279165)

# SRSPortIndex ::=ENUMERATED {id1000, id1001, id1002, id1003,...}

# SRSPosPeriodicConfigHyperSFNIndex ::=ENUMERATED {even0, odd1}

# SRSPosResourceID ::= INTEGER (0..63)

# SRSPreconfiguration-List ::= SEQUENCE (SIZE (1.. maxnoPreconfiguredSRS)) OF SRSPreconfiguration-Item

# SRSPreconfiguration-Item ::= SEQUENCE {

sRSPosRRCInactiveValidityAreaConfig		SRSPosRRCInactiveValidityAreaConfig,

posValidityAreaCellList				PosValidityAreaCellList,

iE-Extensions			ProtocolExtensionContainer{{ SRSPreconfiguration-Item-ExtIEs}}			OPTIONAL,

...

}

# SRSPreconfiguration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

9.4.7	Constant Definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

id-RANSharingAssistanceInformation					ProtocolIE-ID ::=844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= xxx

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////