3GPP TSG-RAN WG3 Meeting #126	R3-247891

Orlando, USA, 18-22 November, 2024

Title:	LS on PDCP SN gap report handling during UE mobility

Response to:

Release:	Rel-18

Work Item:	NR_XR_enh-Core

Source:	RAN3

To:	RAN2

Cc:

# Contact Person:

# Name:	Yuanping Zhu

# Email:		zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

For Rel-18 XR, it is unclear how to handle the PDCP SN gap report during UE’s inter-gNB mobility. If a UE discarded some UL packets before handover, the UE reports the PDCP SN gap report to the source gNB, but the target gNB may not be aware of those discarded packets if no PDCP SN gap is reported to target gNB. This may result in unnecessary delay in the target gNB.

The following solutions were discussed in RAN3:

- Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB.

- Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB.

- Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

RAN3 would like to ask RAN2 to evaluate whether Alternative 3 can be supported.

# 2. Actions:

# To RAN2:

ACTION: RAN3 kindly asks RAN2 to provide feedback on whether Alternative 3 can be supported.

# 3. Date of Next RAN3 Meetings:

RAN3#127	17th Feb – 21st  Feb, 2025				Athens, GR

RAN3#127bis	07th April – 11th April, 2025				China, CN