3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411118

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on BS demodulation performance for MMSE-IRC

Agenda item:	8.16.3

Document for:	Approval

- Introduction

In WID [1] on Demodulation performance requirements enhancements, the objective of BS demodulation performance for MMSE-IRC is shown as below.

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

For objective of BS MMSE-IRC demodulation in WID, we discuss definition of Dominant interferer proportion (DIP), SINR definition, interference model, test parameter, and metric for BS MMSE-IRC demodulation as below:

Definition of Dominant interferer proportion (DIP)

To take account for simultaneously transmitted interferers from inter-cell sources, the definition of dominant interferer proportion (DIP) is needed. The definition of DIP in sub-clause B.6.1 of TS 36.104 copied as below.

## B.6.1 Dominant interferer proportion Each inter-cell interferer involved in enhanced performance requirements type A and type B is characterized by its associated dominant interferer proportion (DIP) value: ( = 1,…, ) where is the received energy from the i-th strongest inter-cell interferer involved in the requirement scenario and where is the the energy of the white noise source consistent with the definition provided in clause 8.1 and is the total number of simultaneously transmitted inter-cell interferers involved in a given requirement scenario.

Based on above analysis, we propose to reuse the definition of DIP in TS36.104 for NR BS demodulation performance for MMSE-IRC,

Proposal 1:  Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 for NR BS MMSE-IRC demodulation.

SINR definition

For BS demodulation performance for MMSE-IRC, the SINR definition incorporating the above DIP is needed. Generally, the SINR definition in sub-clause 8.1 of TS 36.104 can be taken as a starting point. Compared with eNodeB, there are three different BS types (1-C, 1-H, and 1-O) for NR, each with a different reference point (i.e., antenna connector for BS type 1-C, TAB connector for BS type 1-H, and RIB for BS type 1-O), therefore the antenna point used in the SINR definition for LTE is not applicable for NR, thus the SINR definition for NR need slight modifications to accommodate both conducted demodulation requirements and radiated demodulation requirements.

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a single antenna port. is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna port. The respective energy of each inter-cell interferer relative to is defined by its associated DIP value.

With the above considerations in mind, for conducted demodulation requirement for BS type 1-C and BS type 1-H, we can define the SINR as below:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Similarly, for radiated demodulation requirement for BS type 1-O, we can define SINR as below:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a RIB.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Proposal 2: Define the SINR as below for conducted demodulation requirement for BS type 1-C and BS type 1-H:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Proposal 3: Define the SINR as below for conducted demodulation requirement for BS type 1-O:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a RIB.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Interference model

In sub-clause B.6.2 and B.6.3 of TS 36.104, the interference model for synchronous scenario and interference model for asynchronous scenario were defined. According to WID, only synchronous deployments were considered. So we think that the interference model for synchronous scenario in sub-clause B.6.2 of TS 36.104 can be reused for NR BS MMSE-IRC demodulation.

Proposal 4: Reuse the interference model for synchronous scenario in sub-clause B.6.2 of TS 36.104 for NR BS MMSE-IRC demodulation.

Test parameters

For synchronous scenario, the test parameters for enhanced performance requirement type A for synchronous scenario in Table 8.2.6-1 of TS 36.104 are copied as below:

Table 8.2.6-1: Test parameters for enhanced performance requirement type A

| Parameter |  | Unit | Tested signal | Interferer 1 (Note 1) | Interferer 2 (Note 1) |
| --- | --- | --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  |  | 4 | N/A | N/A |
| RV sequence |  |  | 0, 2, 3, 1, 0, 2, 3, 1 | N/A | N/A |
| DIP (Note 2) | Set 1 | dB | N/A | -1.11 | -10.91 |
|  | Set 2 | dB | N/A | -0.43 | -13.78 |
| Cell Id |  |  | 0 | 1 | 2 |
| Interference model |  |  | N/A | As specified in clause B.6.2 | As specified in clause B.6.2 |
| Cyclic Prefix |  |  | Normal |  |  |
| Uplink-downlink allocation for TDD |  |  | Configuration 1 (2:2) |  |  |
| Demodulation reference signal for PUSCH |  |  | =0, =0, =0 Group hopping and sequence hopping are disabled. |  |  |
| Note 1: One explicit interferer, i.e., interferer 1, is modelled for tests with 2 RX antennas. Two explicit interferers are modelled for tests with 4 or 8 RX antennas. Note 2: The respective received energy of each interferer relative to is defined by its associated DIP value as specified in clause B.6.1. DIP set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. Note 3: All cells are time-synchronous. |  |  |  |  |  |

From above table, The DIP set1 and set2 are important parameters for BS MMSE-IRC demodulation, for LTE, the DIP set1(-1,11, -10,91) and set2(-0,43, -13,78)  were derived respectively in homogeneous and heterogeneous network scenarios with Methodology 1 for statistical measurements in TR 36.884.  It is noteworthy that in TR 36.884, the derivation of DIP set1 and set2 were based on passive antenna model with ITU UMa and UMi propagation model. Although BS types with active antenna (BS type 1-H, 1-O) are defined for NR, BS with passive antenna (BS type 1-C) are also defined for NR. Moreover, from perspective of interference, the interference from passive antenna is larger than that from active antenna, since the narrow beam generated by active antenna can achieve greater interference suppression. Therefore, it is reasonable that reuse the DIP set1 and set2 for LTE BS MMSE-IRC demodulation for NR BS MMSE-IRC demodulation.

Proposal 5: Reuse the DIP set1 and set2 for LTE BS MMSE-IRC demodulation for NR BS MMSE-IRC demodulation as starting point.

Cases for link level evaluations in Table 8.1-3 of TR 36.884 are copied as below, the MCS, propagation condition, antenna configuration and DIP set were defined for LTE BS MMSE-IRC demodulation, and these cases were also defined in TS 36.104.

Table 8.1-3: Cases for link level evaluations

| Case Num | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- |
| 1 | 6 | (EPA5, ETU5) | 1x2 Low | (-0,43, N/A) |
| 2 | 15 | (EPA5, ETU5) | 1x4 Low | (-0,43, -13,78) |
| 3 | 20 | (EPA5, ETU5) | 1x8 Low | (-0,43, -13,78) |
| 4 | 6 | (EVA70, ETU70) | 1x2 Low | (-1,11, N/A) |
| 5 | 15 | (EVA70, ETU70) | 1x4 Low | (-1,11, -10,91) |
| 6 | 20 | (EVA70, ETU70) | 1x8 Low | (-1,11, -10,91) |

From above table, for MCS, the MCS 6, 15, 20 were defined for LTE BS MMSE-IRC demodulation, the MCS 6, 15, 20 are QPSK(R= 0.36), 16QAM(R=0.5), and 16QAM(R=3/4) respectively. When it comes to NR, it is reasonable that the same modulation and code rate as LTE can be reused for NR. According to Table 5.1.3.1-1 of TS 38.214, the MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) for NR are close to MCS 6, 15 for LTE. However there is no 16QAM(R=3/4) defined in NR,we can use MCS 24(64QAM, R=772/1024) instead, where the code rate is close to 3/4 used for LTE. Hence we propose to use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024), and MCS 24(64QAM, R=772/1024) for NR BS MMSE-IRC demodulation.

Proposal 6: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) for NR BS MMSE-IRC demodulation.

From above table, (EPA5, ETU5) and (EVA70, ETU70) are defined for LTE BS MMSE-IRC demodulation, the serving and interferers used different propagation conditions. Referring to B.2 of TS 36.104, maximum tap delay for EPA, EVA, and ETU is 410ns, 2510ns, and 5000ns respectively, and the EPA5 and ETU5 shows that 5Hz maximum Doppler frequency fD are defined, and EVA70, ETU70 shows that 70Hz maximum Doppler frequency fD are defined. When it comes to NR, we can refer to propagation condition for normal PUSCH demodulation, the TDLB100-400 Low, TDLC300-100 Low, TDLA30-10 Low, and TDLA30-10 Low are defined for QPSK, 16QAM, 64QAM and 256QAM respectively, it is reasonable that interferers can use same propagation condition for serving. So we propose,

| MCS | Propagation condition (Serving, interferers) |
| --- | --- |
| MCS 5(QPSK, R=379/1024) | (TDLB100-400 Low, TDLB100-400 Low) |
| MCS 13(16QAM, R=490/1024) | (TDLC300-100 Low, TDLC300-100 Low) |
| MCS 24(64QAM, R=772/1024) | (TDLA30-10 Low, TDLA30-10 Low) |

Proposal 7: Use the following propagation condition for serving and interferers for NR BS MMSE-IRC demodulation.

| MCS | Propagation condition (Serving, interferers) |
| --- | --- |
| MCS 5(QPSK, R=379/1024) | (TDLB100-400 Low, TDLB100-400 Low) |
| MCS 13(16QAM, R=490/1024) | (TDLC300-100 Low, TDLC300-100 Low) |
| MCS 24(64QAM, R=772/1024) | (TDLA30-10 Low, TDLA30-10 Low) |

From above Table 8.1-3, for LTE BS MMSE-IRC demodulation,1x2 Low, 1x4 Low, and 1x8 Low are defined for MCS 6, 15, and 20 respectively. We think that the same method can be reused for NR BS MMSE-IRC demodulation. So we propose,

| MCS | Antenna configuration for serving and interferers |
| --- | --- |
| MCS 5(QPSK, R=379/1024) | 1x2 Low |
| MCS 13(16QAM, R=490/1024) | 1x4 Low |
| MCS 24(64QAM, R=772/1024) | 1x8 Low |

Proposal 7: Use the following antenna configuration for serving and interferers for NR BS MMSE-IRC demodulation.

| MCS | Antenna configuration for serving and interferers |
| --- | --- |
| MCS 5(QPSK, R=379/1024) | 1x2 Low |
| MCS 13(16QAM, R=490/1024) | 1x4 Low |
| MCS 24(64QAM, R=772/1024) | 1x8 Low |

For NR BS normal PUSCH demodulation, the requirement for CP-OFDM (Transform precoding, disabled) and DFT-s-OFDM (Transform precoding, enabled) are defined, so for NR BS MMSE-IRC demodulation, it is reasonable to define requirements for both CP-OFDM and DFT-s-OFDM.

Proposal 8: Define NR BS MMSE-IRC demodulation requirements for both CP-OFDM and DFT-s-OFDM waveform.

For CP-OFDM waveform, the test parameters in Table 8.2.1.1-1, channel bandwidth, and SCS for PUSCH with transform precoding disabled in sub-clause 8.2.1 of TS 38.104 can be reused, which some parameter is not needed and can be removed. The 5/10/20MHz CWB for 15kHz SCS, and 10/20/40/100MHz CBW for 30KHz SCS can be reused for CP-OFDM for NR BS MMSE-IRC demodulation.

Proposal 9: 5/10/20MHz CBW for 15kHz SCS, and 10/20/40/100MHz CBW for 30KHz SCS can be reused for CP-OFDM for NR BS MMSE-IRC demodulation.

According to test parameter for CP-OFDM in Table 8.2.1.1-1, the following parameters can be reused for CP-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Proposal 10: Use the following parameters for CP-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

For DFT-s-OFDM waveform, the test parameters in Table 8.2.2.1-1, channel bandwidth, and SCS for PUSCH with transform precoding enabled in sub-clause 8.2.2 of TS 38.104 can be reused, which some parameter is not needed and can be removed. The 5MHz CWB for 15kHz SCS, and 10MHz CBW for 30KHz SCS can be reused for DFT-s-OFDM for NR BS MMSE-IRC demodulation.

Proposal 11: 5MHz CBW for 15kHz SCS, and 10MHz CBW for 30KHz SCS can be reused for DFT-s-OFDM for NR BS MMSE-IRC demodulation.

According to test parameter for DFT-s-OFDM in Table 8.2.2.1-1, the following parameters can be reused for DFT-s-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Proposal 12: Use the following parameters for DFT-s-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Metric

It is reasonable to use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation. For study, whether 30% of maximum throughput as metric for NR BS MMSE-IRC demodulation needs to be studied.

Proposal 13: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation.

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Proposal 1:  Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 for NR BS MMSE-IRC demodulation.

Proposal 2: Define the SINR as below for conducted demodulation requirement for BS type 1-C and BS type 1-H:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Proposal 3: Define the SINR as below for conducted demodulation requirement for BS type 1-O:

For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a RIB.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

Proposal 4: Reuse the interference model for synchronous scenario in sub-clause B.6.2 of TS 36.104 for NR BS MMSE-IRC demodulation.

Proposal 5: Reuse the DIP set1 and set2 for LTE BS MMSE-IRC demodulation for NR BS MMSE-IRC demodulation as starting point.

Proposal 6: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) for NR BS MMSE-IRC demodulation.

Proposal 7: Use the following antenna configuration for serving and interferers for NR BS MMSE-IRC demodulation.

| MCS | Antenna configuration for serving and interferers |
| --- | --- |
| MCS 5(QPSK, R=379/1024) | 1x2 Low |
| MCS 13(16QAM, R=490/1024) | 1x4 Low |
| MCS 24(64QAM, R=772/1024) | 1x8 Low |

Proposal 8: Define NR BS MMSE-IRC demodulation requirements for both CP-OFDM and DFT-s-OFDM waveform.

Proposal 9: 5/10/20MHz CBW for 15kHz SCS, and 10/20/40/100MHz CBW for 30KHz SCS can be reused for CP-OFDM for NR BS MMSE-IRC demodulation.

Proposal 10: Use the following parameters for CP-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0}, {0, 1}, {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Proposal 11: 5MHz CBW for 15kHz SCS, and 10MHz CBW for 30KHz SCS can be reused for DFT-s-OFDM for NR BS MMSE-IRC demodulation.

Proposal 12: Use the following parameters for DFT-s-OFDM for NR BS MMSE-IRC demodulation for serving and interferers.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Proposal 13: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation.

- References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements, Moderator (vivo), RAN#103