3GPP TSG-RAN WG4 Meeting # 112	R4-2412763

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Views on 8Rx requirements with interference

Source: 	Huawei, HiSilicon

Agenda item:	8.16.2

Document for:	Discussion

Background

A new work item on NR demodulation performance: Phase 5[1] was approved in RANP#104 meeting. One objective is to define the 8Rx requirements with inter cell and intra cell inter-user interference:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

This contribution provides our views

- Discussions

General views

Receiver assumptions

Two 8Rx receiver types have been defined for Rel-18 8Rx performance requirements: Baseline receiver: 8Rx joint processing and simplified receiver: Separate 4Rx joint processing.  The main purpose of introducing interference conditions is to verify MMSE-IRC receiver, the most complicated step of which that should be verified is Ruu calculation. For simplified receiver, the dimension of Ruu matrix is 4x4, which has been verified in Rel-17 MMSE-IRC 4Rx requirements definition, so there is no necessity to verify it again. Hence we propose to only define requirements with Baseline receiver to verify the implementation of 8x8 Ruu calculation.

Observation 1: The most complicated step which should be mainly verified for MMSE-IRC is Ruu calculation, 4x4 Ruu matrix calculation has been verified in R17 MMSE-IRC 4Rx requirements definition.

Proposal 1: RAN4 shall define 8Rx requirements with interference with joint 8Rx processing. I.e. Baseline receiver.

Criteria of test setup selection

Similar approach as Rel-17 2Rx/4Rx MMSE-IRC requirements definition, the reference receiver should be MMSE-MRC and RAN4 should study the gain of MMSE-IRC over reference receiver under given conditions. One criteria of test setups selection is sufficient performance gain should be guaranteed. On the other hand, the test SINR should be at reasonable range (at least higher than -6dB).

Proposal 2:  Reference receiver should be MMSE-MRC, RAN4 should guarantee the gain of MMSE-IRC over reference receiver for test setup selection. Furthermore, test SINR should be higher than -6dB

Release independence

2Rx/4Rx MMSE-IRC requirements with inter-cell and intra-cell interference are introduced in Rel-17, but it is mandatory from Rel-16 and optional for Rel-15, follow this logic, it is reasonable to define Rel-19 8Rx MMSE-IRC requirements with inter-cell and intra-cell requirements as mandatory from Rel-18.

Proposal 3: 8Rx MMSE-IRC requirements with inter-cell and intra-cell inter user interference should be mandatory from Rel-18

PDSCH requirements with inter-cell interference

Interference model

Two scenarios were introduced in Rel-17:

- Scenario 1: Two interference cells: INR1=7.77dB, INR=2.29dB

- Scenario 2: One interference cell: INR=7.58dB

Our view is that UE is agnostic to deployment and receiver algorithm doesn’t change as interference power level changed. So there is no necessity to define requirements with two different interference levels and for simplicity, interference model introduced in Rel-17 can be reused. We did some initial simulations which are shown in Figure 1-1, it can be observed that under such interference model, MMSE-IRC performance gain over MMSE-MRC is significant.

Proposal 4: RAN4 to only reuse Rel-17 interference model specified for 2 interference cells for 8Rx MMSE-IRC requirements definition.

Rank

Only Rank1 is considered for Rel-17 2Rx/4Rx MMSE-IRC requirements definition. However, 8Rx support up to 8 layers and has more spatial freedom to mitigate interference. Configuring Rank1 will result in very low SINR which may cause radio link failure. Based on our initial simulation results on Rank1 for MCS13, target SINR are lower than -6dB. Hence, RAN4 should choose higher Rank. Based on our simulation results, Rank4 has sufficient performance gain and reasonable SINR range. For rank2, since number of CDM group without data is 1, UE can only perform interference estimation within 1 CDM group, while for rank 4, UE can perform interference estimation within 2 CDM groups. So, propose Rank4 to verify the interference covariance calculation with 2CDM groups

Observation 1: For Rank1 with 8Rx, the SINR is lower than -6dB, which is infeasible for requirements definition.

Observation 2: For Rank4, UE can perform interference covariance calculation with 2CDM groups and the target SINR range is reasonable.

MCS

For Rank4, we simulate MCS13 and MCS19 which are typical values for evaluation, we prefer MCS13, which is more practical to be scheduled in cell edge conditions.

Channel model

We propose TDLA30-10 which is typical channel model for RAN4 requirements definition. Based on the outcome from Rel-18 performance enhancement WI, MMSE-IRC is more sensitive to non-flat channel. Therefore, we propose TDLA30-10 instead of TDLC300-100 to maximum the gain.

Proposal 5: RAN4 to consider following test setup as start point:

- TDLA30-10

- Rank4+Low correlation

- MCS13

Initial simulation results

The initial results for PDSCH with inter-cell interference are captured in Figure 2-1:

Figure 2-1: simulation results for 8Rx MMSE-IRC with inter-cell interference

Table 2-1 summarizes the simulation results

Table 2-1: Summary of simulation results for 8Rx MMSE-IRC with inter-cell interference

| Rank | MCS | Target SNR(dB) for MMSE-MRC | Target SNR(dB) for MMSE-IRC | Gain(dB) | Target SINR(dB) for MMSE-IRC |
| --- | --- | --- | --- | --- | --- |
| Rank1 | 13 | 5.8 | 1.3 | 4.1 | -8.1 |
| Rank4 | 13 | 8.6 | 13.9 | 5.3 | 4.5 |
|  | 19 | 13.7 | 18.8 | 5.1 | 9.4 |

CQI requirements with inter-cell interference

Receiver assumptions

To match two different 8Rx demodulation receiver types, there could be two different 8Rx CSI calculation algorithms. Baseline CSI calculation: UE calculate CSI with 8Rx joint processing. Simplified CSI calculation: UE calculate CSI with separate 4Rx joint processing. Two receiver types get two different SINR/ CQI. Same views as receiver assumptions for demodulation requirements, we propose to choose the former one. The reason is that 4x4 interference covariance matrixes calculation has been verified in Rel-17 4Rx requirements definition so we don’t re-consider it again.

Observation 3: 4x4 interference covariance matrixes calculation has been verified in Rel-17 4Rx requirements MMSE-IRC requirements definition, which means CSI calculation based on 4Rx joint processing is pointless to be considered for requirements definition.

Proposal 6: RAN4 to define CQI requirements with 8Rx joint processing based CQI calculation.

General views

It should be noted that only AWGN is covered for 8Rx CQI requirements in current spec. It seems strange that RAN4 skip fading channel requirements and jump directly to requirements with interference.  For the completeness of spec, RAN4 to discuss how to handle this.

Proposal 7: RAN4 to discuss how to handle the issue that there is no 8Rx CQI requirements with fading channel in the spec.

Rank

Rank1 was selected for 2Rx/4Rx CQI requirements with inter-cell interference and Rank 4 was selected for 8Rx CQI requirements. Same views as PDSCH requirements, we don’t think Rank1 is proper for high-freedom 8Rx UE in the real deployment, so higher rank should be considered. We provide the simulation results for rank2 and rank4 respectively, it can be observed that for Rank 4, the SINR is very high when gain achieves the reasonable range, which seems not conform with the assumptions of cell edge conditions. In contrast, case of rank2 has more SINR points with higher gain, which looks more reasonable than rank4.

Observation 4: Case of rank2 has more SINR points with higher gain, which looks more reasonable than rank4.

Test setup

For test setup of serving cell, test setup of Rel-18 8Rx CQI tests can be reused and for test setup of interference cell, test setup of Rel-17 2Rx/4Rx CQI requirements with inter-cell interference can be used as start point for evaluation.

Proposal 8: RAN4 to use following test setup for 8Rx CQI requirements with interference as start point for evaluation:

Serving cell:

Rank2

2T8R

TDLA30-5, Low

Others: Reuse test setup of Rel-18 8Rx CQI requirements

Interference cell:

INR=10.04dB

Rank1

1T8R

AWGN

Others: Reuse test setup of Rel-17 2Rx/4Rx CQI requirements with inter-cell interference

Test metric

We propose the following test metric, which is same as Rel-17 2Rx/4Rx CQI requirements with inter-cell interference.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.1.2.3-2; b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Proposal 9: RAN4 to reuse following test metric for Rel-17 2Rx/4Rx CQI requirements:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.1.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Simulation results

- We provide initial simulation results shown in Figure 2-2 with following parameters:

Duplex mode: FDD

Bandwidth/SCS: 10MHz

Rank: Rank 2, Rank4

CSI delay:10ms

Channel: Serving cell: TDLA30-5 low, interference cell: AWGN

Antenna configuration: Serving cell: 2T8R for Rank2, 4T8R for Rank4, interference cell:1T8R

Cases to be simulated:

- Case1: Demodulation algorithm: MMSE-IRC, CSI calculation algorithm: MMSE-IRC, inter-cell interference with INR=10.04dB is present

- Case2: Demodulation algorithm: MMSE-IRC, CSI calculation algorithm: MMSE-MRC, inter-cell interference with INR=10.04dB is present

- Case3: Demodulation algorithm: MMSE-MRC, CSI calculation algorithm: MMSE-MRC, white noise with same power as sum of interference and white noise assumed in case1/2

Figure 2-2: Simulation results for CQI requirements with inter-cell interference.

PDSCH requirements with MU-MIMO

Rank

Rank1+1 and Rank 2+2 were considered in Rel-17/18 MU-MIMO requirements with 4Rx, however, 8Rx UE, supporting up to 8layers, has more spatial freedom and higher capacity to mitigate the interference compared to 4Rx, so rank 2+2 is not enough to verify the capability of 8Rx. In Rel-18, Rank8 has been selected for requirements definition, hence we propose Rank4+4 and Rank 2+2+2 for Rel-19 8Rx requirements definition. Based on our simulation results, rank 4+4 and Rank2+2+2 have significant performance difference between MMSE-IRC receiver and MMSE-MRC receiver and 70% of maxTP is unachievable for MMSE-MRC receiver.

Rank allocation

For Rank2+2+2, we propose serving UE is located in port 0/1, co-scheduled UE1 is located in Port 2/3 and co-scheduled UE2 is located in Port 6/7, which is shown in Figure 2-3:

Figure 2-3: Rank allocation for Rank 2+2+2 test

For Rank4+4, we propose that serving UE is located in port 0/1/4/5, co-scheduled UE1 is located in Port 2/3/6/7, which is shown in Figure 2-4:

Figure 2-4: Rank allocation for Rank 4+4 test

Precoder

To be more realistic, orthogonal PMI instead of random PMI should be selected, we propose the following:” Any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE.

MCS

For Rank 4+4, based on our initial simulation results, MCS13 shows sufficient performance, we propose MCS13.

For Rank 2+2+2, based on our initial simulation results, both MCS13 and MCS19 show sufficient performance gain, we propose MCS19 to cover 64QAM since we have proposed 16QAM for Rank 4+4.

Others

For other test setup, we propose to reuse Rel-17 2Rx/4Rx MU-MIMO tests.

Proposal 10:  RAN4 to consider following test setup for 8Rx requirements with intra-cell inter user interference:

Rank 2+2+2

Serving UE: Antenna Port 1000, 1001

Co-scheduled UE1: Antenna Port 1002,1003,

Co-scheduled UE2: Antenna Port 1006, 1007

Rank 4+4

Serving UE: Antenna Port 1000, 1001,1004,1005

Co-scheduled UE: Antenna Port 1002,1003,1006,1007

Channel: TDLA30-10, Low

MCS:

Rank 2+2+2：MCS19

Rank 4+4：MCS13.

Precoder:

Single Panel Type 1 for all UEs, any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE

Initial results

The initial results are shown in Figure 2-3, with simulation assumptions shown in Table 2-2:

Table 2-2: Simulation assumptions for 8Rx with MU-MIMO

| Parameter |  | Unit | Target UE | Co-scheduled UE(s) |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Bandwidth/SCS |  |  | 10MHz/15kHz |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | - For Rank4+4：1000/1001/1004/1005 - For Rank 2+2+2:1000/1001 | - For Rank 4+4: Co-scheduled UE1: 1002,1003 - For Rank 2+2+2: Co-scheduled UE2: 1006,1007 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 2 | 2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. Single Panel Type 1 for all UEs, any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE |
| Receiver |  |  | MMSE-IRC, MMSE-MRC | N/A |
| Number of HARQ Processes |  |  | 4 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

Figure 2-5: Simulation results for 8Rx MMSE-IRC with intra-cell inter user interference

- Conclusion

- In this paper, we provide our views on 8Rx requirements with interference. The proposals are:

For PDSCH requirements with inter-cell interference:

Observation 1: The most complicated step which should be mainly verified for MMSE-IRC is Ruu calculation, 4x4 Ruu matrix calculation has been verified in R17 MMSE-IRC 4Rx requirements definition.

Proposal 1: RAN4 shall define 8Rx requirements with interference with joint 8Rx processing. I.e. Baseline receiver.

Proposal 2:  Reference receiver should be MMSE-MRC, RAN4 should guarantee the gain of MMSE-IRC over reference receiver for test setup selection. Furthermore, test SINR should be higher than -6dB

Proposal 3: 8Rx MMSE-IRC requirements with inter-cell and intra-cell inter user interference should be mandatory from Rel-18

Proposal 4: RAN4 to only reuse Rel-17 interference model specified for 2 interference cells for 8Rx MMSE-IRC requirements definition.

Observation 1: For Rank1 with 8Rx, the SINR is lower than -6dB, which is infeasible for requirements definition.

Observation 2: For Rank4, UE can perform interference covariance calculation with 2CDM groups and the target SINR range is reasonable.

Proposal 5: RAN4 to consider following test setup as start point:

- TDLA30-10

- Rank4+Low correlation

- MCS13

For CQI requirements with inter-cell interference:

Observation 3: 4x4 interference covariance matrixes calculation has been verified in Rel-17 4Rx requirements MMSE-IRC requirements definition, which means CSI calculation based on 4Rx joint processing is pointless to be considered for requirements definition.

Proposal 6: RAN4 to define CQI requirements with 8Rx joint processing based CQI calculation.

Proposal 7: RAN4 to discuss how to handle the issue that there is no 8Rx CQI requirements with fading channel in the spec.

Observation 4: Case of rank2 has more SINR points with higher gain, which looks more reasonable than rank4.

Proposal 8: RAN4 to use following test setup for 8Rx CQI requirements with interference as start point for evaluation:

Serving cell:

Rank2

2T8R

TDLA30-5, Low

Others: Reuse test setup of Rel-18 8Rx CQI requirements

Interference cell:

INR=10.04dB

Rank1

1T8R

AWGN

Others: Reuse test setup of Rel-17 2Rx/4Rx CQI requirements with inter-cell interference

Proposal 9: RAN4 to reuse following test metric for Rel-17 2Rx/4Rx CQI requirements:

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.1.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

For PDSCH requirements with intra cell inter user interference:

Proposal 10:  RAN4 to consider following test setup for 8Rx requirements with intra-cell inter user interference:

Rank 2+2+2

Serving UE: Antenna Port 1000, 1001

Co-scheduled UE1: Antenna Port 1002,1003,

Co-scheduled UE2: Antenna Port 1006, 1007

Rank 4+4

Serving UE: Antenna Port 1000, 1001,1004,1005

Co-scheduled UE: Antenna Port 1002,1003,1006,1007

Channel: TDLA30-10, Low

MCS:

Rank 2+2+2：MCS19

Rank 4+4：MCS13.

Precoder:

Single Panel Type 1 for all UEs, any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE

- Reference

[1]    Revised WID on NR demodulation performance: Phase 5. China Telecom, NTT DOCOMO