3GPP TSG-RAN WG4 Meeting #112bis										R4-2415022

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.20.2.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

WF: - FFS on the followings: - Option 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed. - Option 2: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. If we do not define co-location ACS requirement. Add the note in the spec that the ACS requirement is not applied to the co-location scenario.

Proposal 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

2.2 In-band blocking

Last meeting we have approved the WF below:

WF: - FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

Since during study item, 100% grid-shift is used as the simulation assumption, in order to be consistent with the conclusions of the study item, the simulation assumptions should also be consistent. So we suggest to consider 100% grid-shift.

Proposal 2: 100% grid-shift should be considered

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

Proposal 2: 100% grid-shift should be considered

# 4. Reference

[1] TR 38.858,

[2] R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF