3GPP TSG-RAN WG4 Meeting # 112bis                                                          R4-2415107

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on modification of existing Rx requirements for SBFD operation

Source: 	CATT

Agenda item:	6.20.2.3

Document for:	Discussion

- Background

WF [1] of RAN4#112 concluded that further studies are needed for some Rx RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Rx RF requirement

  - OTA sensitivity

About OTA sensitivity degradation value caused by SI, according to the SI feasibility analysis results of each company in [1], 1.0dB is a more appropriate value.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

  - Dynamic range requirement

In [3], dynamic range simulation considered both uplink signal and gNB-to-gNB CLI signal.

When the grid shift is 100% and the CDF is 100%, the IoTs of scenario1, scenario 3 and scenario 5 are close to 20dB, and the IoT of scenario 6 is much higher than existing requirement, IoT of scenario 9 is 7.6dB, and which is close to existing requirement.

When the grid shift is 10% and the CDF is 100%, IoTs of both FR1 indoor and FR2-1 indoor scenarios can reuse the existing requirements. IoTs of scenario1 and 5 are much higher than 20dB.

Observation 1: Dynamic range of FR1 urban macro and micro scenarios are much higher than existing requirements.

Observation 2: Dynamic range of FR1 indoor scenario is close to existing requirement.

Observation 3: Dynamic range of FR2-1 urban macro scenario is much higher than existing requirement.

Observation 4: Dynamic range of FR2-1 indoor scenario is close to existing requirement.

Proposal 2: Dynamic range of FR1 indoor scenario can reuse the existing requirement.

Proposal 3: Dynamic range of FR2-1 indoor scenario can reuse the existing requirement.

  - OTA ACS requirement

For ACS requirement, there’re two views if the non-SBFD or SBFD REFSENS should be the baseline REFSENS level. The decision may depend on if the SBFD sub-band DL is switched on for the ACS test. This discussion is related to all of the requirements which use DESENS LEVEL as the requirement. Our understanding is that non-SBFD BS approach is reused that Tx is switched on for the Rx requirements. So the OTA sensitivity degradation should be taken into account for the baseline REFSENS.

Proposal 4: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

  - In-band blocking

According to [3], the simulation results of in-band blocking for FR1 and FR2-1 are shown in table 1 and table 2, and the interference are from TDD DL to SBFD UL.

Table 1: SBFD conducted in-band blocking simulation results (FR1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | Conducted in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 1 | FR1 | Urban Macro -> Urban Macro | -25.4 | 0.3 |
| 3 | FR1 | Indoor -> Indoor | -34.4 | —— |
| 5 | FR1 | Micro -> Micro | -32.1 | -17.2 |

Table 2: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | IBB over wanted signal (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 6 | FR2-1 | Urban Macro -> Urban Macro | 24.7 | 36.6 |
| 9 | FR2-1 | Indoor -> Indoor | 19.9 | —— |

For FR1 urban macro scenario, the in-band blocking signals power levels are much higher than the existing IBB requirement.

For FR1 indoor scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

For FR1 micro scenario, the in-band blocking signals power level of 100% grid shift is close to the existing IBB requirement, but the power level of 10% grid shift is much higher than the existing IBB requirement.

For FR2-1 urban macro scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

For FR2-1 indoor scenario, the in-band blocking signals power level is lower than the existing IBB requirement, and can reuse the existing requirements.

Observation 5: The simulation result of in-band blocking level for FR1 macro scenario is much higher than existing represent.

Observation 6: The simulation result of in-band blocking level for FR1 indoor scenario is close to the existing requirement.

Observation 7: The simulation result of in-band blocking level for FR1 micro scenario is higher than the existing requirement.

Observation 8: The simulation results of in-band blocking levels for FR2-1 urban macro and indoor scenarios are lower than the existing requirements.

Proposal 5: For FR1 indoor scenario, the existing IBB requirement can be reused.

Proposal 6: For FR2-1 macro and indoor scenarios, the existing IBB requirement can be reused.

  - Rx intermodulation

In WF [1], the necessity of new RX intermodulation requirement with one interfering signal need further study.

WF:

- FFS on the followings:

  - Option 1: Whether IMD requirement for single interfering signal scenario is needed. Investigate whether such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. For some SBFD DL and UL spectrum configurations, the single interference signal scenario is implicitly captured by the SBFD RX blocking requirement, even if the frequency range of the single interference signal is from out-of-band to in-band rather than only in-band.

Observation 9: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 7： The single interference signal scenario isn’t needed.

WF:

- FFS on the following:

  - Option 1: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

For the power level of the receiver intermodulation interference signal, whether BS2BS CLI needs to be considered depends on the power level values of the interference, and the power level can be simulated. The in-band blocking of the SBFD slot has been simulated by RAN4, and the interference signals are from BS of frequency adjacent system, these interference signals can be considered by legacy Rx intermodulation, and new RX intermodulation requirement with two interfering signals is not necessary.

Observation 10: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 8: New RX intermodulation requirement with two interfering signals is not necessary.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

Observation 1: Dynamic range of FR1 urban macro and micro scenarios are much higher than existing requirements.

Observation 2: Dynamic range of FR1 indoor scenario is close to existing requirement.

Observation 3: Dynamic range of FR2-1 urban macro scenario is much higher than existing requirement.

Observation 4: Dynamic range of FR2-1 indoor scenario is close to existing requirement.

Proposal 2: Dynamic range of FR1 indoor scenario can reuse the existing requirement.

Proposal 3: Dynamic range of FR2-1 indoor scenario can reuse the existing requirement.

Proposal 4: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement

Observation 5: The simulation result of in-band blocking level for FR1 macro scenario is much higher than existing represent.

Observation 6: The simulation result of in-band blocking level for FR1 indoor scenario is close to the existing requirement.

Observation 7: The simulation result of in-band blocking level for FR1 micro scenario is higher than the existing requirement.

Observation 8: The simulation results of in-band blocking levels for FR2-1 urban macro and indoor scenarios are lower than the existing requirements.

Proposal 5: For FR1 indoor scenario, the existing IBB requirement can be reused.

Proposal 6: For FR2-1 macro and indoor scenarios, the existing IBB requirement can be reused.

Observation 9: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Proposal 7： The single interference signal scenario isn’t needed.

Observation 10: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 8: New RX intermodulation requirement with two interfering signals is not necessary.

- Reference

[1] TR 38.858

[2] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, HiSilicon, RAN4#112

[3] R4-2415106, “SBFD in-band blocking and dynamic range simulation results”, CATT, RAN4#112bis