3GPP TSG-RAN WG4 Meeting #112bis				                                          R4-2415179

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	6.15.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

In last meeting, most of the applicable scenarios of Multi-Rx based L3 measurement, e.g., power class, mobility status etc., have been agreed, and it was also agreed the conditions to apply L3 measurement delay reduction by optimizing Rx BSF is that Multi-Rx simultaneous reception of UE is in active mode. But the condition of Multi-Rx simultaneous reception of UE in active mode is still open, and the following agreement was captured in the WF [1].

Issue 1-2-1: FFS: Multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for Multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that Multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 Multi-Rx simultaneous reception.

Although it was agreed that the condition of active mode in Rel-19 is not assumed to be the same as Rel-18, we think the Rel-18 condition can be one of the applicable conditions. In Rel-18 Multi-Rx, the UE is in Multi-Rx operation if UE is configured with group-based beam reporting (GBBR) report. In this case, since the UE is already in Multi-Rx operation, we think the operation can be used for L3 measurement, i.e., UE can perform Multi-Rx based L3 measurement.

Agreements: - For fast beam sweeping, the UE is in Multi-Rx operation if following condition is met: - UE is configured with group-based beam reporting (GBBR) report.

But we can also understand that Rel-19 Multi-Rx is for L3 measurement and do not need to totally rely on the condition for L1 measurement. So we think a new indication for Multi-Rx based L3 measurement can be introduced. Therefore we suggest the condition of Multi-Rx in active mode can be defined as UE is configured with GBBR report or is configured with Rel-19 new indication.

Proposal 1: Introduce a new network signaling to indicate UE performing Multi-Rx based L3 measurement.

Proposal 2: The UE is in Multi-Rx operation for L3 measurement if following condition is met:

- UE is configured with group-based beam reporting (GBBR) report, or

- UE is configured with Rel-19 NW signaling for Multi-Rx based L3 measurement.

For searcher assumption, the following way forward was captured in the approved WF [1]:

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT)

According to the discussion in last meeting, the common understanding is that 2 searchers in total are assumed for a UE which is same as legacy. And based on the existing CSSF definition as below, for the measurement outside gap, one searcher is used for PCC and the other searcher is used for SCC, inter-frequency without gap and inter-RAT measurement without gap. With this assumption, we understand even if no SCC is configured, the searchers still need to be shared by intra-frequency, inter-frequency and inter-RAT frequency measurement without gap.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD]. FR2 only intra band CA; N/A; N/A; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only inter band CA; N/A; N/A; 1+NPCC_CSIRS; 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM).

The issue here is only one target carrier is assumed in this BSF optimization discussion since we do not consider CA case at this stage, so one searcher may be spare in some cases (e.g., for intra-frequency measurement, only one serving carrier is measured) if following legacy assumption. In such case, it is possible to consider using the other searcher in this carrier.

But based on the previous agreements, for Multi-Rx based L3 measurement, the target carrier can be same or different with serving carrier. If the two carriers are different, i.e., inter-frequency measurement, it cannot be assumed that 2 searchers are used for single carrier, because the two searchers need to be allocated to different carriers, i.e., one searcher for serving carrier and the other one for inter-frequency measurement.

To accommodate the possible CA/DC discussion and align the assumption for intra-frequency and inter-frequency measurement, we think one searcher for single carrier can be assumed.

Proposal 3: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

With the assumption above, according to the discussion in last meeting, another issue is the additional SSB processing time. If the UE receives reference signals from different cells within the same SMTC using Multi-Rx, since the multiple SSBs within the SMTC are from different directions and have different beam index, UE may not be able to process the SSBs simultaneously. Thus additional processing time is needed. Usually we think the interval between two SMTCs can accommodate this processing time, and the only exception case is that the measurement starts from the end of one SMTC occasion. To consider this case and define the genera requirements, we are fine to add this additional processing time in the delay requirements.

Proposal 4: With the assumption in proposal 3, additional processing time due to multiple SSBs within one SMTC can be added in the delay requirements.

For power consumption, based on the discussion in last meeting, there are three directions to resolve the issue, i.e., defining additional trigger conditions, using Rel-18 mechanism (Multi-Rx condition and UAI reporting) or up to Rel-19 network indication.

[Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion).

For additional trigger conditions, the proposal is to only activate Multi-Rx based L3 measurement under certain conditions, e.g., low mobility or at cell edge. But we think there is no need to limit the applicable scenarios. As long as UE can support Multi-Rx operation, the enhanced solution can be used for general L3 measurements.

On the other hand, for this power consumption issue, we understand it also exists in Multi-Rx operation for L1 measurement in Rel-18 and the following indication on UE preference was introduced as shown below, with which we think the issue can be resolved. And we think this mechanism can also be used for L3 measurement. Meanwhile, as we discussed in proposal 1 and 2, both Rel-18 GBBR report condition and Rel-19 introduced signaling can be used to indicate active mode of Multi-Rx operation. So we understand direction (2) and (3) can be combined together.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

Proposal 5: The power consumption issue can be resolved by UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling.

For DRX issue, we understand the requirements applicability of enhanced solution is agnostic to the DRX status and no need to define the priority. If the concern is necessity or power saving issue, it can be controlled by network configuration and UE indication and the requirement is better to be general.

Issue 1-2-9: FFS: DRX is configured or not [Way forward]: FFS: - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - Option 2 (Ericsson): FBS may cover DRX cases, no need to deprioritize DRX case.

Proposal 6: No need to deprioritize DRX case.

Regarding the applied requirements to use measurement delay reduction by optimizing Rx BSF, it was agreed to consider scenario 1/2/3/4/5/7 as first priority.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

For the solutions of optimizing Rx BSF, we think the conclusions in Rel-18 Multi-Rx can be used as baseline, i.e., for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. And according to the agreements above, the reduced Rx BSF applies to intra/ inter-frequency SSB based measurement, handover, RRC Re-establishment/RRC Connection Release with Redirection.

Although we do not explicitly agree to discuss the requirements for CSI-RS measurements, after the requirements for SSB measurement is decided, the enhanced Rx BSF can also apply to the associated SSB in CSI-RS based measurement since it refers to formula of SSB based measurement, but no spec updates are needed. Similarly for handover, since the requirements for CHO and DAPS HO refer to the formula of SSB based measurement and handover respectively, the enhanced Rx BSF can also apply but no specification impact is expected.

Proposal 7: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 8: The reduced Rx BSF in proposal 7 can be applied to:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 9: The reduced Rx BSF in proposal 7 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

3 Summary

In this paper, we have some further discussions on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: Introduce a new network signaling to indicate UE performing Multi-Rx based L3 measurement.

Proposal 2: The UE is in Multi-Rx operation for L3 measurement if following condition is met:

- UE is configured with group-based beam reporting (GBBR) report, or

- UE is configured with Rel-19 NW signaling for Multi-Rx based L3 measurement.

Proposal 3: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

Proposal 4: With the assumption in proposal 3, additional processing time due to multiple SSBs within one SMTC can be added in the delay requirements.

Proposal 5: The power consumption issue can be resolved by UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling.

Proposal 6: No need to deprioritize DRX case.

Proposal 7: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 8: The reduced Rx BSF in proposal 7 can be applied to:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 9: The reduced Rx BSF in proposal 7 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

4 References

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN4#112

- R4-2411813, Topic summary for [112][218] NR_RRM_Ph5_Part1, Apple, RAN4#112