# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415541

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.23.4.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#112 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. Furthermore, some scopes of this WI has been confirmed in [3] from RAN #105 meeting.

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

In case#3, If LR is ON and MR is ON with relaxed measurement, both the MR and LR will perform the serving cell measurement, it’s still unclear that which measurement(s) will be used to check the certain criteria, e.g., LP-WUS monitoring entry/exit criteria or MR ON/OFF criteria. In last RAN2#127 meeting, RAN2 agreed that,

### RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In our view, it would be simpler to not combine the samples from different radios. Even in some cases, MR and LR may perform measurement on the SSB, but the measurement performance of MR and LR is different (e.g., due to different noise figure) and we doubt if the measured samples from MR and LR can be combined. Theoretically, it may be possible to combine the measurement samples from MR and LR by using certain proportional factor to reflect the reliability or performance or accuracy difference, however, it would be too complicated for standardization.

Instead, we prefer to use the measurement result from either MR and LR to compare with the corresponding threshold, and as long as one kind of measurement results (either from MR or LR) can meet their own threshold, UE can take the corresponding action, e.g., entering MR fully offloading (from case #3 to case #1), or change from case #3 to case #5. The reason to say their own threshold is because RAN2 didn’t preclude different thresholds for MR and LR measurement.

Proposal 1: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR. As long as one kind of measurement results (either from MR or LR) can meet their own threshold, UE can take the corresponding action, e.g., entering MR fully offloading (from case #3 to case #1), or change from case #3 to case #5.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

In last meeting RAN4 has no conclusion on the revision of the RAN4#110bis agreement, and at least the OOK based LP-WUR serving cell measurement shall be prioritized. RAN1 still didn’t conclude on the OFDM based LP-WUR to measure LP-SS, and we only found the following relevant working assumption in RAN1 agreements in RAN1#117.

Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

With such situation in RAN1, we are fine with the recommendation from the moderator in last meeting that OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state can be discussed after more RAN1 made concrete conclusion to support overlaid OFDM sequence for LP-SS.

Proposal 2: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- FFS on measurement requirements for OFDM based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state after RAN1 has concrete conclusion.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

One thing to clarify that, LR measurement will be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement, and therefore it means LR measurement result shall be comparable to MR measurement or shall be equivalent to MR measurement with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

In WID, another important enhancement is: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. Even though conditions for offloading RRM serving cell measurement from MR to LR and conditions for relaxation of MR serving cell and neighbor cell measurements is mainly decided/designed by RAN2, RAN4 can investigate the mobility performance corresponding to such relaxation, and the mismatch/bias between LP-WUR based measurement result and MR based measurement result. For instance, if LP-SS is used for the measurement to decide when the MR shall be waken up or not, such LP-SS based measurement result at least shall be comparable to the MR based measurement results or some bias offset shall be defined; otherwise, LP-SS triggers MR to wake up for mobility but MR based measurement results may tell that the mobility is not needed after MR wakes up.

In previous RAN2 meeting, there was also some discussion that,

Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN4 shall specify MR relaxation requirements at idle/inactive mode for UE supporting LP-WUR, and there are still some open issues to discuss for the principle of such relaxation.

Proposal 5: the criteria design for RRM relaxation of UE MR for both serving and neighbor cell measurements can be left to RAN2 to decide

Proposal 6: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Issue 1-1-4-1: Entry/exit criteria evaluation when results from MR and/or LP-WUR are available for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Issue 1-1-4-2: Reference signals and measurement types to be used for Entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

We are open to discuss the requirement for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, but depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria. Our understanding is:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

Regarding the time period for UE to meet the criteria, RAN1 had a working assumption as following:

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

Similar as the criteria checking for power saving feature, TS38.304 specified the non-at-cell-edge condition as following:

5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

In our view, for this entry/exit criteria checking for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, we didn’t see strong motivation to have a new evaluation requirement, but we can just use the MR measurement period or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation., i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

Proposal 7: use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

Proposal 8: Depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria; and followings are considered:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the MR/LR serving cell measurement delay requirements for entry/exit criteria evaluation.

Issue 1-1-4-2: Reference signals and measurement types to be used for Entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation - Proposals - P1: Both LP-SS and SSS based measurement can be used for LP-WUR to evaluate the entry/exit condition. FFS whether to define different conditions for different signals. (CATT) - P2: Both RSRP and RSRQ measurements are used for LP-WUR and MR to evaluate the entry/exit condition. (CATT)

In last RAN2 meeting, it was agreed that,

### Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Regarding issue 1-1-4-2, it’s clear that RAN2 had some conclusions already and still working on the threshold design. We recommend to just follow the RAN2 working assumption and agreements, and no need to discuss it in RAN4.

Proposal 9: Follow following RAN2 agreements in RAN2 #127 meeting for issue 1-1-4-2.

- Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

- Working assumption: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

- The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

- The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement

Like in power saving feature, the criteria itself has been defined be RAN2 in TS38.304, and therefore here we also think that RAN2 is the main group for criteria (entry/exit conditions) design. However, for RRM relaxation requirement (e.g., scaling factor), RAN4 can work on the RRM measurement relaxations (e.g., scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 10: RAN2 shall be the main group for criteria (entry/exit conditions) design

Proposal 11: RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

Similar as the discussion in issue 1-1-5, we also think RAN2 shall be the main group for criteria design.

Issue 1-1-8: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

- Proposals - P1: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring (Xiaomi LG) - P2: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and case 1, or case 2 or 3 if case 2 or 3 are supported (vivo). - P3: In order to guarantee the power saving gain, the serving cell quality threshold in entry/exit condition for LP-WUS monitoring and the serving cell quality threshold entry/exit condition for LP-WUR measurement should be considered jointly. (CMCC) - P4: RAN4 needs to consider overall entry/exit condition for LP-WUR measurement, MR measurement relaxation, and LP-WUS monitoring jointly based on RAN2 agreements of entry/exit condition for LP-WUS monitoring. And if there are other conditions that should be considered, RAN4 needs to provide input to RAN2 (LG) - P5: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with RAN1 and RAN2. (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - P6: assumption of thresholds for RAN4 discussion can be (Apple): - the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering. - the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring. Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered not early than MR RRM relaxation or MR serving cell offloading (case 1)

We think the case #1 is the case when UE has very good quality of serving cell, and UE can fully offload from MR to LR. But if the serving cell quality is not that good as case #1, UE may consider to use MR relaxation for serving and neighbor cell measurement. Case #2 is a little bit weird case to consider, since when the LR is ON and neighbor cell measurement is OFF on MR, it seems not necessary for UE to keep MR awake for a relaxed serving cell measurement. In our view the threshold to trigger corresponding offloading and relaxation can be as following figure,

Figure 1. threshold assumption for different cases

If the serving cell measurement shows above the threshold of neighbor cell measurement (or a new threshold of MR offloading), UE can turn off the MR, and only rely on LR to perform serving cell measurement, and no neighbor cell measurement is needed (including higher or equal/lower priority layers measurement). If the neighbor cell measurement is triggered (e.g., below the threshold of intra-freq or inter-freq measurement triggering condition) but serving cell quality is still above threshold of LP-WUS monitoring threshold, the MR can be ON with relaxation for serving and neighbor cell measurement while the LR is also ON for serving cell measurement, i.e., case #3. Then if UE is below LP-WUS monitoring threshold but above the not-at-cell-edge threshold for RRM relaxation, then UE will be in a legacy power saving mode, which means, LR is OFF and MR is ON with neighbor cell measurement relaxation (no serving cell relaxation). Even further, if UE is below not-at-cell-edge threshold for RRM relaxation, it means UE is in legacy IDLE/Inactive mode status without any RRM relaxation.

In order to support the above logic, we have some assumptions for such threshold design, even though the final threshold decision shall be made by RAN2. Firstly, we assume the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.  Secondly, the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring, as LP-WUS monitoring will require the T/F tracking on LR, e.g., via LP-SS.

Proposal 12: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Based on the illustration in figure 1, we believe the case#2 and case#4 can be removed at least at this stage, but of course we can revisit the cases if RAN1/2 has some conclusions conflicting with RAN4 assumptions. And the following case table can be assumed in RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 13: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Issue 1-1-9: LP-WUR status

- Proposals - P1: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring (Apple) - Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement - Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON without RRM measurement (Apple) - Alt 3: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is OFF without RRM measurement. (Apple) - P2: For LP-WUR status before entering offloading or after exiting offloading, postpone the discussion until more RAN1/2 conclusions (vivo)

RAN2 had a working assumption (can revisit if R1/R4 reached different conclusions) in last RAN2#127 meeting: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. However, it’s still unclear if LR-based threshold is required to check by UE, i.e., whether or not UE is required to use LR measurement to check the LR-based threshold for LP-WUS monitoring entry.

In WID, it was stated that: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. As we discussed in previous proposal 4, UE may or may not turn off the MR for serving/neighbor cell measurement, and therefore, two cases can be considered, i.e., case#1 and case #3

- Case #1: MR is OFF or deep-sleep without reception/transmission and LP-WUR is ON for serving cell measurement.

- Case #3: MR is still ON with RRM measurement relaxation on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement.

In addition, before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any). But whether or not LP-WUR is ON for RRM measurement is up to UE implementation.

We prefer Alt 2 understanding, the reason is: this WI is not targeting to enhancement the legacy measurement performance by using both MR and LR, and therefore it shall not change the legacy UE measurement behavior before entering LP-WUS monitoring or after exiting LP-WUS monitoring status.

Proposal 14: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

- Alt 2 (preferred): MR is ON with RRM measurement on serving cell and neighbour cell (if any). But whether or not LP-WUR is ON for RRM measurement is up to UE implementation.

Issue 1-1-11: Considerations on higher priority frequency layer

- Proposals - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

In case #1, when UE fully offload from MR to LR, we assume measurement results from LP-WUR will decide whether the MR shall be waken up or not, however, the MR will be turned off or in ultra-deep sleep mode. The FFS part is mainly about whether the higher priority measurement shall still be kept on MR or not. Current higher priority measurement in RAN4 has two main delays: 1min and 1 hour, as followings.

In order to save the power of UE, if MR still needs to wake up every 1min, we didn’t see much gain for this feature, but we are open to discuss if higher priority measurement on MR can be kept and performed as slow as 1 hour per layer. For simplicity of this feature at UE, we prefer to not consider higher priority layer measurement on MR when the MR is fully offloaded. Thus, the MR neighbor cell measurement shall not be considered for case #1.

But we are still open to discuss the condition for higher priority layer measurement in case#1 if majority companies would like to support it.

Proposal 15: higher priority layer neighbor cell measurement shall also be OFF for case #1.

Issue 1-1-12: Impact on specification

We think this issue can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 16: this issue 1-1-12 can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

# Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

Regarding the accuracy requirement, if we follow the legacy accuracy requirement principle, the dedicated accuracy requirements in TS38.133 section 10 is mainly for the measurement results which is reported to network, and the only measurement accuracy requirement for IDLE/Inactive mode is the EMR feature related measurement. For LR based RRM measurement, since UE will not report anything back to network in IDLE/Inactive mode, our preference is still not to define a dedicated accuracy requirement in TS38.133 section 10, but instead we can reflect such accuracy performance in the core requirement sections as a margin, like in the following example:

Proposal 17: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: Possible interruption related requirements for LP-WUR

- Proposals - P1: Interruption requirements could be defined for the time between LP-WUS received paging indication to MR ready for Paging monitoring (Samsung xiaomi Docomo CMCC) - In addition, between LP-WUR measurement result fulfills exit criteria and MR to start measurement (CMCC) - In addition, between MR measurement result fulfills entry criteria and LP-WUR to start measurement. (CMCC ZTE) - P2: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

In last RAN1 #118 meeting, the following agreement for wake-up delay has been achieved,

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

The definition of wake-up delay is: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring, that is same as what RAN4 discussed in last meeting, i.e., interruption requirement. We think such interruption requirement has already been reflected by UE capability indication as agreed in RAN1, thus no dedicated interruption requirement needs to be defined for this case. RAN4 only needs to define the wake-up delay requirement as defined in RAN1, if such requirement is necessary.

Proposal 18: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Issue 1-2-3: Possible cell selection evaluation requirements for fully offloading case based on LP-WUR

As illustrated in our figure 1, we think the typical case is the S criterion threshold is lower than the MR offloading threshold and neighbor cell measurement triggering threshold, and therefore we don’t need to discuss the LR based S criterion evaluation in case#1.

Proposal 19: No need to discuss the LR based S criterion evaluation in case#1.

Issue 1-2-4: LP-WUR operating carrier frequency

Regarding the whether MR and LR are operation on same carrier or not, RAN1 is also discussing such scenarios. Our preference is to use same carrier scenario as baseline for RAN4 discussion and FFS on the other scenarios if RAN1 has agreement.

RAN4 received one LS (R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode) from RAN1 as following:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

In our view, RAN4 requirement for LP-WUR based RRM can be designed to apply on the bands on which UE can support LP-WUS reception. For other bands where UE cannot support LP-WUS reception, UE may still use MR to perform RRM and the R16/17 power saving feature can be considered instead. Even though we are open to discuss the inter-band LP-WUS reception (MR on one band and LP-WUS/LP-SS on another band), we think more RAN1 conclusions are needed before RAN4 requirement discussion, e.g., which band pair can be considered to support inter-band LP-WUS/LP-SS reception by considering different channel conditions. At the current stage the same carrier scenario shall be used as a baseline case for RAN4 requirement design.

Proposal 20: to define RRM requirement, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has more conclusions.

Issue 1-2-6: Number of Rx for LP-WUR requirements

In last RAN1 meeting, the agreement is as following:

Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Since the LP-SS RSRP/RSRQ measurement metrics was agreed in RAN1, RAN4 only needs to wait until the entire LP-SS design has been concluded. Normally measurement metrics is defined in TS38.215(RAN1 spec), we think RAN1 can lead to decide the measurement metrics (RSRP/RSRQ/SINR/RSSI)  standardization before RAN4 RRM requirement design during this LP-WUR WI. In our view, one thing RAN4 can discuss is the Rx number for LP-SS measurement metrics. In legacy SSB-based RSRP measurement, it specified that,

For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches.

Our understanding is that the LP-WUR is a quite low power-consumption and low-cost component on the UE, and it shall be assumed as single Rx as a more typical case for requirement design. Thus, to help completing the measurement metrics, RAN4 can discuss whether 1Rx is assumed for LR based RRM first.

Proposal 21: Single Rx is assumed for LR based RRM measurement.

Issue 1-2-7: Other considerations

In previous meeting, RAN1 also discussed the LP-WUS in eDRX, but no conclusion has been made. In our view, besides LP-WUS, we also need to investigate the UE RRM measurement for LP-SS in eDRX, e.g., whether LP-SS measurement shall follow the eDRX/DRX cycle or not. If the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, the RRM requirement shall only be defined based on the LP-SS periodicity. In our view, DRX is only applied for MR but not for LP-WUR, and therefore, LP-SS measurement in IDLE/Inactive mode shall only follow LP-SS periodicity.

Proposal 22: LP-SS measurement delay requirement in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

# MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

In the WID, the MR relaxation scope is as following:

Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

In the power saving feature, the RRM relaxation for IDLE and connected mode is only for neighbor cell measurement, and no relaxation is allowed for serving cell measurement. The neighbor cell measurement relaxation is decided by serving cell measurement results. RAN4 needs to carefully check if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation requirement (e.g., scaling factor). In our view, if the neighbor cell measurement relaxation is based on the serving cell measurement, it implies that neighbor cell measurement shall always have the more relax measurement than serving cell measurement. The rationale is also obvious: only if UE can get reliable serving cell measurement results, then it can use such results to decide whether neighbor cell measurement can be relaxed or not.

Proposal 23: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

As proposed by companies, we think 16 could be used as starting point for relaxation, and based on proposal 23, we would like to propose 16 for serving cell measurement relaxation and neighbor cell measurement relaxation.

Proposal 24: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

# LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

In the WID, the scope for connected mode is to discuss the procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring, however, like in R16 WUS design, it would not have any RRM impact by using LP-WUS in connected mode. Furthermore, WID mentioned that,

In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

All the RRM/RLM/BFD/CSI measurement would be as same as legacy case, i.e., use MR to perform these functionalities. Thus, no RRM scope shall be needed for connected mode in this WI.

Proposal 25: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

# Others

Issue 1-6-1: FR2 LR based RRM

In the past RAN#105 plenary meeting, the FR2 LP-WUS scope has been discussed and agreed in [RP‑242361 Way forward on LP-WUS for FR2] as following:

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Since the FR2 is confirmed as a scope for LP-WUS WI, RAN4 needs to discuss (1) whether we need to design requirement for FR2 LP-WUR based RRM, and (2) when to start the discussion for such requirement design. Our understanding is FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement, since FR2 case would be more complicated than FR1 by considering the Rx beam sweeping.

In RAN4 Idle/inactive mode requirement for SSB, the Rx beam sweeping has been applied for serving cell measurement and evaluation as following examples:

In LP-SS based serving cell measurement, UE will follow the LP-SS periodicity to do the measurement instead of DRX in legacy case, then we will probably use the different sweeping factor for different LP-SS periodicity; and furthermore if LP-SS has multiple repetitions inside each LP-SS occasion, UE may perform Rx beam sweeping on these repetitions and save the N1 for LP-SS occasion measurement. The most straightforward requirement could be that: the different Rx beam sweeping factor(N1) shall be applied for different LP-SS periodicities, and the N1 shall be decreased when LP-SS periodicity increases. However, since the LP-SS design is still on-going in RAN1, RAN4 can discuss the detailed requirement in future meetings.

Proposal 26: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

Issue 1-6-2: LR based RRM with EMR in IDLE/inactive mode

In RAN4 #111 and 112 meeting, the different cases for MR offloading and relaxation are captured in the WF R4-2410296 and R4-2413899, and so far at least case#1 and case#3 are valid case (like in proposal 13) to consider in this WI,

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

However, the EMR measurement has not been considered yet for both case #1 and case #3. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running, like followings:

Then, if EMR measurement is also configured and TS331 is running for case#1 and case#3, the MR status may need further discussion. For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, 	UE may keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 27: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR. As long as one kind of measurement results (either from MR or LR) can meet their own threshold, UE can take the corresponding action, e.g., entering MR fully offloading (from case #3 to case #1), or change from case #3 to case #5.

Proposal 2: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- FFS on measurement requirements for OFDM based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state after RAN1 has concrete conclusion.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: the criteria design for RRM relaxation of UE MR for both serving and neighbor cell measurements can be left to RAN2 to decide

Proposal 6: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 7: use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

Proposal 8: Depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria; and followings are considered:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the MR/LR serving cell measurement delay requirements for entry/exit criteria evaluation.

Proposal 9: Follow following RAN2 agreements in RAN2 #127 meeting for issue 1-1-4-2.

- Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

- Working assumption: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

- The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

- The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Proposal 10: RAN2 shall be the main group for criteria (entry/exit conditions) design

Proposal 11: RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 12: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Proposal 13: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 14: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

- Alt 2 (preferred): MR is ON with RRM measurement on serving cell and neighbour cell (if any). But whether or not LP-WUR is ON for RRM measurement is up to UE implementation.

Proposal 15: higher priority layer neighbor cell measurement shall also be OFF for case #1.

Proposal 16: this issue 1-1-12 can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 17: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Proposal 18: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 19: No need to discuss the LR based S criterion evaluation in case#1.

Proposal 20: to define RRM requirement, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has more conclusions.

Proposal 21: Single Rx is assumed for LR based RRM measurement.

Proposal 22: LP-SS measurement delay requirement in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Proposal 23: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

Proposal 24: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

Proposal 25: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

Proposal 26: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

Proposal 27: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# References

[1] R4-2413899, WF for RRM core requirements for LP-WUSWUR, Moderator (vivo), RAN4 #112

[2] R4-2411818, Topic summary for [112][223] NR_LPWUS, RAN4 112, vivo, RAN4 #112

[3] RP-242361, Way forward on LPWUS for FR2, vivo, RAN #105