3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415789

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Analysis for NR NTN HPUE Rx Requirements

Agenda Item:	6.8.2.2.2

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. In it, the key agreements:

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

Issue 4-2: RSD for NR-NTN

- Agreement

  - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE.

    - Further discuss the following aspects:

      - Use similar way in TN spec by introducing a new table of RSD compared to the PC3

      - Whether default Tx-Rx frequency separation is used for test point

A paper [4] was submitted for including an initial study on NTN HPUE RF Rx Requirements for both NR-based and Iot-based NTN HPUE Tx requirements, in which some topics were not really discussed.

In this paper, a discussion of the impact of HPUE to RSD requirements for NR NTN are provided.

HPUE Impact for NR-NTN RSD

PC2

As discussed in [4], it is proposed to reuse requirements based on similar cases.

For example, band n1 does not suffer from own Tx leakage, thanks to the relatively large duplex distance. The RSD values are all 0dB for n1 for power class 2. It can be seen that frequency separation of n1 is 190MHz.

By comparison, n256/n255/n254 also have large frequency separation as following:

The basic situations are:

- N256: separation same to n1, also in 2GHz range;

- N255: similar to n24, smaller gap but also lower frequency;

- N254: very large separation

At least for n256 and n254, it can be safe to say that the RSD situation would either similar or better compared to n1, thus RSD =0 can be foreseen. N255 may need more study.

A number of references has been mentioned in [4], such as [6][7][8].

Proposal 1: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

As already been agreed, power class 2 is confirmed to be feasible for both handheld and non-handheld UE.

It should be noted currently there is no differentiation of handheld / non-handheld UE for the RSD requirements, and the understanding is these requirements are based on common components and architectures.

Proposal 2: For power class 2, there is no differentiation of handheld and non-handheld UE.

PC1.5

Since there is no FDD PC1.5 defined in TN yet, there is no clear precedence for PC1.5 NR NTN requirements, and there is no conclusion yet on the PC1.5 of NR-NTN yet.

However, according to the discussion on [5], PC1.5 is feasible for PC1.5 2Tx NR NTN, and there is proposal to keep PC1.5 for 2Tx at least.

If this could be agreed, then it should not be too difficult for PC1.5 2Tx RSD derivation, since its similarity to PC2 2Tx in the derivation process.

Observation 1: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx.

Proposal 3: For power class 1.5, more study on RSD is needed for NR-NTN, depending on the conclusion on the feasibility and agreed architecture.

Furthermore, it should be noted the feasibility study of PC1.5 2Tx for NR NTN is based on handheld UE, thus the requirements should also be based on handheld UE. According to tradition and typical implementation, even the non-handheld type can share the same requirements with handheld ones as long as handheld is feasible.

Whether PC1.5 1Tx should be considered or not is FFS according to [4].

Proposal 4: If PC1.5 2Tx RSD is studied, it should be based on handheld UE, and can also applied to non-handheld.

PC1

It is noted that there are already a number of PC1 requirements defined in NR and LTE, and all of them are 1Tx architecture and for non-handheld type. This is easy to be understand since the duplexer that could be working in the PC1 is beyond what a handheld UE could use.

It should be noted that currently PC1 often typically reuse Rx requirements from PC3, basically means that similar self-dense is expected. A number of TRs such as [9][10][11] have documented some study history and derivation of requirements.

Observation 2: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

According to the feasibility analysis in [5], PC1 1Tx for NR-NTN is also proposed to be non-handheld only.

According to clause 6.3 “UE self-dense” of [9], the RSD of several new bands were based on the comparison of bands 14, 20 and 28 which at that time is already in the spec. The comparison is based on a “difficulty rank” which is associated with the spectrum arrangement including the channel bandwidth and UL-DL separation. Although the details can still be discussed, it is highly likely that the current band under discussion n254/n255/n256 would be categorized as “easy bands” since the duplex gap is fairly large.

Observation 3: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Based on the analysis and observations, the following proposal is provided:

Proposal 4: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

PC2

Proposal 1: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2: For power class 2, there is no differentiation of handheld and non-handheld UE.

PC1.5

Observation 1: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx.

Proposal 3: For power class 1.5, more study on RSD is needed for NR-NTN, depending on the conclusion on the feasibility and agreed architecture.

Proposal 4: If PC1.5 2Tx RSD is studied, it should be based on handheld UE, and can also applied to non-handheld.

PC1

Observation 2: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 3: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412100, Initial analysis for NTN HPUE RF Rx Requirements, vivo, RAN4#112

[5] R4-2415788, Feasibility study and analysis for NR NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] TR 38.861, Study on high power UE (power class 2) for one NR FDD band

[7] TR 38.896, High power UE (power class 2) for NR FR1 FDD single band

[8] R4-2307239, n8 PC2 RSD for 25MHz DL CBW, Skyworks, RAN4#109

[9] TR 37.880, High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

[10] TR 37.828, High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands

[11] TR 37.829, High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18