3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415863

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM requirements for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.8.3.4

Document for:	Discussion

Introduction

RRM requirements less than 5MHz in NTN are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the specific RRM requirements listed under sub-topic 1-3 of [1].

In this paper, we will provide our views on RRM requirements for less than 5MHz for NTN.

Discussion

HO

Issue 1-3-2: Handover Agreement: Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements.

In TN HO requirements, the impact of PBCH puncturing in SSB of a target cell with less than 5MHz BW is captured by increase of T∆.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

In NTN specific HO requirements including CHO and satellite switch, T∆ is also included as the time for fine time tracking and acquiring full timing information of the target cell. It is straightforward to update the T∆ in these requirements in the same way as in TN, i.e. from Trs to 3*Trs.

Proposal 1: For CHO and satellite switch to an NTN target cell with less than 5MHz, update the T∆ in the corresponding requirements from Trs to 3*Trs.

Timing

Issue 1-3-3: whether timing requirements for NTN in current TS38.133 will be impacted becasue of less than 5MHz bandwidth WF: FFS on - Option 1: - No impacts

In TN, the timing requirements are same for UE operating in a cell with normal BW and with less than 5MHz BW. NTN timing requirements are based on TN, with additional consideration of UE GNSS error and satellite location estimation error. The additional consideration factors are not related to serving cell BW, so we do not see any reason to change NTN timing requirements for less than 5MHz BW.

Proposal 2: RAN4 to confirm existing timing accuracy requirements apply for UE operating in an NTN cell with less than 5MHz.

RLM/BFR

Issue 1-3-4: RLF/BFD/CBD WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: - The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario

In TN, the hypothetical PDCCH parameters for RLM and BFD are updated for less than 5MHz. In particular, 3 sets pf parameters are defined for serving cell DL Transmission BW of 12RB, 15 RB and 20RB. In our view, the parameters from TN can be re-used for NTN with less than 5MHz.

On the other hand, in RF session currently only 15RB DL Transmission BW is supported for NTN, so RAN4 should define hypothetical PDCCH parameters for RLM and BFD at least for 15RB, and other BW can be considered if they are supported in RF session.

Proposal 3: RAN4 to define hypothetical PDCCH parameters for RLM and BFD at least for 15RB for UE operating with less than 5MHz in an NTN cell.

For the RLM and BFD evaluation period, in TN the requirements are same for cell with normal BW and with less than 5MHz BW. The reason is that PSS and SSS which are used for the evaluation are not impacted by the reduced cell BW. The same reason applies also for NTN, so the existing NTN RLM and BFD evaluation periods should be reused for less than 5MHz in NTN.

Proposal 4: Existing NTN RLM and BFD evaluation periods are reused for less than 5MHz in NTN.

L3 measurement

Issue 1-3-5: Measurement(SSB index detection requirements) WF: FFS on: - Proposal 1: Whether the time period for time index detection within NTN intra/inter-frequency measurements with/without MG shall be redefined. - Option 1: Yes - Option 2: FFS - Proposal 2a: - The new SSB index detection requirements can be defined by add number of SSB samples as Rel18 TN with less than 5MHz. - Proposal 2b: - RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap

In TN, the delay requirements for SSB index reading in L3 measurement are updated for less than 5MHz by adding 4 and 3 samples on top of TN requirements for normal cell BW, for intra- and inter-frequency. As the side condition and requirements for normal cell BW in NTN are same for TN, it is straightforward to re-use the number of additional samples as in TN.

Proposal 5: RAN4 to update the delay requirements for SSB index reading for less than 5MHz in NTN by adding X SSB samples on top of existing requirements, where X is same as in TN less than 5MHz.

L1 measurement

Issue 1-3-6: L1-RSRP Measurement WF: FFS on - Proposal 1: - For L1-RSRP measurement based on SSB there is no RRM impacts,

In TN the requirements for L1 measurement are same for cell with normal BW and with less than 5MHz BW. The reason is that PSS and SSS which are used for the measurement are not impacted by the reduced cell BW. The same reason applies also for NTN, so the existing NTN SSB based L1 measurement periods should be reused for less than 5MHz in NTN.

Proposal 6: RAN4 to confirm no impact on SSB based L1 measurement requirements for supporting less than 5MHz in NTN.

Conclusions

In this paper we provided provide our views on RRM requirements for less than 5MHz for NTN.

Proposal 1: For CHO and satellite switch to an NTN target cell with less than 5MHz, update the T∆ in the corresponding requirements from Trs to 3*Trs.

Proposal 2: RAN4 to confirm existing timing accuracy requirements apply for UE operating in an NTN cell with less than 5MHz.

Proposal 3: RAN4 to define hypothetical PDCCH parameters for RLM and BFD at least for 15RB for UE operating with less than 5MHz in an NTN cell.

Proposal 4: Existing NTN RLM and BFD evaluation periods are reused for less than 5MHz in NTN.

Proposal 5: RAN4 to update the delay requirements for SSB index reading for less than 5MHz in NTN by adding X SSB samples on top of existing requirements, where X is same as in TN less than 5MHz.

Proposal 6: RAN4 to confirm no impact on SSB based L1 measurement requirements for supporting less than 5MHz in NTN.

Reference

- R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, Moderator (Xiaomi)