3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415865

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	6.23.4.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR relaxation

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

We do not support to consider Case #2.

- There is no clear necessity to require MR to measure the serving cell since LR is already measuring the serving cell.

  - In particular, the MR measurement will be with relaxation, so it is much more infrequent compared to the LR measurement.

  - Some companies commented that LR measurement may be less accurate than that of MR. In our view, however, it is both necessary and feasible to keep similar accuracy between LR and MR.

  - Some companies commented that MR measurement may be used by UE to assist LR measurement. We believe this can be left to UE implementation rather than mandated by requirements.

- There is additional power consumption due to MR measurement even with relaxation.

- It will complicate the spec since another threshold (coverage level) is needed to switch between Case #1 and Case #2.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

We support to consider Case #4. In our understanding, Case #4 is a specific case when neighbour cell measurement is triggered by higher priority carriers. Since evaluation for cell reselection to a higher priority carrier does not require a comparison between serving cell and neighbour cell measurement, there is no need to require UE to use MR to measure serving cell.

Proposal 2: RAN4 to support Case #4 for MR and LR measurement.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement | Remarks |
| --- | --- | --- | --- | --- |
| #4 Relaxed case c | OFF | ON with relaxation measurement | ON | Neighboring cell measurement on higher priority carrier is triggered |

On combining the MR and LR measurement, we support option 1, i.e. no combining is considered from RAN4 requirement perspective. In our views, measurements Case #3 are used for evaluating

- a) Whether LR exit criterion is met

- b) Whether neighbor cell measurement start or stop criterion is met

- c) Whether a neighbor cell meets cell reselection criterion

For each criterion, the spec should make it clear whether the evaluation is based on MR or LR, by defining corresponding threshold. For example, evaluating LR exit condition should be based on LR measurement, and threshold for LR measurement should be defined. The measurement should meet the delay and accuracy defined for LR. Of course, UE implementation can use LR, MR or both to get the measurement as long as it can meet the requirements. We do not see the need to specify combined LR and MR measurement in spec. Also, it may be quite complex to define delay and accuracy requirements for combined measurement.

Proposal 3: RAN4 not to consider combining the measurements across LR and MR from requirement perspective. LR and/or MR measurement can be individually used for evaluation of each criterion.

Since RAN4 agreed to support Case #1 and #3, we need to discuss how UE switches between the two cases. In our view, the switch depends on whether neighbour cell measurement is triggered. Currently, start or stop of neighbour cell measurement is based on evaluation of serving cell against threshold SIntraSearchP, SIntraSearchQ, SnonIntraSearchP or SnonIntraSearchQ, which are thresholds for MR measurement.

For Case #1 to Case #3, only LR measures serving cell in Case #1, so LR measurement should be used to evaluate whether the neighbour cell measurement start criterion is met. LR measurement based on SSB is directly comparable against the existing thresholds, but LR measurement based on LP-SS is not. One option is to define new threshold for LP-RSRP and LP-RSRQ, and another option is to use the existing threshold with an offset. We suggest RAN4 to further discuss the two options.

Proposal 4: RAN4 to discuss following options for switch from Case #1 to Case #3 based on LR measurement.

- Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

- Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

For Case #3 to Case #1, both LR and MR measure serving cell in Case #3, so RAN4 needs to further discuss which measurement(s) is used for the evaluation. In our view, LR measurement should be at least considered because it is much more frequent than MR. On the other hand, stopping neighbour cell measurement is not time critical from mobility perspective, so MR measurement can also be included. In this case, UE should stop neighbor cell measurement only when both measurements are below their corresponding thresholds, as otherwise it may cause ping-pong. We suggest RAN4 to further discuss the following options.

Proposal 5: RAN4 to discuss following options for switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

- Option 1: LR measurement only

- Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

We support to define following requirements for different LR architectures.

- OOK based LR measurement on LP-SS

- OFDM based LR measurement on LP-SS

- OFDM based LR measurement on SSB

1) is straightforward since OOK based LR can only measure LP-SS. OFDM based LR can measure both LP-SS and SSB, so we support to define requirements for both 2) and 3). If only one of 2) and 3) is defined, it means that OFDM based LR is mandated to measure a certain signal, which unnecessarily limits the flexibility of OFDM based LR.

RAN4#110-bis already agreed to define requirements for 1) and 3). For 2) some companies proposed to suspend the discussion until RAN1 makes conclusion on whether overlaid sequence on LP-SS is supported. In our view, RAN4 should define requirements for 2) no matter of RAN1 supports overlaid sequence on LP-SS or not. Supporting overlaid sequence on LP-SS will allow better sync and measurement performance for OFDM based LR, but OFDM based LR can use LP-SS for sync and measurement without overlaid sequence.

One may ask if OFDM based LR can measure SSB, why does it need to measure LP-SS. In our view, LP-SS can be assumed at the same center frequency as LP-WUS, and it is very likely to be outside the bandwidth of initial DL BWP containing SSB, in order to avoid reducing the scheduling resources for common channels. Using LP-SS allows OFDM based LR to perform sync and RRM measurement without RF re-tuning. Also, the measurement from LP-SS can better reflect the channel quality for LP-WUS reception compared to measurement from SSB which may be far away from LP-WUS in frequency domain.

Based on above discussions, we suggest RAN4 to define requirements for 2). The exact requirements and applicability can be FFS until RAN1 conclusion on whether to support overlaid sequence on LP-SS, as well as RAN4 initial evaluation on the performance of 1), 2) and 3), are available.

Proposal 6: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. Exact requirements and applicability are FFS.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

As RAN4 has agreed to support Case #3 for MR and LR measurement, RAN4 should define relaxed MR measurement requirements for both serving and neighbor cells.

It is noted that Case #3 are when serving cell RRM is offloaded to LR, so the condition for MR relaxation should not be “UE supporting LP-WUR”. It is up to RAN2 to decide whether MR relaxation is supported for UE supporting LP-WUR before serving cell RRM is offloaded to LR.

Proposal 7: RAN4 to specify relaxed MR measurement requirements for both serving and neighbor cells when serving cell RRM is offloaded to LR. Wait for RAN2 conclusion on whether MR relaxation is supported before serving cell RRM is offloaded to LR for UE supporting LP-WUR.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Recommendations: Suggest to discuss this issue on the separately issue listed as the following: - On requirements for evaluating entry/exit criteria for LP-WUR monitoring - On requirements for evaluating entry/exit criteria for RRM relaxation and serving cell measurement offloading

In RAN2#126, the following is agreed regarding entry/exit of LP-WUS monitoring.

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Our understanding of the agreement is that

- When the entry condition for LP-WUS monitoring is met, UE may monitor LP-WUS with LP-WUR and if it does so, it may turn off MR for paging monitoring, and UE should wake up MR for paging reception when it receives WUS targeting at it.

- When the exit condition for LP-WUS monitoring is met, UE should turn on MR for paging monitoring.

For the procedure to work properly, RAN4 needs to define requirement for serving cell evaluation against the entry/exit condition for LP-WUS monitoring. The requirements should include at least 3 aspects – measurement delay, measurement accuracy and measurement filtering.

Measurement delay

RAN4#110-bis already agreed to define requirements for LR measurement which is used for evaluating the exit condition, and it at least includes the measurement delay in terms of number of samples. What RAN4 has missed so far is the requirements for MR measurement which is used for evaluating the entry condition, but most likely the existing serving cell evaluation requirements in clause 4.2.2.2 (for evaluation against the S-criterion) can be re-used also for evaluation against the entry condition for LP-WUS monitoring.

Measurement accuracy

RAN4#110-bis also discussed the measurement accuracy, but it was left open whether and how to define the accuracy requirement. In our view, the accuracy requirements are important to make sure that UE makes correct evaluation of entry/exit conditions based on LR measurement, so it should be defined, and how it is defined is discussed in Issue 2-1-1 in this paper.

Measurement filtering

The issue was raised up in RAN4#111 by P1-1 of Issue 1-1-4. In general, we support to define filtering as part of the requirements for serving cell evaluation against the entry/exit condition, which is also defined in existing serving cell evaluation requirements in clause 4.2.2.2 (for evaluation against the S-criterion).

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

In our view, the filtering requirement is different from delay requirement. The latter defines the upper limit on the time that UE can use to achieve certain accuracy, while the former defines the lower limit on the time the UE has to spend on the evaluation to eliminate the impact of fast fading. The filtering requirement also defines the minimum separation between samples.

Summary

Based on above discussions, we have the following proposal on the RRM requirements to be defined.

Proposal 8: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including

- Measurement delay (number of samples)

- Measurement accuracy (margin for evaluation)

- Measurement filtering (number of samples and separation between samples)

Issue 1-1-4-1: Entry/exit criteria evaluation when results from MR and/or LP-WUR are avaiable for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Recommendations: Companies are encouraged to provide views on whether combine results from LP-WUR and MR is needed or if so, solutions on how to combine

In our view, evaluation of the entry/exit condition should be based on individual measurement from LR and/or MR, depending on how the thresholds for the condition are defined in RAN2.

- If a single threshold is defined for either LR or MR, then LR or MR measurement should be used.

- If both thresholds are defined for LR and MR, then both LR and MR measurement should be used individually, i.e. LR measurement is used to compare with LR threshold, and MR measurement is used to compare with MR threshold.

We do not see the need to support combined LR and MR measurement as in Proposal 3. On the other hand, we agree that LR and MR measurements should be comparable, so that the LR threshold can be derived from the MR threshold with an offset. We understand this is guaranteed by the measurement definition in RAN1, and we do not see additional RRM requirements needed.

Issue 1-1-4-2: Reference signals and measurement types to be used for Entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

When LR measurement is used for evaluation, different thresholds are needed for LR measurement based on SSB and LP-SS. By definition, LP-RSRP/RSRQ is not directly comparable with SS-RSRP/RSRQ, and as we discussed in Proposal 4, we can either define new thresholds for LP-RSRP/RSRQ or define offsets to make SS-RSRP/RSRQ based thresholds applicable for LP-RSRP/RSRQ.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement

Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

In RAN2#126, the following agreement was reached regarding serving cell RRM measurement offloading.

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

It can be seen that RAN2 will focus on specifying the offloading criterion. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR, unless triggered by other WGs.

Proposal 9: RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR unless triggered by other WGs.

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

In RAN2#126, the following agreement was reached regarding MR measurement relaxation.

- RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

It can be seen that RAN2 will further discuss the neighbor cell measurement relaxation criteria. The MR measurement of serving cell can be discussed after RAN4 concludes on the Cases/States to be considered for MR and LR measurement as discussed in section 2.1.2. To avoid duplicated discussion, we suggest RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

Proposal 10: RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself.

RAN1/2 has agreed on the baseline for the entry/exit condition for LP-WUS monitoring, and it is based on serving cell quality, i.e. RSRP and RSRQ. In our view, serving cell quality mainly reflects the coverage level, and another factor that needs to considered for determining whether UE should enter or exit LP-WUS monitoring is the inter-carrier interference level.

As the dynamic range of LR is smaller compared to MR, e.g. the bitmap for ADC is assumed to be 4 or 8 bits, it is more likely to get blocked by the inter-carrier interference. The issue was discussed in RF session, where the conclusion was that “RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR”.

In our view, interference level that LR can tolerate is smaller than that for MR. On the other hand, the actual interference level does not change depending on whether UE is using LR or MR in another carrier which may be used by another operator. Therefore, even UE is close to the serving gNB, i.e. the serving cell RSRP and RSRQ are high, the LR may not be usable because of high inter-carrier interference. In this case, the UE should not use LR as the LP-WUS detection performance cannot be guaranteed.

We understand the key question is whether RSRP and RSRQ measurement from LR can indicates the inter-carrier interference level. If so, we may not need additional entry/exit condition; if not, additional entry/exit condition on inter-carrier interference level besides the serving cell quality should be considered.

Proposal 11: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference.

In RAN2#127 the following working assumption was made, and it is subject to RAN1/4 check.

- Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

In our view, LR based threshold is not needed for the entry condition of LP-WUS monitoring.

- It complicates UE implementation as UE cannot turn off LR after exiting LP-WUS monitoring or before entering LP-WUS monitoring.

- UE always has the freedom to include other conditions than MR measurement to determine whether to enter LP-WUS monitoring. UE is allowed to start monitor LP-WUS and stop monitoring the legacy PO, but is not mandated to do so when entry condition is met.

- UE can exit LP-WUS monitoring after it finds the LR measurements meets the exit condition.

- It complicates RAN4 requirements for evaluation of LP-WUS entry condition.

Based on above reasons, we have the following proposal.

Proposal 12: RAN4 to informs RAN2 to revisit the working assumption on entry condition of LP-WUS monitoring: LR based thresholds are not considered for LP-WUS monitoring entry condition.

Issue 1-1-8: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

In our view, the exact entry/exit conditions for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation are to be discussed in RAN2. If RAN4 is expected to provide some high-level suggestion to RAN2, we suggest that the 3 events are based on same entry/exit condition.

- This will allow meaningful UE power saving, e.g. if UE skips PO monitoring but the measurement still relies on MR without relaxation, MR cannot enter ultra-deep sleep mode and there is not much power saving gain compared to the case when UE still monitors PO.

- This will simplify the spec and NW/UE implementation. For example, NW can configure a single threshold to enable meaningful UE power saving rather than manipulating multiple thresholds which results in many combinations most of which are artificial and not very meaningful for UE power saving.

Proposal 13: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation.

Issue 1-1-9: LP-WUR status

In our understanding, the LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring is depending on the entry condition of LP-WUS monitoring as discussed in Issue 1-1-7. If RAN2 working assumption is confirmed, then LR needs to be turned ON and is required to perform serving cell measurement to evaluate the entry condition of LP-WUS monitoring. Otherwise, LR can be turned OFF.

Proposal 14: Postpone the discussion on LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring after RAN4 concludes on entry condition of LP-WUS monitoring.

Issue 1-1-10: UE measurement behivour after receiving LP-WUS signal

Recommendations: Maybe it is not a RAN4 issue, suggest to check RAN1/2 for clarification.

In our understanding, when UE receives LP-WUS, UE will wake up MR to monitor PO. The time period from when UE receives LP-WUS to when UE receives PO is a transition period where UE will do MR wake up and sync. As MR is not fully operating, we do not think RAN4 should define measurement requirements for MR during this time period.

After UE reads PO, there can be 2 cases

- If UE finds it is paged, then it becomes legacy scenario where UE should follow existing requirements for MR measurement

- If UE finds it is not paged, then UE goes back to LP-WUS monitoring and should follow requirements for LR and possibly also MR measurement as discussed for Case #1, #3 and #4 in Issue 1-1-1.

In both cases, we do not see additional RRM requirements needed.

Proposal 15: RAN4 not to define MR measurement requirements for the time period from when UE receives LP-WUS to when UE receives PO.

Issue 1-1-11: Considerations on higher priority frequency layer

Recommendations: Companies are encouraged to bring solutions/views on this issue.

In our view, UE should always perform measurement on higher priority carriers, if configured. Configuring higher property carriers is an important tool from NW perspective to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE, and it should not be totally compromised due to power saving. Otherwise, it may impact the usage of the LP-WUR feature, i.e. NW may not enable LP-WUR feature when it wants to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE.

In current spec, the requirements for higher priority carriers depends on whether serving cell condition:

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, higher priority carriers are measured according to requirements in clause 4.2.2.7, i.e. every (60 * Nlayers) s. This is already a relaxed requirement compared to the case below. We are open to discuss whether the requirements should be further relaxed when LR measurement is ON (i.e. Case #4).

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, higher priority carriers are measured together with other carriers according to requirements in clause 4.2.2.4. When LR measurement is ON, the measurement should be relaxed based on agreement of Case #3 in Issue 1-1-1.

Proposal 16: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

LP-WUR measurement

Issue 1-2-1: Accuracy requirements

Recommendations: How and where to capture the accuracy requirement can be discussed later.

Among the options for how to specify the requirements, we support option 2, which is same as cell reselection requirements, where accuracy is implicitly defined as margin (highlighted in yellow in the copied requirements). So far, dedicated accuracy requirement in clause 10 are all for reported measurement in RRC_CONNECTED, and we prefer to keep same principle for this WI.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Proposal 17: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: Possible interruption related requirements for LP-WUR

Recommendations: Companies are encouraged to bring solutions/views on this issue.

In RAN1#118 the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

In our view, it may be too early to conclude whether RAN4 needs to define interruption requirements as it depends on how UE should monitor PO after it receives LP-WUS which has not been concluded in RAN1.

It is noted that if UE is not expected to receive any PO in a time period following RAN1 spec, e.g. during the time period between a LO and a reference PO/PF, then RAN4 does not need to define interruption for this time period.

Proposal 18: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Issue 1-2-3: Possible cell selection evaluation requirements for fully offloading case based on LP-WUR

Recommendations: Companies are encouraged to bring solutions/views on this issue.

We do not see the need to define cell selection evaluation requirements for LR. In a reasonable deployment, when UE is close to cell edge where S criterion needs to be evaluated, UE should have already exited LR measurement, and MR measurement should have started.

Proposal 19: RAN4 not to define cell selection evaluation requirements for LR.

Issue 1-2-4: LP-WUR operating carrier frequency

We support to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency. In our understanding, RAN1/2 have not supported the scenario where LR and MR are operating on different carrier frequencies, which technically will make UE implementation more complex and reduce the power saving gain due to RF retuning.

Proposal 20: RAN4 to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency.

Issue 1-2-6: Nubmer of Rx for LP-WUR requirements

Recommendations: Encourage companies to check whether P1 of this issue is agreeable or not.

We support to define LR requirements based on the assumption of 1RX because LR is a low power and low cost device, and such devices are typically equipped with 1RX.

Proposal 20: RAN4 to define LR requirements based on the assumption of 1RX.

Issue 1-2-7: Other considerations

In our view, there is no strong need to evaluate the entry condition of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED. Entering LR is anyway not a time critical task, and it is more reliable to use the measurements in RRC_IDLE or RRC_INACTIVE to evaluate the entry condition. Coupling the entry condition with MR measurement in RRC_CONNECTED will complicate UE implementation.

Proposal 21: RAN4 not to consider evaluation of entry conditions of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED.

MR relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

On the delay requirements for relaxed MR measurement, as the relaxation is aimed at power saving, we believe the power saving gain should be the main factor to determine the relaxation factor. Figure 1 shows our evaluation on the power saving gain from SI for MR relaxed measurement, with different relaxation factors.

Figure 1: Power saving gain for MR relaxed measurement with different relaxation factors

It can be seen that the power saving gain reduced obviously when relaxation factor becomes smaller. With 10 times relaxation, the gain is 20% in ideal case for paging, and to achieve meaningful gain in realistic case, the relaxation factor needs to be > 20. We suggest RAN4 to consider 16 as the smallest relaxation factor.

Proposal 22: For MR measurement with relaxation the relaxation factor should be >= 16.

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

In our view, serving and neighbor cell measurements should share the same scaling factor because the power consumption of the MR measurement is determined by the more frequent measurement among serving and neighbour cell, i.e. the smaller scaling factor among the two. Meaningful power saving gain is obtained only when both measurements are relaxed with relaxation factor >= 16.

Proposal 23: Same relaxation factor applies to serving and neighbor cell measurements.

Issue 1-3-3: Accuracy for relaxed MR measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 24: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Recommendations: Suspend discussion on this topic until more progress from other group.

It is true that the objective of LP-WUR at RRC_CONNECTED mode does not involve RAN4, but it may be too earlier to preclude any RRM impacts as the RAN1/2 discussions are ongoing, and there may be RRM impacts. RAN4 can first focus on the LP-WUR requirements in RRC_IDLE or RRC_INACTIVE, and postpone the discussion for RRC_CONNECTED mode until more RAN1/2 conclusions are available.

Proposal 25: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

Proposal 2: RAN4 to support Case #4 for MR and LR measurement.

Proposal 3: RAN4 not to consider combining the measurements across LR and MR from requirement perspective. LR and/or MR measurement can be individually used for evaluation of each criterion.

Proposal 4: RAN4 to discuss following options for switch from Case #1 to Case #3 based on LR measurement.

- Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

- Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

Proposal 5: RAN4 to discuss following options for switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

- Option 1: LR measurement only

- Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

Proposal 6: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. Exact requirements and applicability are FFS.

Proposal 7: RAN4 to specify relaxed MR measurement requirements for both serving and neighbor cells when serving cell RRM is offloaded to LR. Wait for RAN2 conclusion on whether MR relaxation is supported before serving cell RRM is offloaded to LR for UE supporting LP-WUR.

Proposal 8: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including

- Measurement delay (number of samples)

- Measurement accuracy (margin for evaluation)

- Measurement filtering (number of samples and separation between samples)

Proposal 9: RAN4 not to further discuss the entry/exit condition for serving cell RRM measurement offloading to LR unless triggered by other WGs.

Proposal 10: RAN4 not to further discuss the entry/exit condition for MR RRM measurement relaxation, unless triggered by other WGs.

Proposal 11: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference.

Proposal 12: RAN4 to informs RAN2 to revisit the working assumption on entry condition of LP-WUS monitoring: LR based thresholds are not considered for LP-WUS monitoring entry condition.

Proposal 13: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation.

Proposal 14: Postpone the discussion on LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring after RAN4 concludes on entry condition of LP-WUS monitoring.

Proposal 15: RAN4 not to define MR measurement requirements for the time period from when UE receives LP-WUS to when UE receives PO.

Proposal 16: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

Proposal 17: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Proposal 18: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Proposal 19: RAN4 not to define cell selection evaluation requirements for LR.

Proposal 20: RAN4 to define RRM requirements based on the assumption that LR and MR are operating on the same carrier frequency.

Proposal 20: RAN4 to define LR requirements based on the assumption of 1RX.

Proposal 21: RAN4 not to consider evaluation of entry conditions of LP-WUS monitoring, LR measurement or MR measurement relaxation based on MR measurement in RRC_CONNECTED.

Proposal 22: For MR measurement with relaxation the relaxation factor should be >= 16.

Proposal 23: Same relaxation factor applies to serving and neighbor cell measurements.

Proposal 24: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

Proposal 25: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo