3GPP TSG-RAN4 Meeting #112-bis	R4-2415890

# Hefei China, 14th – 18th October, 2024

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Testability

Agenda item:	6.17.3.1

Document for:	Discussion

# Introduction

In this paper, we focus on CSI prediction and address the issues that were unresolved during the last meeting.

# Prior Agreements

Following agreement happened during the last meeting [1].

Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

# Discussion

RAN4 already agreed relative throughput to be the requirement for CSI prediction. The PMI reporting test cases of [2] also uses relative throughput to be the requirement. According to [2], “The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively”

PMI prediction related test cases of Rel-18, e.g., “Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI” (6.3.2.1.8 of [2]) use random precoding based throughput as the baseline, too.

AI-ML based CSI prediction tests can follow the flow of Rel-18 PMI prediction related test cases and use random precoding-based throughput as the baseline throughput.

Observation 1: PMI prediction related test cases of Rel-18, e.g., “Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI” (6.3.2.1.8 of [2]) use relative throughput as requirement metric. Random precoding-based throughput is used as the baseline in those test cases.

Proposal 1: RAN4 selects random precoding-based throughput as the baseline in the test cases for AI-ML based CSI prediction where relative throughput is used as requirement metric.

Proposal 2: The test case for AI-ML based CSI prediction can follow the flow of PMI prediction related test cases of Rel-18.

# Conclusions

Observation 1: PMI prediction related test cases of Rel-18, e.g., “Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI” (6.3.2.1.8 of [2]) use relative throughput as requirement metric. Random precoding-based throughput is used as the baseline in those test cases.

Proposal 1: RAN4 selects random precoding-based throughput as the baseline in the test cases for AI-ML based CSI prediction where relative throughput is used as requirement metric.

Proposal 2: The test case for AI-ML based CSI prediction can follow the flow of PMI prediction related test cases of Rel-18.

# References

[1] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[2] 38.101-4, “UE radio transmission and reception; Part 4: Performance requirements (Rel 18)”, V18.5.0