3GPP TSG-RAN WG4 Meeting #112bis	R4-2415965

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On AI/ML Based CSI Compression

Agenda item:	6.7.2.1

Document for:	Discussion

Introduction

This contribution is devoted to the discussion of requirements, testability, and interoperability aspects of AI/ML-enabled CSI reporting use-case.

The active discussion of the approaches to define and derive reference and test decoder model for two-sided CSI compression model has continued at RAN4#112. Further steps in the study of training and selecting models with CNN architecture (Option 3) still contain several open issues [1] that are discussed in the paper. It is also expected that the companies will update their simulation results based on the parameters further aligned in [2]. Correspondingly, in the paper we share the results of encoder and decoder training based on the agreed parameters. Additionally, we check the performance of the trained models on the datasets provided by other companies (OPPO and CIATC) and different from the one used for training. Furthermore, we elaborate on the derivation/selection of the test decoder and reference encoder following Option 4 (i.e., when the exact models are not expected to be specified) and verification of the test decoder.

Discussion

Results of decoder/encoder training

According to the agreements from the previous meeting RAN4#112 [2], the following CNN based reference encoder and decoder model parameters were agreed:

- For the results with quantization, to use a sigmoid function in the encoder for normalization

- Sigmoid in the decoder is optional so it was not used by us

- Not to use drop-out layers

- Use (1-SGCS) as loss function as reference

- Use Adam optimizer

- Fixed learning rate of 10^-4

- Batch Size: 256

- 150 epochs

- Stopping Criterion: No improvement over 25 epochs.

- Weight initialization: Default

- Kernel Parameters: Default

With these agreed parameter settings also summarized in Table 1, we have updated the CNN based reference encoder and decoder model performance, as shown in Table 2. The encoder and decoder were trained following Type 1 procedure (i.e., joint training) on a 630K sample dataset with 80% used for training, 10% for validation, and 10% for testing. The total trainable parameters are about 1.93M, with achieved mean training and testing SGCS performance of 0.721 and 0.705 at 64 bits overhead, respectively. It is worth nothing that no dropout layer is used so the training and testing SGCS performance have some slight difference. Compared to the Rel16 eTypeII codebook (0.691 mean SGCS for parameter combination 1), 2% SGCS gain is observed.

Table 1: Agreed Hyperparameters used during Type I joint training.

| Hyperparameter | Value |
| --- | --- |
| Optimizer | Adam |
| No of Epochs | 150 |
| Batch Size | 256 |
| Learning rate | 10-4 |
| Validation patience | 25 |
| Loss function | 1-SGCS |
| L2 regularization coefficient (λ) | 0 |
| Uniform Scalar Quantization | 2 bits |

The same model, with ideal hyperparameter settings, is also trained on datasets publicly shared from OPPO and CAICT, containing 600K and 100K samples, respectively. The training and testing SGCS performances are detailed in Table 2. Given that the validation patience is set to 25, the actual number of training epochs is less than 150 epochs for all three datasets. Table 3 presents the training and testing SGCS performances for the same model, with ideal hyperparameter settings, but trained without quantization to eliminate the impact of quantization.

Table 2: Trained AI/ML encoder-decoder pair SGCS performance with 2-bit quantization and 64-bits overhead.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters | Rel16 SGCS | SGCS gain Rel 16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.721 | 0.705 | 99 | 1.93M | 0.691 | 2.0% |
| OPPO | 600K | 0.728 | 0.703 | 138 |  | - | - |
| CAICT | 100K | 0.715 | 0.676 | 75 |  | - | - |
| Mixed | 300K | 0.710 | 0.686 | 73 |  | - | - |

RAN4 to consider the Table 2 above to collect SGCS performance for CSI encoder-decoder pairs trained on diverse datasets with two-bit quantization of CSI feedback.

Table 3: SGCS of Reference CNN model without Quantization.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters |
| --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.769 | 0.755 | 150 | 1.93M |
| OPPO | 600K | 0.775 | 0.752 | 150 |  |
| CAICT | 100K | 0.766 | 0.729 | 80 |  |
| Mixed | 300K | 0.761 | 0.718 | 143 |  |

RAN4 to consider the Table 3 above to collect SGCS performance for CSI encoder-decoder pairs trained on diverse datasets without quantization of CSI feedback.

To better understand the differences among datasets from various sources, we suggest that companies in RAN4 provide the cumulative distribution function (CDF) of the SGCS for both the Rel16 eTypeII baseline and the reference CNN models, compared to the ideal eigenvectors. The CDF plots of SGCS for Rel16 eTypeII baseline is shown in Figure 1.

Figure 1: CDF plots of SGCS for Rel16 eTypeII baseline

Finally, to gain a comprehensive understanding of the differences between datasets, cross-validation was performed using models trained on the NOKIA dataset, OPPO dataset, CAICT dataset, and a Mixed dataset comprising 300,000 samples, with 100,000 samples from each individual dataset. Each model was tested on both its respective dataset and the mixed dataset. The results indicate that models trained on individual datasets do not generalize well across different datasets. However, the model trained on the mixed dataset demonstrates relatively equal performance across various datasets. The overall performance on Mixed dataset is limited to the limited number of samples of this dataset, i.e., 300K instead of ~600K.

Table 4: Cross-validation of encoder-decoder par with different datasets.

| Train on |  | Test on |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NOKIA | OPPO | CAICT | Mixed |
| NOKIA | 630K | 0.705 | 0.645 | 0.595 | 0.648 |
| OPPO | 600K | 0.626 | 0.707 | 0.639 | 0.657 |
| CAICT | 100K | 0.497 | 0.586 | 0.677 | 0.587 |
| Mixed | 300K | 0.680 | 0.689 | 0.688 | 0.686 |

As shown in Table 4 above, the trained reference encoder-decoder pairs exhibited dataset dependency, indicating that a model trained on a single dataset may not perform well on other datasets, even under identical scenario settings. Training on a mixed dataset significantly enhances the model’s generalization ability, with larger mixed datasets yielding better performance.

Derivation of test decoder and encoder with Option 3

At the previous RAN4#112 meeting the following steps were agreed to proceed with the study of test decoder derivation following Option 3, i.e., when the test decoder is expected to be fully specified in RAN4 including the architecture/backbone/parameters and the values of the weights [3]:

Issue 4-1: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

As it can be seen, there is several steps in the plan that require further discussion.

Test Decoder(s) selection criteria

One of the main questions that was not treated so far is how RAN4 will select one or more decoders for further analysis? Performance and complexity were listed at RAN4#111 as potential evaluation criteria [3]:

RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

However, since the architecture/parameters of the encoder + decoder in the ongoing study are fixed, the complexity of all the models submitted by the companies is the same. Thus, complexity cannot be used as a criterion for now.

The performance of the models are compared based on the results reported by the companies. SGCS (Squared Generalized Cosine Similarity) of AI/ML-based and eTypeII CSI feedback, both relative to ideal CSI, was selected as metric.

Some variations in the performance are observed as reported in [4]. These variations can be explained by multiple factors such as:

- Difference in the dataset that is used for model training and verification/testing

- Difference in the implementation of the of channel evaluation and/or pre-processing because the variations are also observed for eTypeII CSI feedback that is supposed to be sued as a reference

- Difference in the training and/or hyperparameters of the encoder + decoder AIML models.

Even though an additional effort was taken last meeting to align the dataset and training parameters (addressing 1 and 3 above) [2], the selection of a single test decoder is still not straightforward. For example, the encoder demonstrating the best performance could also be an outlier, i.e., when eTypeII performance on the same dataset also demonstrates the performance higher than for the other companies. In this case, closeness to the average/mean performance value across all the companies could be a more reasonable selection criterion. Additionally, the relative gain in comparison to eTypeII CSI feedback could be an additional selection condition. Hence, additional conditions are needed to select the decoder(s) with the most representative performance across the companies.

If only a single decoder needs to be selected, then it will be reasonable to select the decoder with the medium performance gain in comparison to eTypeII decoder. However, for the study purpose, it would be beneficial to keep a few decoders, in this case three decoders can be selected for the further study:

- The decoder demonstrating best relative performance

- The decoder with relative performance closest to the mean

- The decoder demonstrating the worst relative performance

The reason for that is that even if a test decoder with the best performance is selected for standardization with Option 3, it will be used in the channel emulator implemented by the TE vendor, and this implementation will be different from the dataset that was used to train the encoder and decoder.

Additional criteria for the selection of decoder(s) from the set contributed by all the companies are needed. The decoder demonstrating best performance on one dataset may not be that efficient on another dataset, e.g., in the channel emulator implementation provided by the TE vendor.

RAN4 can consider for the further study three decoders: with highest, lowest and closest to mean relative performance gain in SGCS over eTypeII decoder.

The problem of training the test decoder by each of the companies individually is that it is harder to justify and explain the potential differences in performance. Moreover, quite probably, companies will use Type 1 training (i.e., joint training as a single entity) that could provide too optimistic performance results in comparison to Type 3 training (i.e., separate or sequential training) that is more practical when the encoder training is done after the test decoder is already trained.

Dataset aggregation can provide additional means of comparison and validation for the test decoder. The aggregated dataset can be used for objective comparison of encoder/decoder pair that was trained by different companies.

It is enough if such a verification dataset can contain only eigenvectors {V} that are coming to the input of the encoder. The same V from the dataset can be also used for the calculation of the SGCS after decoding.

The dataset can come either from a single company of from several companies.

Use of aggregated dataset {V} of channel eigenvectors can be used for much more objective performance comparison/validation of encoders/decoders contributed by the companies.

RAN4 to use an aggregated dataset of eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies if it is necessary to select a single test decoder (can be considered as Option 1a).

When an aggregated dataset is used for model training, a single decoder demonstrating best SGCS performance can be selected for the further study.

Next, in the Step (4) the encoders are trained based on selected decoder(s), and it will be necessary to compare the performance of these encoders with selected decoder(s). The question is again what will be criteria for the comparison of the performance? The same issues as discussed above arises: the encoder trained and tested on one dataset may not show the same level of performance on a different dataset, e.g., on the channel generated in the testing setup.

Similarly to the selection of the test decode, the performance of the encoder trained based on given decoder can demonstrate difference performance on different datasets/channel generators from different vendors.

Therefore, to avoid unrealistic evaluation of performance based on the same dataset type that was use for training, the only way is to check the performance on the other datasets and compare the variation of performance on those. If a large degradation is not observed that would mean that the encoder/decoder couple can be sued in different testing setups.

To conclude about the feasibility of Option 3, RAN4 needs to ensure that there is no significant performance variation in between “own” encoder – test decoder(s) by different companies in different datasets/aggregated dataset.

Additionally, since (relative) user throughput is usually considered in RAN4 in the requirements, some TPut-based metric should be studied in addition to SGCS-based.

RAN4 would need to check the span of the throughput-based test metric in between the companies to conclude about the success of Option3 feasibility study.

Partially specified decoder with Option 4

At the previous RAN4#112 meeting, the following sub-options were agreed to proceed with the study of test decoder specification Option 4, i.e., when the test decoder is expected to be partially specified:

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors - FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors - FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Let’s first consider Sub-option 4a, which is characterized by a specified dataset which is used to train the test decoders. This dataset must at least contain channel/eigenvector realizations V. In order to characterize the feedback interface (the decoder input) for interoperability, the dataset must also contain the latent messages c.  Therefore, the specified dataset consists of channel/eigenvector and latent message pairs {V, c}.  The channel/eigenvector realizations V can be generated by agreeing on the distribution of channels, through synthetic generation of channel samples, collected channel data, or some combination of these sources.  These channel/eigenvector realizations can be the same dataset used to train a reference encoder and decoder.  The latent messages c are obtained through a specification of the feedback interface.  This specification is most naturally obtained through a reference encoder which can be developed (along with a reference decoder) using a dataset containing the channel/eigenvector realizations V.  As described in the next section, a test decoder developed using this dataset must be verified to ensure a sufficient performance level in order that test encoders developed by various vendors are very likely to yield the same test results for a given DUT.  As is often done during model training, the specified dataset can be divided into training, validation, and testing portions where the performance specification for the verification procedure is set on the testing part of the dataset.

The specified dataset for Sub-option 4a should consist of channel/eigenvector and latent message pairs {V, c}. The latent messages c in the dataset for Sub-option 4a are readily obtained from a reference encoder. Test decoder performance specifications for verification for Sub-option 4a can be set based on a testing portion of the specified dataset.

In Sub-option 4b, the development of test decoders is based on the existence of a specified reference encoder.  Since the reference encoder implicitly specified the feedback interface, a test equipment vendor can create its own dataset of {V, c} pairs where the latent messages are obtained using the reference encoder.  As with Sub-option 4a, test decoder verification can be performed by setting a performance specification based on a testing portion of the created dataset.  However, to ensure consistency of performance between test equipment vendors, it would be better to set performance specifications for verification based on a common testing dataset.  This dataset can be smaller than the dataset used to train the test decoder but would need to be standardized to make it common between equipment vectors.

Datasets for training the test decoder in Sub-option 4b can be created by the test equipment vendor using the reference encoder. A standardized testing dataset can be used in Sub-option 4b as the basis for performance verification of the test decoders.

Our analyses of Sub-options 4a and 4b above make it possible to recognize similarities and differences between the two sub-options which play a role in determining the feasibility of these options.  In both cases, we see a role for a reference encoder and a specified dataset.  One difference in the two options is where the training dataset is created.  In Sub-option 4a, the dataset is specified and common for all vendors, but with Sub-option 4b, the training dataset can be created by the equipment vendor.  In both cases, a specified dataset can be used to set the performance specifications for decoder verification to ensure performance consistency among equipment vendors.  In Sub-option 4a, the verification dataset is a portion of the larger specified dataset whereas it is the only specified dataset in Sub-option 4b and is significantly smaller than the Sub-option 4a dataset.

Sub-options 4a and 4b differ in how the training dataset is obtained. For consistency of test decoder performance between test equipment vendors, a common specified verification dataset is desirable.

In Sub-option 4c, no reference encoder or dataset is specified, but constraints are applied “on the decoder output ordering”.  It is not clear to us what is meant by the term “decoder output ordering” and how constraints on it would allow the creation of an interoperable test decoder.  It is our understanding that this sub-option is related to the proposal in [5]. However, Proposal 1 in that document refers to “the reference enc/dec pair”, apparently contradicting the indication that no reference encoder is needed.  The proposal reads “For any latent message z∈Z, where Z is the latent message space, denote the decoder output of the reference enc/dec pair as xref(z) and the decoder output of the test decoder (from another pair) as xtest(z), the test decoders has to satisfy: xref(z)-xtest(z)=minx∈Xtestxref(z)-x, where Xtest is the space of the test decoder output.”  Assuming that there is a reference encoder and decoder, there is apparently a straightforward solution to this constraint where xtestz=xref(z).  That is, the reference decoder is the solution.  For further analysis of this sub-option, we request additional clarification by its proponents.

For further analysis of Sub-option 4c, additional clarification of constraints “on the decoder output ordering” is requested of its proponents.

In order to investigate the feasibility of Sub-options 4a and 4b, we propose to follow the general outline of the procedure being used for Option 3, making modifications to incorporate the characteristics of each sub-option.  For Sub-option 4a, consider the following procedure:

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Generate dataset {V, c} – standardized, common dataset.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training and verifying them using the standardized {V, c}.

- Companies train their own encoder (replicating the UE vendor design process) using an architecture of their choice.

  - For training their encoders, companies could choose between two alternatives:
Alternative 1: Use the standardized dataset
Alternative 2: Use their own dataset

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

Steps 1 and 2 of this procedure are designed to create a reference encoder/decoder pair which will define the CSI feedback interface – i.e., the encoder output/decoder input and how it maps to channel information.  The model architecture and hyperparameter values agreed during the Option 3 feasibility study could be reused here with an agreed dataset {V}.  The dataset created in Step 3 can be generated using the original dataset {V} from Step 1 and the reference encoder.  As discussed above for Sub-option 4a, this dataset can be split into training, validation, and testing parts, where the testing part is used for test decoder verification in Step 4.  The dataset {V, c} represents the standardized, common dataset.  In Step 4, companies train test decoders using the standardized dataset and a decoder architecture of their choosing.  This step mimics the process used by test equipment vendors to create their test decoders.  It includes model verification using the testing portion of the standardized dataset to show that their decoder meets a prescribed level of performance.  The purpose of Step 5 is to mimic the design process used by UE vendors to create their encoders used for deployment of their devices.  Companies will create their own encoder using an architecture of their choice.  Training can be performed using either the standardized dataset from Step 3 or a dataset of their own created by leveraging the reference encoder (i.e., the encoder input set {V} can be different than the one in the standardized dataset). Since Steps 4 and 5 are meant to mimic the work of two distinct entities (test equipment vendors and UE vendors), ideally there should be some variation in the decoder and encoder architectures.  Finally, the process of evaluating the UE encoders is replicated in Step 6 by testing each Step 5 UE encoder using each of the Step 4 test decoders. Ideally, the performance of each UE encoder should be identical with every test decoder, but realistically the performance should be close enough that the same test result would be achieved with any of the test decoders.

There are details which would need to be discussed and agreed for several of these steps, such as the method used to generate the dataset in Step 1 and its composition.  In addition, the details of the creation of the reference encoder/decoder used in this feasibility study must be discussed, but aspects of the Option 3 study can be used as a starting point, as noted above.  The details of the verification step, such as the performance threshold may also need to be determined.  Finally, the logistics for carrying out Step 6 must be discussed.

RAN4 to adopt the following procedure for the feasibility study steps of Sub-option 4a (specified dataset):

- Generate a dataset of channel/eigenvector realizations {V}

- Create reference encoder/decoder.

- Generate dataset {V, c} – standardized, common dataset.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training and verifying them using the standardized {V, c}.

- Companies train their own encoder (replicating the UE vendor design process) using an architecture of their choice.

  - For training their encoders, companies could choose between two alternatives:
Alternative 1: Use the standardized dataset
Alternative 2: Use their own dataset

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

Sub-option 4b is based on a reference encoder, which changes the constraints on the test decoder training dataset, as discussed above. Consider the following procedure for investigating the feasibility of Sub-option 4b:

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Create a verification test dataset (common).

- Each company (mimicking a test equipment vendor) generates their own training dataset {V, c} using the common reference encoder to create the encoder output c from the encoder input V.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training them using the dataset {V, c} from the previous step and verifying them using the verification dataset from Step 3.

- Companies train their own encoder (mimicking a UE vendor) using an architecture of their choice, training the encoder using a dataset different from the dataset in the previous step.

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

Steps 1 and 2 are identical to those in our proposal for studying the feasibility of Sub-option 4a.  However, since the dataset is not standardized in this sub-option, it does not need to be common if the reference encoder/decoder creation process does not require it.  In Step 3, a common verification dataset is created.  The dataset is common to provide a known basis for the performance tests of all test decoders.  Steps 4 and 5 mimic the test decoder design process used by a test equipment vendor.  Since there is no standardized dataset for training, the vendor utilizes the reference encoder along with a set of channel/eigenvector realizations to generate a training dataset in Step 4.  In Step 5, the vendor designs and trains its test encoder and verifies its performance using the verification dataset from Step 3.  The vendor is free to use its own model architecture.  Step 6 mimics the process where a UE vendor designs its own encoder.  Since the training dataset is not standardized, it must be created by the UE vendor.  Companies training their encoders in this step are encouraged to use a different dataset than that used in Step 5 to model the separation between test equipment and UE vendors.  Finally, Step 7 is identical to the last step in the Sup-option 4a procedure where the UE encoders are evaluated using all of the test decoders.

RAN4 to adopt the following procedure for the feasibility study steps of Sub-option 4b (reference encoder):

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Create a verification test dataset (common).

- Each company (mimicking a test equipment vendor) generates their own training dataset {V, c} using the common reference encoder to create the encoder output c from the encoder input V.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training them using the dataset {V, c} from the previous step and verifying them using the verification dataset from Step 3.

- Companies train their own encoder (mimicking a UE vendor) using an architecture of their choice, training the encoder using a dataset different from the dataset in the previous step.

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

Verification of test decoder

In the previous meeting (RAN4#111) the definition of a reference encoder and a reference decoder was generally agreed. The agreements are captured as below.

Issue 4-1: Reference encoder/decoder Agreement: Reference encoder: - The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. Reference decoder: - The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder. In option 3 the Reference decoder is the test decoder. For option 4, there might be a need to have reference encoder and/or reference decoder

However, based on the option (Option 3 / Option 4) that will be the way forward for test decoder derivation, it will become necessary to verify the test decoder. This is to ensure that the test decoder is indeed interoperable and satisfies the minimum performance criteria that is specified for it for an encoder.

The following options to verify the test decoder can be considered:

- UE vendor implements the reference encoder and test decoder verification is performed with an actual UE, i.e., UE vendor’s implemented reference encoder can be used in the test setup for test decoder’s verification.

  - During the test, the UE should possess reference encoder potential together with any other decoder the UE vendor wants to use in the test.

  - The testing procedure will include, firstly, verification of the decoder with the reference encoder, and then the actual test with an encoder implemented by the UE vendor for the real network.

- Test equipment vendors can implement the reference encoder at their end, and may test only the AIML functionality, without necessarily having complete UE side operations.

  - It could be a kind of SW verification of the test decoder based on the reference encoder without using any real/test UE.

- The stakeholders will mutually agree on the use of a UE simulator/emulator which can be a 3rd party device or framework just for testing the encoder with the test decoder which is not necessarily a full-fledged UE supporting all the functionalities of a UE.

For all above listed options, the minimum performance target for test decoder’s verification should be agreed during the specification phase considering various results across different vendors.

For test decoder’s verification fully specified reference encoder should be used.

For test decoder’s verification, reference encoder may be implemented by UE vendors, TE vendors or a third-party implementation of UE sided operations including reference encoder.

For test decoder’s verification, the minimum performance target should be agreed and specified considering various results from different stakeholders.

Conclusion

This constitution is devoted to the discussion of the ways how to train and select encoder and decoder to be used in the testing of AIML-based CSI compression. Additionally, we present the results of training and cross-validation of trained encoder-decoder pair on our own dataset and on the datasets by two other companies.

The following Observations and Proposals were made:

On Results of decoder/encoder training:

Table 2: Trained AI/ML encoder-decoder pair SGCS performance with 2-bit quantization and 64-bits overhead.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters | Rel16 SGCS | SGCS gain Rel 16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.721 | 0.705 | 99 | 1.93M | 0.691 | 2.0% |
| OPPO | 600K | 0.728 | 0.703 | 138 |  | - | - |
| CAICT | 100K | 0.715 | 0.676 | 75 |  | - | - |
| Mixed | 300K | 0.710 | 0.686 | 73 |  | - | - |

- RAN4 to consider the Table 2 above to collect SGCS performance for CSI encoder-decoder pairs trained on diverse datasets with two-bit quantization of CSI feedback.

Table 3: SGCS of Reference CNN model without Quantization.

| Dataset | Samples | Training SGCS | Testing SGCS | Epoch | Model Parameters |
| --- | --- | --- | --- | --- | --- |
| NOKIA | 630K | 0.769 | 0.755 | 150 | 1.93M |
| OPPO | 600K | 0.775 | 0.752 | 150 |  |
| CAICT | 100K | 0.766 | 0.729 | 80 |  |
| Mixed | 300K | 0.761 | 0.718 | 143 |  |

RAN4 to consider the Table 3 above to collect SGCS performance for CSI encoder-decoder pairs trained on diverse datasets without quantization of CSI feedback.

Table 4: Cross-validation of encoder-decoder par with different datasets.

| Train on |  | Test on |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | NOKIA | OPPO | CAICT | Mixed |
| NOKIA | 630K | 0.705 | 0.645 | 0.595 | 0.648 |
| OPPO | 600K | 0.626 | 0.707 | 0.639 | 0.657 |
| CAICT | 100K | 0.497 | 0.586 | 0.677 | 0.587 |
| Mixed | 300K | 0.680 | 0.689 | 0.688 | 0.686 |

- As shown in Table 4 above, the trained reference encoder-decoder pairs exhibited dataset dependency, indicating that a model trained on a single dataset may not perform well on other datasets, even under identical scenario settings. Training on a mixed dataset significantly enhances the model’s generalization ability, with larger mixed datasets yielding better performance.

On derivation of test decoder and encoder with Option 3:

Additional criteria for the selection of decoder(s) from the set contributed by all the companies are needed. The decoder demonstrating best performance on one dataset may not be that efficient on another dataset, e.g., in the channel emulator implementation provided by the TE vendor.

RAN4 can consider for the further study three decoders: with highest, lowest and closest to mean relative performance gain in SGCS over eTypeII decoder.

Use of aggregated dataset {V} of channel eigenvectors can be used for much more objective performance comparison/validation of encoders/decoders contributed by the companies.

RAN4 to use an aggregated dataset of eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies if it is necessary to select a single test decoder (can be considered as Option 1a).

When an aggregated dataset is used for model training, a single decoder demonstrating best SGCS performance can be selected for the further study.

Similarly to the selection of the test decode, the performance of the encoder trained based on given decoder can demonstrate difference performance on different datasets/channel generators from different vendors.

To conclude about the feasibility of Option 3, RAN4 needs to ensure that there is no significant performance variation in between “own” encoder – test decoder(s) by different companies in different datasets/aggregated dataset.

RAN4 would need to check the span of the throughput-based test metric in between the companies to conclude about the success of Option3 feasibility study.

On Partially specified decoder with Option 4:

The specified dataset for Sub-option 4a should consist of channel/eigenvector and latent message pairs {V, c}. The latent messages c in the dataset for Sub-option 4a are readily obtained from a reference encoder. Test decoder performance specifications for verification for Sub-option 4a can be set based on a testing portion of the specified dataset.

Datasets for training the test decoder in Sub-option 4b can be created by the test equipment vendor using the reference encoder. A standardized testing dataset can be used in Sub-option 4b as the basis for performance verification of the test decoders.

Sub-options 4a and 4b differ in how the training dataset is obtained. For consistency of test decoder performance between test equipment vendors, a common specified verification dataset is desirable.

For further analysis of Sub-option 4c, additional clarification of constraints “on the decoder output ordering” is requested of its proponents.

RAN4 to adopt the following procedure for the feasibility study steps of Sub-option 4a (specified dataset):

- Generate a dataset of channel/eigenvector realizations {V}

- Create reference encoder/decoder.

- Generate dataset {V, c} – standardized, common dataset.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training and verifying them using the standardized {V, c}.

- Companies train their own encoder (replicating the UE vendor design process) using an architecture of their choice.

  - For training their encoders, companies could choose between two alternatives:
Alternative 1: Use the standardized dataset
Alternative 2: Use their own dataset

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

RAN4 to adopt the following procedure for the feasibility study steps of Sub-option 4b (reference encoder):

- Generate a dataset of channel/eigenvector realizations {V}.

- Create reference encoder/decoder.

- Create a verification test dataset (common).

- Each company (mimicking a test equipment vendor) generates their own training dataset {V, c} using the common reference encoder to create the encoder output c from the encoder input V.

- Companies design their own test decoders (replicating the design process of TE vendors) using their architecture of choice, training them using the dataset {V, c} from the previous step and verifying them using the verification dataset from Step 3.

- Companies train their own encoder (mimicking a UE vendor) using an architecture of their choice, training the encoder using a dataset different from the dataset in the previous step.

- Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders).

On Verification of test decoder:

For test decoder’s verification fully specified reference encoder should be used.

For test decoder’s verification, reference encoder may be implemented by UE vendors, TE vendors or a third-party implementation of UE sided operations including reference encoder.

For test decoder’s verification, the minimum performance target should be agreed and specified considering various results from different stakeholders.

- References

- R4-2414323, WF on AI/ML, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2414447, Alignment of model training and data set parameters for AI/ML-based CSI compression, Nokia, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2401570, WF on AI/ML, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.

- R4-2411786, Summary of e-mail discussion on AI/ML Model for CSI testing Option 3 feasibility study, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2412130, On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case, National Taiwan University, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.