3GPP TSG-RAN WG4 Meeting #112bis	R4-2416015

Hefei, China, 14-18 October 2024

Source:	Nokia

Title:	Discussion on Conditional LTM RRM requirements

Agenda item:	6.24.2.2

Document for:	Discussion

Introduction

In RP-242356, RAN plenary agreed to update the NR_Mob_Ph4 work item description to include specification of conditional LTM starting from the October RAN meetings:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

The work on conditional LTM (CLTM from now on) will start in all working groups in the October meeting. In this contribution we discuss the RAN4 aspects of CLTM up to the point which is reasonable without any existing RAN1/RAN2 agreements.

Discussion on conditional LTM requirements

As the work for CLTM is only to start in this meeting in RAN1/RAN2, there are for now no agreements about the design on top of the plenary scope. However, it can be foreseen that conditional LTM will be defined by using the principles of Rel-18 LTM and conditional L3 handover (CHO) as the baseline. With this assumption, the following high level principle is expected:

UE is configured with one or more LTM candidate cells, and for at least one of those candidate cells, the network configures at least one condition for conditional LTM cell switch. When such condition becomes fulfilled based on UE measurements, the UE will execute a conditional cell switch to the candidate cell/beam in question without a cell switch command.

The definition and configuration of the conditions to trigger CLTM is up to RAN1 and RAN2, and hence RAN4 should wait for RAN1/RAN2 agreements to evaluate whether and what kind of impact the defined conditions/events have on RAN4 RRM requirements.

The conditions for triggering CLTM are up to RAN1/RAN2 to define. RAN4 to take into account such agreements once available.

In Rel-18 LTM, the specification supports early DL and UL sync as optional features. For Rel-19 CLTM, what we would see as the expectation is that at least one of early DL sync or UL sync would need to be considered mandatory to differentiate CLTM from CHO. Whether these procedures are to follow Rel-18 principle or whether any enhancements are to be specified for CLTM purposes, is to be discussed by RAN1/RAN2. However, the goal of Rel-19 work should be to define a feature that clearly introduces gains in terms of cell switch interruption reduction compared to legacy CHO.

The design of early DL/UL sync in CLTM is up to RAN1/RAN2 discussion and agreements. It is foreseen that at least one of early DL and/or UL sync is to be defined as mandatory for CLTM to ensure optimization of CLTM interruption compared to CHO.

Whether any updates are needed to the RAN4 Rel-18 early TCI state activation or PDCCH ordered RACH delay requirements depend on whether the Rel-18 procedures are reused for CLTM or whether RAN1/2 agree on any enhancements to the DL and UL synchronization procedures.

FFS whether updates are needed to the Rel-18 early TCI state activation and PDCCH ordered RACH delay requirements, or whether new requirements due to CLTM specific DL/UL sync procedures are needed.

One obvious RRM impact from CLTM is that RAN4 needs to define conditional cell switch delay requirements. RAN plenary has agreed to limit the CLTM work to intra-CU case without DC configuration. Hence, cell switch delay requirements are only needed for conditional PCell switch in intra-CU scenario. Furthermore, cell switch delay requirements should also cover the case of subsequent LTM, as this scenario is supported by the WID. Whether there is any need to differentiate between initial and subsequent conditional cell switch delay is to be studied.

RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario.

Cell switch delay requirements shall cover also subsequent conditional PCell switch. FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch.

When looking at the existing cell switch delay requirements for Rel-18 LTM and CHO, the delays are defined as:

DLTM = Tcmd + TLTM-interrupt, where

TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms

DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tinterrupt, where

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Most of the components of these delays are matching to each other to great extent. The components that are specific to CHO are:

TEvent_DU which is the uncertainty time from when the UE has decoded the CHO command until the time the condition for CHO becomes met.

Tmeasure is the time from the end of TEvent_DU that is allowed for the UE to measure and realize the condition is met before the UE is required to start CHO execution.

These types of components need to be included in the CLTM delay to cover the time until the condition for CLTM becomes met and the time it takes for the UE to measure and realize the condition is met. The definition of TEvent_DU for CHO can likely be reused for CLTM, but the definition of Tmeasure depends at least on the type of measurements, L1 or L3, that are used for CLTM condition evaluation.

Other components that are at least expected to be included in the delay are TLTM-processing and TLTM-IU, where the definition of TLTM-IU depends on the support of RACH-less and RACH-based CLTM.

The need of TLTM-RRC-processing, Tfirst-RS and TRS-proc depends on how and at which point, after UE receiving LTM RRC configuration, early DL synchronization and ASN.1 decoding and validity check are expected to happen: before the condition evaluation begins, before the condition becomes met, or after the condition becomes met. In the worst case, these parameters would be included in the cell switch delay and interruption. However, the goal should be that these steps are to be completed before the execution of CLTM cell switch.

The starting point for the CLTM delay is: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. FFS the exact definition of each component.

The need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design.

Related to the need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any other components, one aspect that is to be discussed is the starting point of the CLTM cell switch delay. The conditional cell switch is triggered at the time the condition is met, but as this is not visible to the network, in the existing specification, the delay requirement for CHO has been written to start from the moment the UE receives the RRC configuration. For CLTM, the starting point of the delay may be the time UE receives LTM RRC configuration as in CHO, or something else. This depends on RAN1/RAN2 discussions about when the CLTM condition evaluation is to be considered active i.e. when the UE is expected to evaluate the conditions and execute CLTM if a condition becomes met. Before there are such agreements available, RAN4 is to leave the starting point and any potential additional delay component related to that open.

The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2.

The ending point of conditional cell switch can be expected to follow Rel-18 LTM cell switch delay requirement i.e. it is the time the UE sends either RACH preamble (RACH-based cell switch) or first UL transmission (RACH-less cell switch) on the target cell. Whether requirements for both RACH-less and RACH-based CLTM need to be supported depends on whether RACH-less CLTM through early TA acquisition is supported and whether it is to be defined as mandatory or optional.

The ending point of CLTM cell switch is the first UL transmission (RACH-based or RACH-less) similar to Rel-18 LTM. FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

Another point to be discussed is the interruption part of the delay. RAN4 needs to define when the CLTM interruption is to start i.e. which of the CLTM cell switch delay components are considered as part of the interruption (TCLTM_interrupt) and which are outside the interruption. As discussed earlier in this paper, RAN4 should aim to define the interruption to be shorter than CHO interruption, to ensure that CLTM has benefits over CHO. The details on how to achieve this depends at least partially on RAN1/2 design on whether the preparation for conditional cell switch - early DL and UL sync and RRC processing – are to happen before or after the condition becomes met.

RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption.

Conclusion

The following observations and proposals were made in this paper:

Observation 1: The conditions for triggering CLTM are up to RAN1/RAN2 to define. RAN4 to take into account such agreements once available.

Observation 2: The design of early DL/UL sync in CLTM is up to RAN1/RAN2 discussion and agreements. It is foreseen that at least one of early DL and/or UL sync is to be defined as mandatory for CLTM to ensure optimization of CLTM interruption compared to CHO.

Proposal 1: FFS whether updates are needed to the Rel-18 early TCI state activation and PDCCH ordered RACH delay requirements, or whether new requirements due to CLTM specific DL/UL sync procedures are needed.

Proposal 2: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario.

Proposal 3: Cell switch delay requirements shall cover also subsequent conditional PCell switch. FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch.

Proposal 4: The starting point for the CLTM delay is: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. FFS the exact definition of each component.

Proposal 5: The need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design.

Proposal 6: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2.

Proposal 7: The ending point of CLTM cell switch is the first UL transmission (RACH-based or RACH-less) similar to Rel-18 LTM. FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

Proposal 8: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption.