3GPP TSG-RAN WG4 Meeting #112bis	R4-2416376

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Consideration of test time reduction for XR device

Source:	OPPO

Agenda item:	6.12.2.1

Document for:	Approval

- Introduction

In the last RAN4 #112 meeting in Maastricht, there was a lot progress on the OTA test methodology for XR devices, especially on XR device type. The way forward was agreed in RAN4 group [1]. The test time reduction was also discussed with no consensus. There options were proposed for further discussion.

This contribution further discusses and provides proposals on the consideration on OTA test time reduction for XR devices.

- Discussion

As agreed in the last meeting, there two types of XR devices prioritized in this work item, i.e. glasses and head mounted display.

Besides, it is down-selected that only XR devices with 5G link to gNB in networks are specified as DUT in this WI, i.e. XR5G-V4, XR5G-A3 and XR5G-A4 defined in TR 26.928.

XR Device Types (Table 4.8-1 of TR26.928)

To provide portability, the devices covered by XR5G-V4, XR5G-A3 and XR5G-A4 typically use built-in batteries. And the battery usage life is a big challenge due to limited battery capacity. Generally, the XR devices with built-in batteries can provide 2-3 hours usage duration under normal working conditions, which can not support a full test with legacy smart phone OTA test methodology. Therefore, test time reduction solutions are not only better-to-have but also must-have demand for XR devices.

Observation: Test time reduction solutions are not only better-to-have but also must-have demand for XR devices, considering the battery usage lift of XR devices.

Regarding Option 1 of the WF of last meeting, charging the battery in the middle of the measurement is anyway achievable approach with potential larger measurement uncertainty, which can be the back-up solution if other solutions are confirmed not valid.

Proposal 1: Option 1, i.e. charge the battery in the middle of the measurement, can be the back-up solution if other solutions are confirmed not valid.

Regarding Option 2 of the WF, technically, when the power amplifier working at different output power level, the conducted complex impedance will change on the smith chart, and further impact the antenna’s radiation efficiency. Therefore, more validations are needed to check whether the impact is within an acceptable level, i.e. only a small uncertainty is introduced.

Proposal 2: Option 2, i.e. test TRP at X dB below the max power, needs more validations to check whether different output power level will affect the total antenna efficiency and TRP.

Option 3, to my understanding, is a variant of Option 1. All of 266 test points are separated into 4 subgroups, and are tested one by one.

Besides the three options in the WF, coarse measurement grid can also be a candidate solution for further study. In Rel-17 and Rel-18 TRP TRS work items, the coarse measurement grid solution is studied. But it is based on the real/typical antenna patterns of smart phones, not XR devices. It is recommended to perform the coarse measurement grid study based on the antenna patterns of practical XR devices.

Proposal 3: It is recommended to perform the coarse measurement grid study based on the antenna patterns of practical XR devices.

- Conclusions

This contribution further discusses the consideration on OTA test time reduction for XR devices. And provide the following proposal.

Observation: Test time reduction solutions are not only better-to-have but also must-have demand for XR devices, considering the battery usage lift of XR devices.

Proposal 1: Option 1, i.e. charge the battery in the middle of the measurement, can be the back-up solution if other solutions are confirmed not valid.

Proposal 2: Option 2, i.e. test TRP at X dB below the max power, needs more validations to check whether different output power level will affect the total antenna efficiency and TRP.

Proposal 3: It is recommended to perform the coarse measurement grid study based on the antenna patterns of practical XR devices.

- References

[1] R4-2413535 WF for [112][333] TRP_TRS_MIMO_OTA