3GPP TSG-RAN WG4 Meeting #112bis                                                         R4-2416446

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Nokia

Title:	Simulation Results for RedCap Positioning without Frequency Hopping

Agenda item:	4.2.2.4

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning.

NR RedCap positioning has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2]. In RAN4#107 [3], the link simulation assumptions for RedCap positioning without FH are agreed

In this contribution we further provide the simulation results for RedCap positioning without FH. It is worth noting that the results are generated without RF/BB impairments. Also, note that the results are provided for 1 Rx antenna at the UE.

Simulation Assumptions

Simulation assumptions for RedCap positioning without FH captured in [3] are considered. The DL PRS configurations as listed in Table 1 were used in the simulation study.

| Carrier frequency | SCS (kHz) | PRS BW in RB | sampling rate (Tc) | Num Symbols | Comb Size | Repetition Factor (Rep) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 GHz | 15 | 24 | 256 | 4 | 4 | 6 |
| 2 GHz | 15 | 52 | 128 | 4 | 4 | 6 |
| 2 GHz | 15 | 104 | 64 | 4 | 4 | 6 |
| 4 GHz | 30 | 24 | 128 | 4 | 4 | 6 |
| 4 GHz | 30 | 48 | 64 | 4 | 4 | 6 |
| 4 GHz | 60 | 24 | 48 | 4 | 4 | 6 |

Table 1: DL PRS configurations used for RSTD and UE Rx-Tx studies for SCS = 15 kHz, 30 kHz, 60 kHz, and 120 kHz.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the DL RSTD error in Tc and for 90-percentile CDF of the UE Rx-Tx error in Tc for given Es/Iot for three cells [4].

AWGN Channel

### FR1 UE Rx-Tx Results, 4 samples

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF and 4 samples, for Es/IoT of three cells (-3, -13, -13) dB are listed in Table 2.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 24 | 256 | 64 | 64 | 64 |
| 2 GHz / 15 kHz | 52 | 128 | 32 | 32 | 32 |
| 2 GHz / 15 kHz | 104 | 64 | 16 | 16 | 16 |
| 4 GHz / 30 kHz | 24 | 128 | 32 | 32 | 32 |
| 4 GHz / 30 kHz | 48 | 64 | 16 | 16 | 16 |
| 4 GHz / 60 kHz | 24 | 48 | 16 | 16 | TBD |

Table 2: UE Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

### FR1 UE Rx-Tx Results, reduced number of samples

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF and reduced number of samples, for Es/IoT of three cells (0, -6, -6) dB are listed in Table 3.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 52 | 128 | 64 | 64 | 64 |
| 2 GHz / 15 kHz | 104 | 64 | 32 | 32 | 32 |
| 4 GHz / 30 kHz | 48 | 64 | 32 | 32 | 32 |

Table 3: UE Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

### FR1 RSTD Results, 4 samples

The results of DL RSTD errors in Tc for AWGN channel, derived from the 5%-ile, 95%-ile of the CDF, and 4 samples for Es/IoT of three cells (-6, -13, -13) are listed in Table 4.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 2 GHz / 15 kHz | 24 | 256 | -128 | 128 | TBD | 256 |
| 2 GHz / 15 kHz | 52 | 128 | -64 | 64 | -64 | 64 |
| 2 GHz / 15 kHz | 104 | 64 | -32 | 32 | -32 | 32 |
| 4 GHz / 30 kHz | 24 | 128 | -64 | 96 | TBD | 64 |
| 4 GHz / 30 kHz | 48 | 64 | -32 | 32 | -32 | 32 |
| 4 GHz / 60 kHz | 24 | 48 | -32 | 48 | TBD | 16 |

Table 4: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

### FR1 RSTD Results, reduced number of samples

The results of DL RSTD errors in Tc for AWGN channel, derived from the 5%-ile, 95%-ile of the CDF, and reduced number of samples for Es/IoT of three cells (-3, -6, -6) are listed in Table 5.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 2 GHz / 15 kHz | 52 | 128 | -128 | 128 | -128 | 128 |
| 2 GHz / 15 kHz | 104 | 64 | -64 | 64 | -64 | 64 |
| 4 GHz / 30 kHz | 48 | 64 | -64 | 64 | -64 | 64 |

Table 5: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

TDL-A Channel

### FR1 RSTD results, 4 samples

The results of DL RSTD errors in Tc for TDL-A channel, derived from the 5%-ile, 95%-ile of the CDF, and 4 samples for Es/IoT of three cells (-6, -10, -10) are listed in Table 6.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 2 GHz / 15 kHz | 24 | 256 | -256 | 256 | TBD | 64 |
| 2 GHz / 15 kHz | 52 | 128 | -96 | 128 | TBD | 128 |
| 2 GHz / 15 kHz | 104 | 64 | TBD | 112 | -127 | 80 |
| 4 GHz / 30 kHz | 24 | 128 | -96 | 128 | TBD | 64 |
| 4 GHz / 30 kHz | 48 | 64 | -32 | 112 | TBD | TBD |
| 4 GHz / 60 kHz | 24 | 48 | -16 | 96 | TBD | 16 |

Table 6: RSTD error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -10, -10) dB.

Discussion

From the RSTD and UE Rx-Tx results reported above for the AWGN and TDL-A channels using 1 UE receive antennas without frequency hops and without RF/BB impairments, we observed:

When AWGN, TDL-C and TDL-A channels are used, for a given bandwidth, the measurement accuracy improves with the increasing number of measurement samples.

Increasing the UE total measurement bandwidth improves the measurement accuracy when AWGN or TDL-A channels are used.

The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models.

Using 4 measurement samples should be the baseline to define the performance requirements.

Using 1 UE Rx antenna for FR1 and 2 UE Rx antennas for FR2 should be the baseline to define the performance requirements.

Conclusion

In this contribution we have provided first simulation results for DL RSTD and UE Rx-Tx error without RF/BB impairments based on simulation assumptions in [4].

Following observations and proposals are made.

- When AWGN, TDL-C and TDL-A channels are used, for a given bandwidth, the measurement accuracy improves with the increasing number of measurement samples.

- Increasing the UE total measurement bandwidth improves the measurement accuracy when AWGN or TDL-A channels are used.

- The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models.

- Using 4 measurement samples should be the baseline to define the performance requirements.

Using 1 UE Rx antenna for FR1 and 2 UE Rx antennas for FR2 should be the baseline to define the performance requirements.

- References

| [1] | RP-232670, Revised WID on Expanded and Improved NR positioning, 3GPP TSG RAN#101 |
| --- | --- |
| [2] | TR 38.859, "Study on Expanded and Improved NR Positioning (Release 18)," V18.0.0 |
| [3] | R4-2310075, "Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH", Ericsson |