3GPP TSG-RAN WG4 #113	R4-2417514

Orlando, US, November 18-22, 2024

Agenda item:	7.22.2.2		Adaptation of Common Signal/Channel Transmission

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan was agreed in [1].

Several agreements have been made for SSB adaptation according to [2] and additional open issues have been identified in [3]. In addition, two LSs have been sent to RAN2 and RAN1 asking for further clarification for SSB adaptation [4], [5]. In this paper we provide our views for the issues identified in the way forward as defined in [2] in RAN4 #112bis.

# Discussion

The following agreement has been made in RAN4 #112bis regarding the clarification of the scenario for SSB adaptation:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion:

    - CONNECTED mode, and

    - SSB adaptation for SCell, and

    - SSB adaptation for NCD-SSB, and

    - SSB periodicity adaptation in time-domain

Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

In parallel in RAN1 #118bis, RAN1 could not conclude on the support of SSB adaptation in idle mode. Hence, RAN4 does not need to define requirements for SSB adaptation in idle mode.

Proposal 1: RAN4 does not define requirements for SSB adaptation in idle mode.

A list of potentially impacted requirements has been agreed in the way forward in RAN4 #112bis. In parallel a LS has been sent to RAN2 asking whether SMTC configuration is impacted by SSD adaptation [4]. If this is the case, the requirements for intra- and inter-frequency L3 measurement may be impacted. Hence, requirements that depend on the SMTC window should not be discussed as long as no answer from RAN2 has been received.

Proposal 2: RAN4 should not discuss requirements that depend on the SMTC window as long as no answer from RAN2 has been received.

Issue 1-2-1: Requirements need updating

Agreement:

  - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption:

    - L1-RSRP for serving cell

    - L1-SINR

    - BFD

    - CBD

    - SCell activation

    - TCI state switching

    - NR intra-frequency L3 measurement

    - NR inter-frequency L3 measurement

Note: Above list could be further updated based on RAN4 discussion.

Transition requirements are already defined for several cases in 38.133, e.g., for beam failure detection and L3 measurements when the configuration changes. The underlying principles should also be applied when SSB periodicity is adapted.

Observation 1: Transition requirements are already defined for e.g., beam failure detection and L3 measurements when the configuration changes. The underlying principles should also be applied when SSB periodicity is adapted.

Figure 2-1 shows an example when the measurement period changes. A measurement period is defined based on the periodicity of the SSB, i.e., TSSB, i.e., the measurement period gets longer with an increase in TSSB. Therefore, during the transition due to a changed in TSSB, only the longer measurement period ensures that enough samples are contained in the transition phase. The shorter measurement period may not contain enough samples to achieve the sufficient measurement accuracy.

Figure 2-1: Illustration of Transition in Measurement Period

Observation 2: During a transition in SSB periodicity TSSB, the shorter measurement period may not contain enough samples to achieve the sufficient measurement accuracy.

Hence, we propose that for measurements the requirements during transition should be defined base do the longer measurement period.

Proposal 3: For measurements that are defined based on a measurement period, requirements during the transition phase in case of SSB adaptation should be based on the longer measurement period, i.e., based on the longer of both SSB periodicities, i.e., max(TSSB1, TSSB2).

Situation is different for beam failure detection and for RLM for PCell. Here transition requirements are defined based on an evaluation period that is no less than the minimum of the evaluation periods corresponding to the first configuration and the second configuration, see TS 38.133, section 8.5.10 for example. This makes sense that in such a scenario, the more stringent requirements apply. Also here, the underlying principle can be reused to ensure performance quality..

Observation 3: In case of beam failure detection, requirements during the transition phase are based on the more stringent evaluation period, i.e., the shorter one of both evaluation periods.

The same can be applied also in case of SSB adaption.

Proposal 4: In case of beam failure detection, requirements for the transition phase should be based on min(TSSB1, TSSB2).

Similar requirements based min(TSSB1, TSSB2) on are expected to be applicable for candidate beam prediction.

Proposal 5: RAN4 to investigate whether for CBD requirements in the transition phase can be defined based on min(TSSB1, TSSB2).

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 does not define requirements for SSB adaptation in idle mode.

Proposal 2: RAN4 should not discuss requirements that depend on the SMTC window as long as no answer from RAN2 has been received.

Observation 1: Transition requirements are already defined for e.g., beam failure detection and L3 measurements when the configuration changes. The underlying principles should also be applied when SSB periodicity is adapted.

Observation 2: During a transition in SSB periodicity TSSB, the shorter measurement period may not contain enough samples to achieve the sufficient measurement accuracy.

Proposal 3: For measurements that are defined based on a measurement period, requirements during the transition phase in case of SSB adaptation should be based on the longer measurement period, i.e., based on the longer of both SSB periodicities, i.e., max(TSSB1, TSSB2).

Observation 3: In case of beam failure detection, requirements during the transition phase are based on the more stringent evaluation period, i.e., the shorter one of both evaluation periods.

Proposal 4: In case of beam failure detection, requirements for the transition phase should be based on min(TSSB1, TSSB2).

Proposal 5: RAN4 to investigate whether for CBD requirements in the transition phase can be defined based on min(TSSB1, TSSB2).

# References

- R4-2412508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2146860, WF on RRM requirements for Netw_Energy_NR_eng_Part2, RAN4 #112bis, October 2024

- R4-2415663, Topic summary for [112bis][216], Netw_Energy_NR_eng_Part2, RAN4 #112bis, October 2024

- R4-2416911, LS on SSB adaptation, RAN4 #112bis, October 2024

- R4-2416913, LS on SSB relation in on-demand SSB and SSB adaptation on SCell, RAN4 #112bis, October 2024