3GPP TSG-RAN WG4 Meeting # 113	R4-2417539

Orlando, US, 18th – 22nd November 2024

Agenda Item:	7.2.4.2

Source: 	Nokia

Title: 	BS parameters for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This topic was discussed at TSG RAN4#112bis and the WF was agreed [2].

This contribution further discusses the BS parameters based on the agreed WF and the simulation results provided in [3] and [4].

# 2.	Discussion

2.1		BS antenna parameters for Urban Macro

It is stated in the agreed WF.

Issue 4-17: BS antenna parameters For Urban Macro: Agreement: - Make assumption that reply can say that array size is [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible. - Double check the coverage and [3072] before making final decision on reply - If coverage issues is found for [3072], minimum array size [4096] to achieve coverage could be discussed

The simulation results in [3] and [4] have shown that the 16x24 array and 4x1 sub-array can provide satisfactory DL and UL coverage in urban macro scenario with 450 m inter-site distance and 20% indoor UE ratio, either with 1 or 3 UE per slot. Smaller BS antenna array and sub-array size would reduce the inter-site distance (meaning existing sites in 3.5 GHz may not be reused) and degrade indoor UE coverage. On the other hand, larger BS antenna array with the similar sub-array size would provide enhanced coverage but at the expense of implementation complexity with the larger number of transceivers in the antenna array. Therefore, it is proposed to confirm 16x24 array and 4x1 sub-array as a trade-off between cell coverage and implementation complexity.

2.2		Output power for different BS class

It is stated in the agreed WF.

For output power for different BS class: Agreement: - For wide area BS, 46dBm for TRP for dual polarization; - For dense urban BS, [36dBm]for TRP for dual polarization; - For Indoor: 23dBm per polarization

The output power for different AAS BS classes in the LS replies on parameters for 4400 to 4800 MHz [5] and 7125 to 8400 MHz [6] are shown as follows:

|  |  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- | --- |
| 4400 to 4800 MHz | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 28 | 28 | 28 | 16 | N/A |
| 7125 to 8400 MHz | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | N/A | 22 | 22 | 16 | N/A |

To target the same DL coverage for 14800 to 15350 MHz as for 4400 to 4800 MHz and 7125 to 8400 MHz, the EIRP output power of the BS should be 6 dB more than those for 7125 to 8400 MHz. Comparing to 7125 to 8400 MHz, the 3 times larger array size (16*24 Vs 8816) for 14800 to 15350 MHz could provide 5 dB more conducted power per array and 5 dB more array beamforming gain, thus the conducted output power per sub-array or element should be 4 dB less than that for 7125 to 8400 MHz. Therefore, the output power for different AAS BS classes in the LS reply on parameters for 14800 to 15350 MHz should be as follows:

|  |  | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| 7125 to 8400 MHz | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 18 | 16 | N/A |

For the coexistence simulation, the current output power for different BS class is currently defined in TR 38.922 [7] as follows:

| BS max TX power in dBm (2 polarizations) | 23dBm | 46dBm | 36dBm |
| --- | --- | --- | --- |

Therefore, the output power for different BS class in the WF including the [36 dBm] for dense urban BS can be confirmed for coexistence simulation.

2.3		Emissions mask and spurious emission

It is stated in the agreed WF.

Issue 4-10: Emissions mask Agreement: For the reply, refer to the following table as basis - [category B] and category A - Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A Frequency offset of measurement filter 3dB point, Df; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz £ Df < 50 MHz; 0.05 MHz £ f_offset < 50.05 MHz; -7dBm-750foffsetMHz-0.05; 100 kHz. 50 MHz f < min(100 MHz, Dfmax); 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax); -14 dBm; 100 kHz. 100 MHz £ Df £ Dfmax; 100.5 MHz £ f_offset < f_offsetmax; -13 dBm; 1MHz. - [Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B Frequency offset of measurement filter 3dB point, Df; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz £ Df < 50 MHz; 0.05 MHz £ f_offset < 50.05 MHz; -7dBm-750foffsetMHz-0.05; 100 kHz. 50 MHz f < min(100 MHz, Dfmax); 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax); -14 dBm; 100 kHz. 100 MHz £ Df £ Dfmax; 100.5 MHz £ f_offset < f_offsetmax; -15 dBm; 1MHz. ] Issue 4-14: Spurious emission Agreement: For the reply, refer to the following table as basis - [category B] and category A - Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A Frequency range; Limit; Measurement Bandwidth; Note. 30 MHz – 1 GHz; -13 dBm; 100 kHz; Note 1. 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band; 1 MHz; Note 1, Note 2. NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1.. - Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B Spurious frequency range; Limit; Measurement bandwidth. 30 MHz – 1 GHz; -36 dBm; 100 kHz. 1 GHz – 18 GHz; -30 dBm; 1 MHz. 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band; -20 dBm; 10 MHz.

Category B emission mask and spurious emission requirements are applicable in Europe and included in the LS replies on parameters for 4400 to 4800 MHz [5] and 7125 to 8400 MHz [6], thus they could also be included in the LS reply for 14800 to 15350 MHz.

2.4		ACLR and ACS

It is stated in the agreed WF.

Issu 4-13: ACLR Potential agreement: Postpone to next meeting according to the simulation results. Issue 4-16: ACS agreement: Postpone to next meeting according to the simulation results.

The simulation results in [3] and [4] have shown that for both downlink and uplink, the average and 5%-tile throughput losses of the victim UE in all simulated cases are limited to around 5% with downlink ACIR offset of -9 dB compared to the current ACLR and ACS specified for FR1. This is also aligned with the trend that ACLR and ACS can be more relaxed in higher frequency range from coexistence viewpoint. Therefore, it is proposed the ACLR and ACS for 14800 to 15350 MHz should be relaxed for 9 dB compared to the current ones specified in FR1, i.e., 36 dB ACLR and ACS.

# 3.	Conclusion

This contribution further discusses the BS parameters based on the agreed WF at TSG RAN4#112bis and the obtained simulation results.

Proposal 1: To confirm 16x24 array and 4x1 sub-array as the BS antenna array size in urban macro scenario.

Proposal 2: The output power for different AAS BS classes in the LS reply on parameters for 14800 to 15350 MHz should be:

|  |  | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| 7125 to 8400 MHz | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 18 | 16 | N/A |

Proposal 3: Confirm the 36 dBm output power for dense urban BS for coexistence simulation.

Proposal 4: Category B spurious emission and spurious emission requirements could be included in the LS reply for 14800 to 15350 MHz.

Proposal 5: The ACLR and ACS for 14800 to 15350 MHz should be relaxed for 9 dB compared to the current ones specified in FR1, i.e., 36 dB ACLR and ACS.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2417107, “WF on 15GHz BS parameters”, ZTE.

[3]	R4-2417536, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 1 UE per slot”, Nokia.

[4]	R4-2417537, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 3 UE per slot”, Nokia.

[5]	R4-2410576, “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[6]	R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[7]	R4-2417200, “TR 38.922 version 0.3.0”, Ericsson.