3GPP TSG-RAN WG4 Meeting # 113                                                               R4-2417552

Orlando, US, 18 – 22 November, 2024

3GPP TSG RAN Meeting #106	RP-24xxxx

Madrid, ES, December 9-12, 2024	(revision of RP-yyxxxx)

# Source:	China Telecom

# Title:	New WID on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

# Document for:	Information

Agenda Item:	10

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

Acronym: NR_eff_BW_util

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_NR_eff_BW_util | RAN4 | 890040 | Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_channel_raster_enh | 991045 | NR channel raster enhancement | The WI “NR_channel_raster_enh” introduced new channel rasters to address the issue raised by the SI “FS_NR_eff_BW_util”, namely the location of the possible channel raster in some NR bands with 100kHz channel raster entries. |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

In Release 18, a Study Item (SI) titled "FS_NR_eff_BW_util" was established to facilitate more efficient utilization of licensed spectrum that does not conform to existing NR channel bandwidths, commonly referred to as "irregular channel bandwidth." To date, the following irregular channel bandwidths have been proposed:

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11, 12 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels.

During the study of solutions to achieve the highest spectral efficiency, a long-standing discussion was raised on how/whether to locate a narrower UE channel bandwidth within a wider cell-specific BS channel bandwidth. The WI " NR_channel_raster_enh " addressed this issue by introducing enhanced channel raster (10kHz entry) for the FR1 bands with 100kHz channel raster.

In light of the findings from both the SI and WI, it is advisable to initiate consideration for corresponding normative work in Release 19.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

Potential objectives for this work can be formulated as presented below:

- Next larger channel bandwidth:

-	Enable support for configuring and using standard channel bandwidth larger than the actual non-standard spectrum allocation.

-	Enable support for configuring UEs to the channels larger than SIB1 system bandwidth.

- Overlapping channels from network perspective:

-	Specify the necessary changes to use one common SSB to indicate two separate cells

-	Specify the necessary changes to use two overlapping SSBs.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
|  | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g. rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#110 | Core part |
| 38.101-5 | NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements | RAN#110 | Core part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#110 | Core part |
| 38.108 | NR; Satellite Access Node radio transmission and reception | RAN#110 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#110 | Core part |
| 38.331 | NR; Radio Resource Control (RRC) protocol specification | RAN#110 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.
Secondary co-Rapporteur(s) are possible for specific secondary task(s)}.

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN4

Secondary responsible Working Group(s):

RAN2

# 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

China Telecom