3GPP TSG-RAN WG4 Meeting #113												R4-2417590

Orlando, USA, 18 – 22 November, 2024

Agenda item:			7.12.2.1

Source:	Huawei, HiSilicon

Title:	on XR OTA tests

Document for:

# Introduction

The WF [1] in the last meeting captures the following issues. This contribution discusses them further.

Issue 2-2-1: Performance metric for XR devices

Agreements:

  - The glasses and head mounted displays should have the same performance metrics.

  - Further discuss whether performance requirements for the above two types of XR devices could be the same or not.

Issue 2-3-1: Coarse measurement grids for XR device testing

Agreements:

  - RAN4 further discuss coarse measurement grid based on XR device antenna pattern.

  - FFS whether current coarse grid in TR 38.870 can be used for XR device testing.

# Discussion

Generally speaking, the more space is given to antenna design, the higher the total antenna efficiency. As both TRP and TRS are directly linked to total antenna efficiency, this would lead to different performance values for glasses and head mounted displays due to the fact that head mounted displays have more space for antenna designs than glasses. If the same performance requirement were applied to both glasses and head mounted displays, glasses would be disadvantaged while head mounted displays would be able to meet the performance requirement with relative ease, because the requirement would be derived from a pool of glasses and head mounted displays.

Proposal 1: glasses and head mounted displays should have separate performance requirements.

Coarse grid can be used to reduce measurement time and would be helpful in OTA tests for glasses as the associated batteries may have limited capacity. However, batteries in head mounted displays could have sufficient capacity to last the entire measurement duration using normal grid.

Observation 1: XR glasses may need coarse measurement grid while normal measurement grid could be used for head mounted displays.

Other form of coarse grid could also be considered. For example, Icosahedron grid uses 12 points and provides above 90% correlation with results from normal spherical grid with a factor of four in measurement time reduction according to [2].

Proposal 2: other coarse grids such as Icosahedron grid may be worth studying to reduce measurement time.

# Conclusion

This contribution provides the following proposals and observation.

Proposal 1: glasses and head mounted displays should have separate performance requirements.

Observation 1: XR glasses may need coarse measurement grid while normal measurement grid could be used for head mounted displays.

Proposal 2: other coarse grids such as Icosahedron grid may be worth studying to reduce measurement time.

# References

- R4-2416590 WF [112bis][323] TRP_TRS_Ph3

- GSMA TSG AP69_004 New 3D Icosahedron grid for OTA antenna measurement, Feb 2022