3GPP TSG-RAN WG4 Meeting #113												R4-2417592

Orlando, USA, 18 – 22 November, 2024

Agenda item:			7.12.2.3

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:

# Introduction

The WF [1] from the last meeting highlights issues such as UMa and UMi channels and 2x2 vs 4x4 for band n3. This contribution discusses them further.

Issue 1-3: Dynamic MIMO OTA test and UE performance metric

Agreements:

- Consider dynamic channel MIMO OTA tests to include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for processing.

Issue 2-1-2: 4x4 requirement or 2x2 requirement for band n3

Agreements:

- RAN4 should make the decision on defining which MIMO OTA requirement for band n3 at RAN4#113 (Nov. 2024).

  - Option 1: 4x4 MIMO OTA requirement

  - Option 2: 2x2 MIMO OTA requirement

# Discussion

3GPP developed two main channel models, viz. UMa and UMi to cover different outdoor propagation scenarios. MIMO OTA tests should therefore test devices for both UMa and UMi channels to ensure that MIMO OTA tests cover more deployment scenarios. If MIMO OTA tests are limited to either UMa or UMi channel, the associated performance would not fully represent realistic network scenarios. Namely operators may not be able to use the MIMO OTA performance with confidence to distinguish or select devices for their network operations.

Proposal 1: dynamic channel MIMO OTA tests should include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for post processing.

Band n3 is not listed in TS 38.101-1 [2] as a mandatory band to support a minimum of four receive antennas as shown below.

## 7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

In general, a minimum four receive antennas is required for bands with frequencies higher than 2500MHz. However, with band n3 at frequency range of 1710 MHz – 1785 MHz, some devices have implemented four receive antennas at band n3. One way to identify whether two- or four-layer MIMO is supported for band n3 is to check IE (Information Element) in TS 38.331 [3] such as the one below.

MIMO-LayersDL :: =   ENUMERATED {twoLayers, fourLayers, eightLayers}

Assuming the above IE is available during MIMO OTA tests as support for this IE is optional, MIMO OTA tests for band n3 can be carried out as normal, the final results can then be post processed into two groups, viz. one group supporting 4x4 MIMO while the other does not. If the number of devices in any group meets the criteria agreed in WF [1] to specify performance requirements, then performance should be derived for that group.

Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum amount of data per band: 15

  - Minimum amount of UE models per band: 12

Proposal 2: carry out normal measurement campaign for band n3, then derive two-layer and/or four-layer MIMO OTA performances if criteria for minimum amount of data per band and UE models per band for two-layer or four-layer are met.

There is a possibility that devices with four receive antennas are classified as 2x2 MIMO because the required IE on four-layer MIMO support is absent. In this case, MIMO OTA performance measurement would be carried out as two-layer with the extra two antennas providing diversity enhancement. This would artificially enhance the two-layer MIMO performance compared with devise with only two receive antennas. Although a possibility, the chance of it happening in reality could be small because device manufacturers would like to “advertise” their capability beyond the minimum mandated by 3GPP.

# Conclusion

Proposal 1: dynamic channel MIMO OTA tests should include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for post processing.

Proposal 2: carry out normal measurement campaign for band n3, then derive two-layer and/or four-layer MIMO OTA performances if criteria for minimum amount of data per band and UE models per band for two-layer or four-layer are met.

# Reference

- R4-2416579  WF for [112bis][324] MIMO_OTA_Ph3

- TS 38.101-1, v18.5, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

- TS 38.331, v18.3, Radio Resource Control (RRC) protocol specification