3GPP TSG-RAN WG4 Meeting #113	R4-2417614

Orlando, US, 18th – 22th November, 2024

Agenda Item:	7.17.4.2

Source:	CAICT

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion

1	Introduction

This contribution provides discussion on following issue from last meeting.

- Issue 2-1: UE Rx knowledge

- Issue 2-3: Absolute RSRP accuracy

- Issue 2-4: Relative RSRP accuracy

2	Discussion

2.1 Issue 2-1: UE Rx knowledge

Issue 2-1: UE Rx beam knowledge Proposals - Option 1: UE Rx beam is known - the UE Rx beam gain has to be taken into account when making the prediction - Option 2: UE Rx beam is unknown - Option 3: depends on some conditions - which conditions? - Option 4: others - Recommended WF - Option 1

During measurement phase, UE needs to measure L1-RSRP of each Tx beam in set A/set B. For a specific Tx beam, the measured L1-RSRP values across multiple measurement instances may or may not correspond to different Rx beams. This depends on UE’s implementation of selecting Rx beam. For example, the UE may set a fixed Rx beam for measurement or use beam sweeping to find the best Rx beam.

After collecting all the measurement results, deciding the model input is also UE-specific. The UE might assume a common Rx beam for all Tx beams. With this approach, the prediction results of Tx beams in Set A can be assumed to use this common Rx beam. Alternatively, the UE could select the largest measured value for each Tx beam, which means the corresponding Rx beam may be different. With this approach, the prediction result of each Tx beam can be assumed to be paired with this corresponding Rx beam. However, the actual best Rx beam on prediction instance may or may not align with the one chosen during measurement phase.

Based on above understanding, we think UE Rx knowledge could be assumed to be known under certain conditions, for example, a common Rx beam is assumed in measurement and model inference, or the Rx beam keeps constant within the measurement window for a Tx beam. Thus, it is suggested to first discuss the Rx beam assumption in RAN4 tests.

Proposal 1: RAN4 to discuss the Rx beam assumption in AI-based beam prediction tests.

2.2 Issue 2-3: Absolute RSRP accuracy

In last meeting, we had following agreement on definition of absolute RSRP accuracy.

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In legacy tests for measurement accuracy, the power level of beams and testing conditions are specified, which allows us to determine benchmark of the actual L1-RSRP of beams. Once the benchmark is established, we can define a range of reported measurement values that could meet the accuracy requirements. Following this understanding, the latter part in the definition, i.e., “ground-truth of L1-RSRP of beam index i” in the definition should also serve as the benchmark for prediction accuracy. Therefore, the measurement for ground-truth should meet a more stringent requirement compared to legacy.

Proposal 2: RAN4 to consider define a more stringent requirements for measurement used to generate ground truth and discuss how to verify the measurement performance.

2.3 Issue 2-4: Relative RSRP accuracy

Issue 2-4: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: - Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is not configured for corresponding measurement. - Relative RSRP accuracy= predicted L1-RSRP of beam index i – measured L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is configured for corresponding measurement. - Option 2: Relative RSRP accuracy is the RSRP difference between the predicted L1-RSRP of different beams - Option 3: Relative RSRP accuracy is modified as follows: - Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted measured L1-RSRP of predicted beam index n, where the beam index n owns the largest predicted measured value obtained by measuring the predicted beams. - Option 4: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value

In TS 38.133, the relative RSRP accuracy is defined as “the L1-RSRP measured from one SSB/CSI-RS compared to the largest measured value of L1-RSRP among all SSBs/CSI-RS resources of the cell on which UE performs L1-RSRP measurements”, which is used to verify the difference between reported DIFF-RSRP and expected DIFF-RSRP among multiple beams. If continuing with this concept, then the relative RSRP accuracy for beam prediction shall represent the reported L1-RSRP of one beam compared to the largest reported value of L1-RSRP among all beams in the report of inference results. The definition of reported L1-RSRP relates to the ongoing discussion in RAN1.

RAN1 #116bis Agreement: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

For BM-case 1, if the beam is configured for corresponding measurement (i.e., the beam is one of the Top-K predicted beams and belongs to Set B), whether to report predicted value or measured value has not been decided yet. If predicted values are reported for all Top-K beams, the option 4 conforms to the concept of relative RSRP accuracy. If predicted value is reported for the beam not configured for corresponding measurement, and measured value is reported otherwise, then option 1 does.

For BM-case 2, only predicted RSRP are included in the inference report. Thus, option 4 can be taken as the definition of relative RSRP accuracy.

Proposal 3: Consider to take option 4 as the definition of relative RSRP accuracy for BM-case 2:

- Option 4: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value

Proposal 4: Suspend the discussion for BM-case 1 until RAN1 makes decision on the content of inference report for BM-case 1.

Conclusion

In this contribution, testability and interoperability issues for beam management are discussed with following proposals:

Proposal 1: RAN4 to discuss the Rx beam assumption in AI-based beam prediction tests.

Proposal 2: RAN4 to consider define a more stringent requirements for measurement used to generate ground truth and discuss how to verify the measurement performance.

Proposal 3: Consider to take option 4 as the definition of relative RSRP accuracy for BM-case 2:

- Option 4: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value

Proposal 4: Suspend the discussion for BM-case 1 until RAN1 makes decision on the content of inference report for BM-case 1.