3GPP TSG-RAN WG4 Meeting #113	R4-2417615

Orlando, US, 18th – 22th November, 2024

Agenda Item:	7.17.2.1

Source:	CAICT

Title:	Discussion on testability and interoperability issues for CSI compression

Document for:	Discussion

1	Introduction

This contribution provides discussion on feasibility of option 4a for CSI compression.

2	Discussion

In last RAN4 meeting, we had following agreement on this issue.

Agreement: Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

For option 4a, we understand that the specified dataset shall at least include {encoder input, encoder output w/ quantization}. The encoder input could contain either raw channel information or eigenvectors. Encoder output w/ quantization is used to align the feedback interface between test encoder and decoder.

- To generate the encoder input, RAN4 needs to agree on the testing conditions with specific channel assumptions. Then the channel data is generated and sampled through synthetic channel generation. Aggregating dataset from multiple sources to form a super dataset could also be considered, but it is necessary to first validate the data distribution across different datasets to ensure consistency.

- To generate the encoder output w/ quantization, in our understanding, a reference encoder is required to ensure the generation being independent of specific implementations. This reference encoder would serve as a benchmark to ensure that all TE vendors can test their decoders against a common dataset, facilitating comparability and interoperability. Quantization method could be explicitly captured as part of the reference encoder used to generate the output data.

- A performance requirement should also be specified to ensure the test decoder developed based on dataset achieves a sufficient performance level.

Proposal 1: The specified dataset for option 4a shall at least contain {encoder input, encoder output w/ quantization}.

Proposal 2: The encoder output in the dataset could be obtained through a reference encoder.

Proposal 3: A performance requirement shall be set for option 4a.

In order to investigate the feasibility of option 4a, it is necessary to specify test conditions and reference encoder, which both are part of the feasibility study for option 3. The evaluation assumptions for option 3 could be reused and the output reference encoder of option 3 could be used to construct the dataset for option 4a.

Observation 1: To investigate the feasibility of option 4a, the evaluation assumptions for option 3 could be reused and the output reference encoder of option 3 could be used to construct the dataset.

Therefore, the process for studying feasibility of option 4a could be outlined as following:

- Step 0: Finalized the feasibility study of option 3.

- Step 1: Generate the dataset of encoder input based on evaluation assumption and generate the data set of encoder output using the reference encoder from option 3.

  - Sub-step 1: validate the consistency of data distribution if datasets are aggregated.

- Step 2: Interested companies develop test decoders based on dataset set {encoder input, encoder output}.

- Step 3: Interested companies submit own encoders.

- Step 4: Conduct a cross-check of performance between all test decoders and all submitted encoders.

- Step 5: Summarize all testing results and determine the performance requirements for option 4a.

Proposal 4: RAN4 to discuss following steps for the feasibility study of option 4a:

- Step 0: Finalized the feasibility study of option 3.

- Step 1: Generate the dataset of encoder input based on evaluation assumption and generate the data set of encoder output using the reference encoder from option 3.

  - Sub-step 1: validate the consistency of data distribution if aggregating dataset is performed.

- Step 2: Interested companies develop test decoders based on dataset set {encoder input, encoder output}.

- Step 3: Interested companies submit own encoders.

- Step 4: Cross check the performance between all test decoders and all submitted encoders.

- Step 5: Summarize all testing results and determine the performance requirements for option 4a.

Conclusion

In this contribution, feasibility of option 4a for CSI compression are discussed with following proposals:

Proposal 1: The specified dataset for option 4a shall at least contain {encoder input, encoder output w/ quantization}.

Proposal 2: The encoder output in the dataset could be obtained through a reference encoder.

Proposal 3: A performance requirement shall be set for option 4a.

Observation 1: To investigate the feasibility of option 4a, the evaluation assumptions for option 3 could be reused and the output reference encoder of option 3 could be used to construct the dataset.

Proposal 4: RAN4 to discuss following steps for the feasibility study of option 4a:

- Step 0: Finalized the feasibility study of option 3.

- Step 1: Generate the dataset of encoder input base on evaluation assumption and generate the data set of encoder output using the reference encoder from option 3.

  - Sub-step 1: validate the consistency of data distribution if aggregating dataset is performed.

- Step 2: Interested companies develop test decoders based on dataset set {encoder input, encoder output}.

- Step 3: Interested companies submit own encoders.

- Step 4: Cross check the performance between all test decoders and all encoders.

- Step 5: Summarize all testing results and determine the performance requirements for option 4a.