3GPP TSG-RAN WG4 Meeting #113	R4-2417617

Orlando, US, 18th – 22th November, 2024

Agenda Item:	7.17.5.2

Source:	CAICT

Title:	Discussion on testability and interoperability issues for positioning

Document for:	Discussion

1	Introduction

This contribution provides discussion on following issue from last meeting.

- Issue 3-1: Requirements for case 1

2	Discussion

2.1 Issue 3-1: Requirements for case 1

In last meeting, RAN4 had following agreement on this issue.

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

For case 1 (AI/ML assisted positioning), the model is implemented internally within DUT, and the model output is intermediate results which is also invisible to outside. Thus, the testability for this case is questionable.

For case 1 (direct positioning), there is no existing tests and requirements on direct positioning performance in current specification. To specify tests and requirements of case 1, the most critical issue is how to set up the test environment, for example, whether to use channel emulators to simulate the testing environment or to establish a real testing site.

- The concern with the former approach is the discrepancy between the emulated environment and real deployment, which may cause the model to perform well in tests but fails in the field. In legacy specification TS 37.571-1, positioning tests assume at most two transmission points and utilize simple channel conditions, such as AWGN channel or a two-tap propagation model. How to emulate propagation conditions at different UE locations under multi-TRP scenario requires further study.

- The challenge in establishing a real testing site is to determine which deployment scenarios and how large scale should be adopted to ensure that the performance tested at the site is sufficiently representative to validate the performance in real deployment. Additionally, whether and how DUT to acquire training data/label from testing site is also a key problem.

Proposal 1: RAN4 to study potential solutions on setting up test environment for case 1, such as,

- Channel emulation based

  - FFS: feasibility of emulating propagation conditions at different UE locations under multi-TRP scenario

- Establishing real testing site

  - FFS: which deployment scenarios and how large scale should be adopted

  - FFS: whether and how DUT to acquire training data/label from testing site

- FFS: other solutions

Conclusion

In this contribution, testability and interoperability issues for positioning are discussed with following proposal:

Proposal 1: RAN4 to study potential solutions on setting up test environment for case 1, such as,

- Channel emulation based

  - FFS: feasibility of emulating propagation conditions at different UE locations under multi-TRP scenario

- Establishing real testing site

  - FFS: which deployment scenarios and how large scale should be adopted

  - FFS: whether and how DUT to acquire training data/label from testing site

- FFS: other solutions