3GPP TSG-RAN WG4 Meeting #113											R4-2417625

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SSB for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Enhancements of network energy savings for NR” has been approved in [1]. The RRM requirement for On-Demand SSB was discussed in WF Error! Reference source not found.. Besides, one corresponding RAN1 LS is received [4] to ask the timeline for On-demand SSB operation on SCell. In this contribution, our view is further provided for the abovementioned topics.

# 2  On-Demand SSB SCell operation (Objective#1 )

RAN1 LS

Based on the RAN1 agreement in the LS [4] as below, the timeline is illustrated in Figure 1.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=  where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and  is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to

Figure 1: OD-SSB timeline

In our understanding, the T_min is provide as the MAC decoding time, which is same as the legacy MAC-CE based SCell activation (i.e. T_HARQ + 3ms), in order to let UE know the starting of SSB. So the RAN1 working assumption can be confirmed.

Proposal 1: Confirm that T_min can be equal to

RRM requirement – background

Also noted the following terminology during the discussion:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- Scenario #2: on-demand SSB can be triggered by gNB when SCell is configured to a UE but before the UE receives SCell activation command

- Scenario #2A: on-demand SSB can be triggered by gNB when UE receives SCell activation command

The corresponding SCell activation timeline for Scenario #2 and Scenario #2A are illustrated in Figure 2 and Figure 3, respectively. Note in Scenario #2, the OD-SSB could come later then the SCell activation comment (Subcase 2 in Figure 2), similar to Scenario #2A, although the OD-SSB activation comment comes earlier.

Figure 2: Scenario #2 - for both Case 1-no always on SSB (with the shaded block) and Case 2 (w/o the shaded block).

Figure 3: Scenario #2A - for both Case 1-no always on SSB (with the shaded block) and Case 2 (w/o the shaded block).

OD-SSB based deactivated SCell L3 measurement (for both Case 1 and Case 2)

Based on Figure 2 and Figure 3, it is noted that the deactivated SCell measurement applies on the subcase 1 in Scenario #2 (not applicable for Scenario #2A).

One example for the legacy time period for deactivated SCell measurement, as shown below, the CSSF was considered in the legacy and it seems not impacted by the OD-SSB procedure.

Table 9.2.5.1-12: Time period for PSS/SSS detection, deactivated PSCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Proposal 2: The CSSF should be considered in the OD-SSB based deactivated Scell measurement.

In RAN#4#112bis [5]

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

The following progress on OD-SSB based deactivated SCell L3 measurement has been reached:

Regarding Issue 1-2-1 (case 1), as during the time window, UE conducts rapid measurements within the specified time window to expedite the 1st measurement report, which can accelerate SCell activation. However, keep fast measurement will consume additional power, comparing to the legacy behavior. Thereby, it is preferable UE to fallback to legacy requirement after this time window.

Proposal 3: For OD-SSB based deactivated SCell measurement (Case 1), after the time window, UE can perform deactivated SCell measurement with legacy requirement (Option 2-1 in Issue 1-2-1).

Regarding Issue 1-3-4 (case 2), as the always-on SSB and OD-SSB may have different periodicity, thus the most simple way is to follow the OD-SSB periodicity when they are available, and the same time window approach as in case 1 can also apply.

Proposal 4: For OD-SSB based deactivated SCell measurement (Case 2), after the time window, UE can perform deactivated SCell measurement with measCycleSCell (new Option in Issue 1-3-1).

RRM requirement –SCell activation

In RAN#4#112 [3]

Issue 1-1-1: OD-SSB work plan

- Agreement:

  - RAN4 to first focus on the MAC CE based single SCell activation case as starting point.

  - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed.

In RAN#4#112bis [5]

Issue 1-4-1: General

Agreement:

- RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios.

The following progress on OD-SSB based deactivated SCell L3 measurement has been reached:

Based on Figure 2 and Figure 3, it can be observed that the OD-SSB may come later than the SCell activation comment in both Scenario 2/2A and case 1&2, so the T_uncertainty, as the waiting time for the OD-SSB, should be considered.

Proposal 5: For SCell activation requirement, the waiting time for OD-SSB should be considered in both case 1&2.

# Conclusion

Proposal 1: Confirm that T_min can be equal to

Proposal 2: The CSSF should be considered in the OD-SSB based deactivated Scell measurement.

Proposal 3: For OD-SSB based deactivated SCell measurement (Case 1), after the time window, UE can perform deactivated SCell measurement with legacy requirement (Option 2-1 in Issue 1-2-1).

Proposal 4: For OD-SSB based deactivated SCell measurement (Case 2), after the time window, UE can perform deactivated SCell measurement with measCycleSCell (new Option in Issue 1-3-1).

Proposal 5: For SCell activation requirement, the waiting time for OD-SSB should be considered in both case 1&2.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1

- R4-2413885, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, Aug. 2024.

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple, Aug. 2024.

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, Oct. 2024.