3GPP TSG-RAN WG4 Meeting #113											R4-2417624

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.26.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NR NTN phase 3

WID/SID:        NR_NTN_Ph3-Core

Document for:	Discussion

# 1 Introduction

This paper presents our perspective on RRM requirement, focusing on the unresolved issues identified in RAN4#112-bis WF [1].

# 2 General consideration

The specified TN Rel-17 RedCap and Rel-18 eRedCap RRM requirements can be leveraged to update the requirements for FR1 NTN Redcap UE. The potential impact from (e)Redcap is summarized as below, with considerations of 1 RX, HD-FDD, and eDRX.

Table 1 The potential impact from (e)Redcap

| Requirements | The potential impact from (e)Redcap |
| --- | --- |
| IDLE/INACTIVE, including SDT | - Update for eDRX - Update for 1 Rx, including UE measurement capability - Update for HD-FDD, for paging reception |
| CONN Mobility (Handover) & CONN Control | - Update for 1 Rx - Update for HD-FDD |
| Timing | - No major update foreseen |
| Signaling characteristics (RLM) (CONN), BFD/CBD | - Update for 1 Rx - Update for HD-FDD, including RLM/BFD L1 indication, UL SR switch |
| Meas. Procedure in CONN | - Update for 1 Rx - Update for HD-FDD, including scheduling restriction |

MDT requirements

#### Issue 2-2-3: The Requirements to be further determined

- FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

It has been noted that no MDT requirements have been established for R17/R18 Redcap UE. However, MDT requirements are already defined for non-Redcap R17 NTN UEs. We recognize that logged MDT reports are advantageous for network deployment and planning purposes, and they could be effectively utilized by non-Redcap R17 NTN UEs.

Proposal 1: Not to define MDT requirements for NTN Redcap UEs.

#### Issue 2-3-1: RedCap R17 relaxed measurement

<Agreement>:

- FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands.

- FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline.

  - If defined, the requirements apply provided that target cell is GSO

IDLE mode relaxed measurement

We recognize that the requirements defined for TN can be mostly applied to GSO. Thus both the R17 NR NTN relaxations for non-Redcap UE and the R17 TN RedCap relaxed measurement requirements can be applicable to GSO for (e)RedCap UE with FR1-NTN bands.

Proposal 2: For (e)RedCap UE with FR1-NTN bands in GSO IDLE mode, both the R17 NR NTN relaxations for non-Redcap UE and the R17 TN RedCap relaxed measurement requirements are applicable.

SMTC and concurrent MG

It has been noted that concurrent MG requirements have yet been established for (e)Redcap UE. If concurrent MG is not supported, then the support of multiple SMTCs will be significantly underutilized, particularly in scenarios where there are 2 LEO satellite with differing timing offsets. Given that the concurrent MG is an optional capability, it is our recommendation that the support 2 SMTC per MO be considered optional as well.

Proposal 3: For (e)RedCap UE with FR1-NTN bands, both the support of concurrent gap and multiple SMTC are optional.

# 3 FR1 1RX NTN (e)RedCap UEs

The clear and comprehensive Way Forward has been provided in the last meeting [1], and most requirements can be defined based on the Way Forward.

Observation 1: The majority of RRM requirements for FR1 1RX NTN Redcap can be established in accordance with the guidance provided in Way Forward R4-2416865.

# 4 HD-FDD

It has been agreed as the following in 2024.08:

<Agreement>

- RAN4 to consider the impact of HD-FDD for (e)Redcap UE with FR1-NTN.

  - The TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point, and RAN4 take into account NTN specific aspects if any.

Besides, as provided in Table 1 and Proposal 1a in WF [1], the potential impact from HD-FDD is listed as below:

Proposal 4: RAN4 to discuss the following HD-FDD related RRM requirement for NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover, Random access, RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR, Uplink spatial relation switch delay

- Scheduling availability

# Conclusion

Proposal 1: Not to define MDT requirements for NTN Redcap UEs.

Proposal 2: For (e)RedCap UE with FR1-NTN bands in GSO IDLE mode, both the R17 NR NTN relaxations for non-Redcap UE and the R17 TN RedCap relaxed measurement requirements are applicable.

Proposal 3: For (e)RedCap UE with FR1-NTN bands, both the support of concurrent gap and mulitple SMTC are optional.

Observation 1: The majority of RRM requirements for FR1 1RX NTN Redcap can be established in accordance with the guidance provided in Way Forward R4-2416865.

Proposal 4: RAN4 to discuss the following HD-FDD related RRM requirement for NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover, Random access, RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR, Uplink spatial relation switch delay

- Scheduling availability

# References

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, Oct. 2024.

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, Oct. 2024.