3GPP TSG-RAN WG4 Meeting #113	R4-2417630

Orlando, FL, US, Nov. 2024

Source	:	Qualcomm Incorporated

Title	:	On UE Rx requirements for the LPWUR

Agenda item	:	7.23.2.2

Document for	:	Approval

# 1.	Introduction

In this contribution we first share our views on open items pertaining to the core requirements. We also identify development direction considering plenary confirmation about scope of WI and FR2.

# 2. 	Discussion

## 2.1	FR2 applicability

### 2.1.1	FR2 work-load

Clarification in the RAN#105 [3] confirms that FR2 is in scope:

- Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Moreover, there is expected to be strong overlap in LPWUS design between FR1 and FR2, and no extra time is deemed necessary. It is therefore prudent for RAN4 to start constructing link-level simulation assumptions for the FR2 scenario. In terms of uncertainty in LPWUS assumptions, RAN1 still has not finished defining the LPWUS even for FR1 (examples: known LPWUS sequence length, chip rate, etc). These details can be left open in RAN4 with the understanding that parallel development in RAN1 will fill in these gaps, for both FR1 and FR2.

### 2.1.2	FR1/FR2 coverage comparison

The appendix contains a comparative coverage analysis across FR1 and FR2, based on values recorded in TR38.830 (coverage enhancement TR). The main observation is that FR1 and FR2 bands are similar from an implementation margin perspective because their respective UL and DL coverages scale similarly enough. i.e. no special procedure is required for FR2

Observation 1: FR1 and FR2 are equivalent from the perspective of designing a LP-WUR with sufficient coverage, and no unique consideration is required for FR2.

### 2.1.3	FR2 receiver topology

It is first useful to revisit the argumentation and assumptions from Rel-15 for PDSCH in FR2. Compared to FR1, FR2 channels are considered less scattering-rich, and this prompted the use of a dual-pol Rx as the building block of any FR2 receiver in RAN4. Further it was assumed that the 2 Rx chains involved in the dual-pol reception would be combined using MRC. For legacy PDSCH reception cases in RAN4, the net result was that the UE could realize whatever antenna gain was assumed in a pol. agnostic sense. Note that this is true even for Redcap UE

Observation 2: The legacy FR2 UE (main) receiver is assumed to comprise 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized. For sensitivity requirements, the chains are assumed to be combined by MRC.

For an FR2 LPWUR, this serves as a natural precedent. It may however be beneficial to assume a slightly simpler receiver for LPWUR for cost and complexity reasons. The simplification here pertains to combining strategy, for example it would choose the Rx chain with the stronger signal (‘polarization selection diversity’). This type of scheme suffers a minor loss in sensitivity, but it does not lose any diversity in the polarization dimension, an acceptable compromise for LPWUR. Further, this assumption allows FR2 to align with FR1 in terms of number of receivers used to determine SNR performance (1RX).

Observation 3: A simplification for the FR2 LPWUR from the 2Rx FR2 MR is adoption of ‘best Rx chain selection’ as the selection method (instead of MRC)

A further open issue is whether the FR2 LPWUR uses a refined beam vs a single element beam. Given the lower power consumption of a single element beam, as well as the similarity in FR1 and FR2 (section 2.1.2 and annex) with a single-element beam assumption, it seems reasonable to use the single element beam as a baseline for FR2 LPWUR.

Observation 4: To accommodate single element beam, the baseline FR2 LPWUR implementation is assumed to have 6 dB lower antenna gain compared to the FR2 main radio assumption for sensitivity.

These assumptions can be revisited if the MSG3 coverage criterion cannot be fulfilled.

Proposal 1: For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features:

- It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized

- Best Rx chain is used for LPWUS (pol. selection diversity)

- Antenna gain is that of a single element

Note that the proposals above are not a requirement, but a guideline towards converging on a sensitivity requirement for FR2 LPWUR.

Finally, the FR1 RAN4 agreement on receiver topology (zero-IF) can be carried over to FR2.

Proposal 2: FR2 LPWUR adopts zero-IF topology baseline assumption from FR1 LPWUR

At this point it is useful to identify the impact of the proposed combining method (pol. selection diversity) on the verification side. Legacy RAN4 procedure for any type of FR2 sensitivity measurement requires testing over two orthogonal DL polarizations and deriving the directional sensitivity as a harmonic mean of the sensitivity values measured for the two DL polarizations. In the worst-case situation, in beam peak direction, the polarization basis of the UE’s receive antenna can be misaligned by 45 to the polarization basis of the DL antenna, so the signal is 3 dB degraded in SNR regardless of which chain is selected by the combining algorithm. In general (rather than the WC situation) the pol. sel. diversity method will result in no more than 3 dB degradation in SNR compared to the main radio.

Observation 5: To accommodate worst case SNR degradation due to ‘best Rx chain selection’ combining, the baseline FR2 LPWUR implementation can be assumed to have 3 dB worse noise figure compared to the FR2 main radio.

### 2.1.4	FR2 LLS assumptions

A good starting point is the set of assumptions agreed for FR1 [2]. Some aspects need to be revisited, both to reflect progress in RAN1, as well as unique aspects to FR2. The FR2 set is shown below with highlighted parameters that change from FR1 agreements

Proposal 3: RAN4 to adopt the following link level simulation assumptions as starting point for the FR2 use case:

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | 0 (up to UE implementation) |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 5MHz (11 RB)-case 3 |  |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [16~17] dB NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | 7.68MHz |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | Not modelled |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

## 2.2	SNR proposal

We previously shared some SNR results [6] based on OFDM-based receivers, where we noted that FR1 and FR2 baseband sections perform similarly. We also reported strong dependence of SNR vs MDR curves on known sequence length and chip rate ‘M’ for realistic OFDM receivers.

To determine if this behavior extends to OOK receivers, we turned to our RAN1 simulator. This simulator is somewhat simpler than our OFDM-based simulator in not considering time and frequency error, but its results remain a useful indicator of relative performance trends. In the figure to the left, SNR vs M trends show that the OOK-based receiver is also projected to retain sensitivity to M and known sequence length.

Applicable to both LPWUR types then, SNR results are contingent on RAN1 specification of LPWUS design, including OOK type, chip rate and known sequence length pairings, as well as identification of sequences for receivers that are susceptible to cross-identification. It is therefore premature to conclude on SNRs for sensitivity requirements for the LPWUR.

Observation 6: It is premature for RAN4 to conclude on SNRs for sensitivity requirements for LPWUR, regardless of receiver architecture, due to lack of sufficient progress on LPWUS specification in RAN1.

Proposal 4:  RAN4 to postpone any SNR discussion for the sensitivity requirement until RAN1 LPWUS definition is complete.

## 2.3	NF and IM proposals

Proposal 5: The table below captures Qualcomm estimates of SNR, NF and IM (implementation margin) for the LPWUR

|  | OOK-based | OFDM-based | Notes |
| --- | --- | --- | --- |
| SNR for MDR = 1% (FAR = 1%) | FFS | FFS | RAN1 has not completed LPWUS design yet. Includes time frequency errors |
| FR1 NF (dB) @ sensitivity | MR + [1] + X | MR + [1] | [1] dB is placeholder for switch losses. Final value may have some dependence on band. X = [0-3] dB is sufficient for stand-alone OOK |
| FR2 NF (dB) @ sensitivity | MR + [1] + X + Y | MR + [1] + Y | [1] dB is placeholder for switch losses. Final value may have some dependence on band. X = [0-3] dB is sufficient for stand-alone OOK Y = 3 dB degradation to accommodate simpler Rx combining than FR2 MR |
| FR2 antenna gain (dB) | MR – 6 | MR – 6 | Legacy assumption is module with 4x1 array. 6 dB degradation assumes single element beam for LPWUR operation |
| Implementation margin (dB) | 2.5 | 2.5 | consistent with legacy demod. assumptions. Includes IQ Image, phase noise, cal errors, fixed point, other analog impairments (excludes time/freq error, which is captured in SNR) |

## 2.4	On guard RBs

If guard RBs are needed to be arranged around the LPWUS they would also be needed for the always-on LPSS signal. Guard RBs therefore represent increased system overhead. From a system perspective, it is preferred that no guard RBs are specified for the LPWUS and LPSS signals.

Proposal 6: To minimize system overhead, RAN4 to consider specifying LPSS and LPWUS with no guard RBs.

## 2.5	Interferer levels

The LPWUR is expected to perform in the same environment as the main radio. There is no system-level justification to design the LPWUR for anything other than the same interferer levels as the main radio. This criterion should drive phase noise, etc., not the other way around.

Proposal 7: The LPWUR shall be hardened to the same absolute interferer levels as the main radio. Phase noise is an implementation detail and RAN4 stops discussing it in jammer context.

## 2.6	Max. input power

Legacy practice for this parameter is to define the requirement for all possible channel bandwidths. The challenge for the LPWUR is that input power can be differently distributed between LPWUS and legacy DL RBs, depending on channel bandwidth. Moreover, since competitive implementations are expected to use AGC, the additional challenge is to set the AGC correctly, i.e. not saturate components while leaving enough LPWUS power for detection. Finally, the core requirement should ensure all channel BWs have a specified max. power guarantee.

Proposal 8: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

It can be left up to RAN5 how to streamline test cases.

# 3.	Conclusion

Observation 1: FR1 and FR2 are equivalent from the perspective of designing a LP-WUR with sufficient coverage, and no unique consideration is required for FR2.

Observation 2: The legacy FR2 UE (main) receiver is assumed to comprise 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized. For sensitivity requirements, the chains are assumed to be combined by MRC.

Observation 3: A simplification for the FR2 LPWUR from the 2Rx FR2 MR is adoption of ‘best Rx chain selection’ as the selection method (instead of MRC)

Observation 4: To accommodate single element beam, the baseline FR2 LPWUR implementation is assumed to have 6 dB lower antenna gain compared to the FR2 main radio assumption for sensitivity.

Proposal 1: For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features:

- It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized

- Best Rx chain is used for LPWUS (pol. selection diversity)

- Antenna gain is that of a single element

Proposal 2: FR2 LPWUR adopts zero-IF topology baseline assumption from FR1 LPWUR

Observation 5: To accommodate worst case SNR degradation due to ‘best Rx chain selection’ combining, the baseline FR2 LPWUR implementation can be assumed to have 3 dB worse noise figure compared to the FR2 main radio.

Proposal 3: RAN4 to adopt the following link level simulation assumptions as starting point for the FR2 use case:

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | 0 (up to UE implementation) |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 5MHz (11 RB)-case 3 |  |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [16~17] dB NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | 7.68MHz |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | Not modelled |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

Observation 6: It is premature for RAN4 to conclude on SNRs for sensitivity requirements for LPWUR, regardless of receiver architecture, due to lack of sufficient progress on LPWUS specification in RAN1.

Proposal 4:  RAN4 to postpone any SNR discussion for the sensitivity requirement until RAN1 LPWUS definition is complete.

Proposal 5: The table below captures Qualcomm estimates of SNR, NF and IM (implementation margin) for the  LPWUR

|  | OOK-based | OFDM-based | Notes |
| --- | --- | --- | --- |
| SNR for MDR = 1% (FAR = 1%) | FFS | FFS | RAN1 has not completed LPWUS design yet. Includes time frequency errors |
| FR1 NF (dB) @ sensitivity | MR + [1] + X | MR + [1] | [1] dB is placeholder for switch losses. Final value may have some dependence on band. X = [0-3] dB is sufficient for stand-alone OOK |
| FR2 NF (dB) @ sensitivity | MR + [1] + X + Y | MR + [1] + Y | [1] dB is placeholder for switch losses. Final value may have some dependence on band. X = [0-3] dB is sufficient for stand-alone OOK Y = 3 dB degradation to accommodate simpler Rx combining than FR2 MR |
| FR2 antenna gain (dB) | MR – 6 | MR – 6 | Legacy assumption is module with 4x1 array. 6 dB degradation assumes single element beam for LPWUR operation |
| Implementation margin (dB) | 2.5 | 2.5 | consistent with legacy demod. assumptions. Includes IQ Image, phase noise, cal errors, fixed point, other analog impairments (excludes time/freq error, which is captured in SNR) |

Proposal 6: To minimize system overhead, RAN4 to consider specifying LPSS and LPWUS with no guard RBs.

Proposal 7: The LPWUR shall be hardened to the same absolute interferer levels as the main radio. Phase noise is an implementation detail and RAN4 stops discussing it in jammer context.

Proposal 8: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2410569, ‘WF for [111][136] NR_LPWUS_UERF’, vivo, RAN4#111, May 2024

[3] 	RP-242361, ‘Way Forward on LP-WUS for FR2’, vivo, RAN#105, Sept. 2024

[4] 	R4-2414309, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#112, Aug 2024

[5] 	R4-2413223, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#112, Aug 2024

[6] 	R4-2415723, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#112-Bis, Oct. 2024

# 5.	Annex: Coverage analysis (comparison b/w FR1 and FR2)

There have been some concerns regarding LP-WUS coverage in FR2, if supported. However, the requirement is to meet the msg-3 PUSCH coverage in the same carrier/FR. For FR2, msg-3 PUSCH coverage is smaller compared to FR1 in general.

Figure 15: LP-WUS and Msg-3 PUSCH coverages in different FRs

Below, we provide an analysis of LP-WUS coverage with respect to FR2. Firstly, let us consider analysis for FR1 (2.6GHz). As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 11 PRBs with SCS 30kHz for LP-WUS.

Table A-1.	Parameters to derive LP-WUS MIL for FR1 (2.6GHz)

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 192 |
| UE number of antenna elements | 1 |
| Power spectrum density | 33dBm per MHz |
| Occupied bandwidth | 3.96 MHz |
| System bandwidth | 95 MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, receiver gain, received signal level, as follows

EIRP=psdPerMhz+10⋅log10occBw+dBiTx+10⋅log10numTxEl=33+10⋅log104.32+8+10⋅log10192=70.2dBm

rxGain=sdBiRx+10⋅log10numRxEl=5+10⋅log101=5dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log103.96⋅106+SNRsensitivityDL WUS

where NFDL and SNRsensitivityDL WUS are noise figure and sensitivity of LP-WUS. From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=183.2-NFDL-SNRsensitivityDL WUS-impLossDL

where impLossDL is an implementation loss. The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

RAN1 agreed to use MILMsg3=153.51dB from TR38.830 Table 5.1.1.2-2 at the RAN1#116bis meeting. Putting these together, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=29.7 dB-SNRsensitivityDL WUS+NFDL+impLossDL ≥0

SNRsensitivityDL WUS is the required SNR to achieve the target performance of LP-WUS and is dependent on LP-WUS design. NFDL and impLossDL are dependent on the LP-WUR implementation.

Next, we can do the same for FR2. As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 12 PRBs with SCS 120kHz for LP-WUS.

Table A-2.	Parameters to derive LP-WUS MIL for FR2

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 128 (per polarization) |
| UE number of antenna elements | 4 (per polarization) |
| Power spectrum density | 40dBm per system BW |
| Occupied bandwidth | 15.84MHz |
| System bandwidth | 95MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, Rx gain, Rx signal level, as follows

EIRP=psdSysBw+10⋅log10occBwsysBw+dBiTx+10⋅log10numTxEl=40+10⋅log1015.8495+8+10⋅log10128=61.3dBm

rxGain=sdBiRx+10⋅log10numRxEl=5+10⋅log104=11dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log1015.84⋅106+SNRsensitivityDL WUS

From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=174.3-NFDL-SNRsensitivityDL WUS-impLossDL

The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

Same as for FR1, MILMsg3 can refer to the value captured in TR38.830 Table 5.2.1.1-2, i.e., MILMsg3=139.72dB. From the above, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=34.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0

Note here that the same beam shapes were assumed at the UE for receiving LP-WUS as well transmitting msg 3. A worse situation for WUS reception is if the UE uses less refined beams for it. In this case there would be a 6 dB reduction in FoM, calculated originally as 34.6 dB above

Table A-3

| FR | The condition that LP-WUS coverage matches or exceeds Msg-3 coverage |
| --- | --- |
| FR1 | 29.7 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 |
| FR2 | 28.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 (*) (*) assuming rough beam for LPWUS detection |

Observation: The coverage criteria (i.e. implementation challenges) looks very similar across FR1 and FR2