3GPP TSG-RAN4 Meeting # 113	R4-2417632

# Orlando, FL, USA, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | 19 | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR on RF requirements for a UE with different TCI states for UL and DL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5 |  |  |  |  |  | Date: |  |  | Nov 8, 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | FOR INFORMATION ONLY: Draft feature CR fro FR2 component of NR_MIMO_Ph5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | FOR INFORMATION ONLY: In this feature, UE transmits UL towards an UL TRP while receiving DL from another TRP. The UL TRP is assumed to transmit SSB to aid in UE beam management. In this mode of operation, the UL and DL have different TCI states. The proposed UE RF requirement is that the UE can transmit sufficient power in the direction of the UL TRP based on SSB or similar beam management reference signal reception from the UL TRP. This is the justification for the new UE RF requirement in 6.6K. No receive functionality is touched and there is no need to re-verify other transmit requirements. This is the justificaiton to limit the UE RF requirement for this feature to 6.6k |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A feature to enhance UL conditions is not standardized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6K |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start changes *********

## 6.6A	Beam correspondence for CA

For intra-band CA in FR2, the same beam correspondence relationship for beam management is supported across CCs in this release of the specification and no requirement is specified. Beam correspondence performance for intra-band CA is fulfilled if the beam correspondence requirements defined in clause 6.6 is met for non-CA case.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, with all CCs active with non-zero UL RB allocation, the following beam correspondence requirements apply for each CC:

1	The minimum peak EIRP requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

2	The common spherical coverage requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

## 6.6K	Beam correspondence for [UEs with different TCI states for UL and DL]

For a UE that supports [capability_indication] and is scheduled with different TCI states for DL and UL respectively, the requirements of clause 6.6 apply. The side conditions specified in clause 6.6 apply to SSB transmissions associated with each of the two TCL states, UL and DL, as well as all signals that maintain QCL type D relation with those SSBs, where applicable.

*** End changes *********