3GPP TSG-RAN WG4 Meeting #113	R4-2417636

Orlando, FL, US, Nov. 2024

Source	:	Qualcomm Incorporated

Title	:	Discrepancy between 38.306 and 38.101-3 on maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16

Agenda item	:	4.2.1

Document for	:	Approval

# 1.	Introduction

There is inconsistency in the default values of dutycycle associated with the capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16. The specific problem is the assumed duty cycle value for the NR band in case the capability value is not signaled.

# 2.	Discussion

## 2.2	View A

On the one hand, TR37.825 (‘High power User Equipment (UE) (power class 2) for E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) (1 LTE TDD band + 1 NR TDD band’ captures the following SAR solution

### 5.6.2 Inter-band EN-DC If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-EN-DC is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-EN-DC is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-EN-DC as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or - if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower: - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2B.4; - else : : :

This is echoed verbatim by 38.101-3, Rel-16 +, so there is consistency between the TR and the core requirement:

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or : : :

Observation 1: TR37.825 and TS38.101-3 (Rel-16) are consistent and suggest that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the deafult duty cycle capability for the NR band

## 2.2	View B

On the other hand, 38.306 records in somewhat different language:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | BC | No | TDD only | FR1 only |

Observation 2: 38.306 suggests that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the deafult duty cycle capability for the E-UTRA band

## 2.3	Proposal

View A and view B conflict with each other, but view B is incorporated into RAN2 signaling and changing it to align with view A would imply a non-backward-compatible change. Changing 38.101-3 to align with view B on the other hand risks changing the intent behind the signalling. This conflict needs resolution, but first RAN4 must agree on the way forward.

Proposal: RAN4 to establish common understanding on the meaning of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16, in context of the following conflicting interpretations if it is not signalled:

1. 38.101-3 says that the default is 30% UL on NR carriers:  the criterion for further decision is if  a  'percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30%'

2. 38.306 says that a default value of '30% shall be applied to all EUTRA TDD uplink-downlink configurations'"

# 3.	Conclusion

Observation 1: TR37.825 and TS38.101-3 (Rel-16) are consistent and suggest that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the deafult duty cycle capability for the NR band

Observation 2: 38.306 suggests that if maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not signaled, 30% is applied as the deafult duty cycle capability for the E-UTRA band

Proposal: RAN4 to establish common understanding on the meaning of maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16, in context of the following conflicting interpretations if it is not signalled:

1. 38.101-3 says that the default is 30% UL on NR carriers:  the criterion for further decision is if  a  'percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30%'

2. 38.306 says that a default value of '30% shall be applied to all EUTRA TDD uplink-downlink configurations'"