3GPP TSG-RAN WG4 Meeting #113	R4-2417681

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	On remaining Rel-18 LTM core requirements

Agenda item:	5.23.1

Document for:	Discussion

Introduction

In this contribution we discuss the remaining open issues for LTM core part requirements based on the RAN4#112 way forward [1].

Inter-frequency BC capability LS from RAN2 R2-2409360

During Feature group discussions RAN4 discussed about inter-frequency BC capability, and whether the inter-frequency definition for L1-measurements in LTM is restricted to within BC. After a while, RAN2 has concluded the following in an informational LS to RAN4.

RAN2 understands that, when the UE indicates support of this capability for a BC and the UE is currently operating in this BC (or a subset of it), the SSBs of candidate cells on which the UE can perform inter-frequency L1 measurements according to this UE capabilities can be in any band on which the UE supports LTM. For example, if the UE has serving cells in bands A and B, indicates support of inter-frequency L1 measurements in a BC (A, B, C) and supports LTM in bands A, B, C, D, E, F and G, the UE can perform inter-frequency L1 measurements of SSBs of candidate cells in bands A, B, C, D, E, F and G. RAN2 agreed to introduce a new UE capability to allow the UE to indicate, per UE, that the SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements. interFreqL1-OnlyInBC-r18 When included, indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC i.e., the UE does not support L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) out of the BC.; UE; No; No; No.

From our point of view, the capability description is clear enough, and it can be reflected in the LTM inter-frequency and with and without gap 38.133. To make requirements clear, one option is to reflect it in the 9.15 section

Text proposal for section 9.15 included in our CR R4-2417682:

For UE supporting interFreqL1-OnlyInBC-r18, for each BC in which the UE indicates in interFreqL1-MeasConfig-r18 capability, the UE is only required to measure and report L1-RSRP SSB measurements configured for the indicated BC.

Reflect the interFreqL1-OnlyInBC-r18 capability in the 38.133 requirements for inter-frequency with and without gap

PDCCH ordered RACH delay

In the current PDCCH ordered RACH delay requirements, TSSB > 0 if the time from receiving the TCI state activation MAC-CE is longer than 160 ms. We think this should be aligned with the cell switch delay requirement where the time is 160 ms from completing the TCI state activation.

In PDCCH ordered RACH delay, TSSB = 0 for FR1 if the time from reception of TCI state activation MAC-CE is not longer than TCI state activation delay + 160 ms.

Early TCI state activation

Activation of TCI states for multiple cells

Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time < Agreement>: - Interested companies can have further offline discussion. If no further progress, the following will be agreed in the Nov meeting. - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

In the RAN4#112 meeting RAN4 discussed whether to define early TCI state activation delay requirements for the case when network would send more than one TCI state activation command in one slot, to activate TCI state(s) for multiple candidate cells. RAN4 tried to find a limited scenario where this requirement would apply, but was not able to find consensus, and hence nothing was agreed. To be able to wrap up Rel-18 work, we think it is preferable to not define TCI state activation delay requirements for the case when more than one candidate cell TCI state activation MAC-CE is received in the same slot, unless RAN4 can agree some reasonable scenario where this would apply.

RAN4 not to define requirements for activation of TCI states for more than one candidate cell in the same slot in Rel-18, unless a reasonable scenario can be agreed by the group.

Moreover, we would like the group to still discuss the requirements for early TCI state activation and especially the known TCI state condition when TCI state is activated for more than one cell in separate slots. As per the current requirements, TCI state is known if there was a L1-RSRP report within 1280 ms before the TCI state activation command. However, at the time of TCI state activation for one candidate cell, the UE is allowed to continue L1 measurements only on the candidate cell with active TCI states. If the network after this sends a TCI state activation command for another candidate cell, the UE may not be measuring this candidate cell. Under the current known TCI state conditions, such TCI state is known if the TCI state activation command is received within 1280 ms from the last L1 report for the candidate cell, even if the UE may not be measuring the cell anymore. We would like RAN4 to confirm that also in this case the TCI state is indeed considered known if the UE reported L1-RSRP within 1280 ms.

A target TCI state in TCI state activation command is known if the UE has sent an L1-RSRP report for the RS of the TCI state within 1280 ms (i.e. under current known TCI state conditions), even if the UE is not required to measure the target cell at the time of TCI state activation due to an earlier TCI state activation for another candidate cell.

No specification change is needed if Proposal 3 is agreeable to the group.

Cell switch delay

PL-RS

In RAN4#112 meeting it was agreed that additional time is not needed for PL-RS measurement during the cell switch under the following conditions:

1.4.2 Conditions of no extra time for PL-RS measurement in cell switch delay Issue 1-4-2-1: Conditions of no extra time for PL-RS measurement in cell switch delay <Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. - Further discuss the wording in the CR next meeting.

However, the above agreement was not captured in the specification yet. We have captured our text proposal in our CR.

Another open issue is related to the implications of this agreement on PL-RS measurement maintenance. At the RAN1 #118bis meeting, the topic of the maximum number of PL-RSs to be maintained for LTM candidate cells was discussed. It was noted that, in the serving cell, as per the RAN1 specification (section 7, 38.213), the UE is not expected to maintain more than four pathloss estimates simultaneously per serving cell for all PUSCH/PUCCH/SRS transmissions. However, consensus was not reached in RAN1 for LTM due to differing interpretations of the above RAN4 agreement.

Some companies in RAN1 believe that this agreement does not affect PL-RS measurement maintenance, and that similar constraints to those of the serving cell may apply to PL-RS measurement maintenance across LTM candidate cells before the reception of the cell switch command. Another interpretation of the agreement is that if the UE performs an L3-RSRP or L1-RSRP measurement on the SSB associated with the PL-RS and reports it, no additional measurements or constraints are needed to maintain the corresponding pathloss estimate, i.e., the UE is expected to retain the PL-RS information.

Therefore, it would be helpful for RAN4 to clarify the intent of this agreement regarding PL-RS measurement maintenance.

Proposal: RAN4 to further clarify the intent of the agreement regarding PL-RS measurement maintenance.

Tfirst-RS

Issue 1-4-1-2: Conditions of Tfirst-RS =0 in cell switch delay <Agreement>: - FFS the following assumption - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if the time gap between early RACH transmission and cell switch command is larger than [160ms] and the L3 measurement interval is larger than 160 ms.

What was left open in the last meeting regarding Tfirst-RS = 0 conditions was whether to extend the condition of time between RACH preamble transmission and cell switch also to the case when L1 measurements are configured. As we have commented previously, in our view there is no need to limit this condition to the case without L1 measurements. No matter if the UE is doing DL synchronization before preamble transmission or not, the UE has to have DL sync when it sends the preamble, so the UE may as well have it 160 ms after the preamble transmission.

The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements.

Issue 1-4-1-1: T/F tracking when TRS as QCL source in cell switch delay Ad hoc agreement <Agreement>: - FFS whether Tfirst-ssb in cell switch delay requirements need to be updated, e.g. based on TRS.

Another open issue is whether to take into account TRS as QCL source in the case when Tfirst-RS != 0. In our view the UE can do the DL sync based on TRS or SSB, whichever comes first, so the requirement may be updated based on this thinking.

When Tfirst-RS != 0,  Tfirst-RS is the time to the first SSB or TRS transmission associated to the target DL TCI state.

T LTM_RRC-processing

Issue 1-4-3-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? <Agreement>: - Further discuss: - FFS: Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) [and/or] PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - FFS: This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion. - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Regarding the conditions for TLTM_RRC-processing = 0, the common understanding in the last RAN4 meeting seemed to be that the most recent TCI state activation commands and PDCCH orders before the cell switch are to be taken into account for early processing of candidate cell configurations. However, the group could not find consensus about UE behavior in case the network sends multiple TCI state activation commands or PDCCH orders in the same slot, and the number of candidate cell configurations to be pre-processed is exceeded. In this case it would not be clear which of the candidate cell configurations the UE shall pre-process.

As we discussed earlier in this paper, in our understanding the problem would occur in very limited scenarios, if at all. Hence, we would propose that RAN4 does not take into account this scenario in the TLTM_RRC-processing = 0 conditions.

No need to consider TCI state activation or PDCCH order for more than one cell in one slot in the TLTM_RRC-processing = 0 conditions.

However, we still have the problem that if network does not activate TCI states or send PDCCH order, or the UE does not support these, and the number of candidate cell configurations exceeds UE capability to pre-process, it is unclear which candidate configurations the UE will pre-process (if any). Hence, the cell switch delay is unclear in this scenario, and we think RAN4 should aim to clarify what the UE is required to do.

Some additional signaling would solve this scenario, but this would require additional RAN2 work, which we would prefer to avoid in maintenance phase. Hence, we would propose that at the time of receiving the LTM RRC configuration, the UE pre-processes the maxNumberConfigs-r18 candidate configurations that are listed first in the LTM candidate cell RRC configuration. After this, TCI state activation or PDCCH order (if received) should be taken into account and the UE shall pre-process the candidate configurations based on these conditions.

If the number of configured candidate cells is larger than UE capability to pre-process, the UE can pre-process the first maxNumberConfigs-r18 candidate cell configurations in the LTM RRC configuration.

Putting this discussion together, we propose the following conditions for TLTM_RRC-processing = 0:

TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and:

-	The number of candidate cells in the LTM candidate cell configuration does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, or

-	The following conditions are fulfilled for the target cell:

-	The target cell is listed as one of the first Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration and the UE has not received TCI state activation command or PDCCH order for any of the candidate cells, or

-	The target cell is one of the most recent Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells for which TCI state is activated or PDCCH order is received, and

-	UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or

-	UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3].

Fast RRC processing LS reply to RAN2 R2-2409377

Related to the fast ASN.1 decoding capability, RAN2 has sent an LS to RAN4 with some questions:

According to the RAN4 feature list, two components are included in the LTM fast RRC processing capability ltm-FastProcessingConfig-r18: - maxNumberStoredConfigCells, indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. - maxNumberConfigs-r18, represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. Currently, only the number of LTM candidate configurations is visible in LTM-config, while the number of SpCell/SCell(s) in LTM candidate configurations is not visible until the UE decodes LTM candidate configurations. RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

The capability description is as follows:

And the supported numbers for the fast processing capability components are defined as:

3GPP TS 38.331 V18.3.0 (2024-09) -- R4 39-6: Fast processing of LTM candidate cell RRC configuration ltm-FastProcessingConfig-r18 SEQUENCE { maxNumberStoredConfigCells-r18 ENUM {n2,n3,n4,n5,n6,n7,n8,n9,n10,n11,n12, n16}, maxNumberConfigs-r18 INTEGER (1..4) }

The discussion in RAN2 has been about whether the number of SCells configured in an LTM candidate configuration impacts the UE capability to decode the candidate configuration. In RAN4, the discussion has been about the number of candidate configurations, and the number of cells in the candidate configuration has not been discussed as an impacting factor. In our understanding, the UE capability is about the number of candidate configurations regardless of whether the configuration includes SCells or not. Furthermore, RAN4 has not defined requirements for SCell activation in LTM, so the existing requirements do not even cover such case.

Hence, we propose the following responses to RAN2:

Reply to RAN2 questions in R2-2409377 as follows:

- Question 1 reply: Yes, this is the correct understanding.

- Question 2 reply: No, the UE does need to know. RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.

- Question 3 reply: Yes, it is up to the network to decide how many serving and candidate cells to configure. As long as the number of LTM candidate configurations and serving cells does not exceed maxNumberStoredConfigCells and the number of LTM candidate configurations does not exceed maxNumberConfigs, the UE shall be able to decode all LTM candidate configurations after receiving the RRC configuration

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: Reflect the interFreqL1-OnlyInBC-r18 capability in the 38.133 requirements for inter-frequency with and without gap

Proposal 2: In PDCCH ordered RACH delay, TSSB = 0 for FR1 if the time from reception of TCI state activation MAC-CE is not longer than TCI state activation delay + 160 ms.

Proposal 3: RAN4 not to define requirements for activation of TCI states for more than one candidate cell in the same slot in Rel-18, unless a reasonable scenario can be agreed by the group.

Proposal 4: A target TCI state in TCI state activation command is known if the UE has sent an L1-RSRP report for the RS of the TCI state within 1280 ms (i.e. under current known TCI state conditions), even if the UE is not required to measure the target cell at the time of TCI state activation due to an earlier TCI state activation for another candidate cell.

Proposal 5: Proposal: RAN4 to further clarify the intent of the agreement regarding PL-RS measurement maintenance.

Proposal 6: The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements.

Proposal 7: When Tfirst-RS != 0,  Tfirst-RS is the time to the first SSB or TRS transmission associated to the target DL TCI state.

Observation 1: No need to consider TCI state activation or PDCCH order for more than one cell in one slot in the TLTM_RRC-processing = 0 conditions.

Observation 2: If the number of configured candidate cells is larger than UE capability to pre-process, the UE can pre-process the first maxNumberConfigs-r18 candidate cell configurations in the LTM RRC configuration.

Proposal 8: TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and:

-	The number of candidate cells in the LTM candidate cell configuration does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, or

-	The following conditions are fulfilled for the target cell:

-	The target cell is listed as one of the first Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration and the UE has not received TCI state activation command or PDCCH order for any of the candidate cells, or

-	The target cell is one of the most recent Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells for which TCI state is activated or PDCCH order is received, and

-	UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or

-	UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3].

Proposal 9: Reply to RAN2 questions in R2-2409377 as follows:

	Question 1 reply: Yes, this is the correct understanding.

	Question 2 reply: No, the UE does need to know. RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.

	Question 3 reply: Yes, it is up to the network to decide how many serving and candidate cells to configure. As long as the number of LTM candidate configurations and serving cells does not exceed maxNumberStoredConfigCells and the number of LTM candidate configurations does not exceed maxNumberConfigs, the UE shall be able to decode all LTM candidate configurations after receiving the RRC configuration

- References

[1] R4-2413960, WF on Further NR mobility enhancements, MediaTek