3GPP RAN WG4 Meeting #113	R4-2417701

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on testability and interoperability issues for CSI compression

Source: 	CATT

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But there are still some issues related to testability and interoperability that need to be further discussed. In this contribution, we will continue to discuss those issues.

Discussion

Decoder selection criteria

In last meeting, another round of simulation results collection was performed and the results were aligned quite well after some pre-processing methods in [2] were adopted and considering the impact of different random seeds. And in next step, companies will share their encoder and decoder models among which a decoder will be selected. However, the criteria for decoder selection have not been specified yet. There are several options for the criteria which are copied below:

- Proposals - Option 1: Best performance (SGCS) and/or mean performance of corresponding eType II CSI feedback - Option 2: select multiple and work in parallel - few decoders can be selected and a subset of companies check feasibility on one decoder - Option 3: random selection, as long as performance is similar it does not matter which decoder is selected - Option 4: decoder trained with the highest number of sample - Option 5: To use an aggregated dataset of channel realizations/eigenvectors {V} for comparison of the performance of encoders/decoders provided by the companies. - Option 6: other criteria

Considering that the structure of decoder is the same and the simulation performances are well aligned, and the chosen decoder, which will not be captured in specification, is only for feasibility study of two-sided model test, it is acceptable to randomly select one from the submitted decoders. In regards with other options, option 5 will lead to extra simulation works and option 4 may not be a good criterion for model selection.

Proposal 1: RAN4 randomly selects a decoder(s) from the submitted decoders.

Reference encoder and/or training dataset

Some companies proposed to document a reference encoder and/or training dataset or Option 3 and they can be used for performance evaluation/verification or TE verification. In general, it is fine to document them since any meaningful outcomes during SI phase can be captured in TR and how this information is used depends on reader themselves. But considering the feasibility study of Option 3 is not finished yet, RAN4 can come back to this issue when the study is completed.

Observation 1: Any meaningful outcomes in SI phase can be documented in TR and how they will be used depends on readers.

Proposal 2: RAN4 decides whether to document reference encoder/training dataset or not after the study on Option 3 is completed.

Regarding the TE decoder verification/validation, evaluating the quality of TE implementations is not in RAN4 scope, and RAN4 should believe RAN5 have or will design mature procedures to ensure fair AI/ML-based tests.

Proposal 3: RAN4 not to verify the TE verification/validation.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Proposal 1: RAN4 randomly selects a decoder(s) from the submitted decoders.

Observation 1: Any meaningful outcomes in SI phase can be documented in TR and how they will be used depends on readers.

Proposal 2: RAN4 decides whether to document reference encoder/training dataset or not after the study on Option 3 is completed.

Proposal 3: RAN4 not to verify the TE verification/validation.

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.

[2] R4-2414447, Alignment of model training and data set parameters for AI/ML-based CSI compression, Nokia, RAN4#112.