3GPP RAN WG4 Meeting #113	R4-2417706

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on testability and interoperability issues for positioning

Source: 	CATT

Agenda item:	7.17.5.2

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. There were still some issues related to testability and interoperability for positioning accuracy enhancement that need to be further discussed [2]. In this contribution, we will present our views on those issues.

Discussion

Testing for case 1

In RAN4#112bis meeting, no agreements for case 1 were achieved and options are shown below:

- Proposals - Option 1: RAN4 to introduce tests for case 1 - ground truth can be determined through use of PRUs or GNSS - Option 2:RAN4 to postpone testing discussions until the requirement definition is better understood - Option 3: RAN4 will not define any tests for case 1

As we discussed in our companion contribution [3], UE may be requested to report the inferred position by network and corresponding performance requirements can be defined if it is proved feasible. After performance requirements are defined, test cases are required. A key issue to define test cases is how to obtain the ground truth of DUT. It is quite challenging in real deployment, while there are some solutions to achieve it in tests since TE can control many parameters, which is not applicable in real deployment. One straightforward method is to set reasonable coordinates of DUT and TRPs that transmit PRS resources, which makes the ground truth of DUT location known to TE, and the coordinates of TRPs can be signalled to DUT for positioning assistance. After DUT reports its inferred position, TE can compare the reported position with the ground truth and then judge if DUT passes the tests against the corresponding performance requirements. Another merit of this method is that DUT cannot cheat in tests because the ground truth is unknown to DUT. Then the main problem becomes how to design reasonable coordinates of DUT and TRPs in one test case, which requires further study.

Proposal 1: RAN4 to define test cases for case 1 if defining performance requirements is proved feasible.

Proposal 2: The ground truth can be set by TE in test. And RAN4 needs to study how to design reasonable coordinates of DUT and TRPs in one test case.

Testing for case 3a/3b

Considering the AI/ML models are deployed in gNB in case 3a and LMF in case 3b, no test cases need to be defined for case 3a/3b.

Proposal 3: RAN4 does not discuss testability aspects related to positioning case 3a/3b.

Conclusions

This contribution discussed some issues in regards with testability and interoperability for AI/ML enabled positioning accuracy enhancement and following proposals are provided:

Proposal 1: RAN4 to define test cases for case 1 if defining performance requirements is proved feasible.

Proposal 2: The ground truth can be set by TE in test. And RAN4 needs to study how to design reasonable coordinates of DUT and TRPs in one test case.

Proposal 3: RAN4 does not discuss testability aspects related to positioning case 3a/3b.

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.

[2] R4-2415231, Topic summary for [112-Bis][127] NR_AI/ML_air, Qualcomm Incorporated, RAN4#112bis.

[3] R4-2417705, Discussion on RRM core requirements for positioning, CATT, RAN4#113.