3GPP TSG-RAN WG4 Meeting # 113	             R4-2417712

# Orlando , US, Nov 18 – 22, 2024

Agenda Item:	5.23.1

Source: 	CATT

Title: 	Discussion on RRM core requirements maintenance for R18 LTM

Document for:	Discussion

1. Introduction

In RAN4 #112 meeting, WF on Further NR mobility enhancements was approved [1]. In this contribution, some open issues on RRM core requirements maintenance for further NR mobility enhancements are further discussed.

2. Discussion

2.1 Early Candidate cell’s TCI state activation

The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case

The WF on the definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case in RAN4 #112 meeting is duplicated as follows [1]:

< Way Forward > FFS the following options: - Option 1 (Apple, MTK, Ericsson): - remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements. - Option 2 (ZTE, vivo): - Confirm the FFS.

In our view, Tfirst-SSB_RACH is the time to the first SSB occasion, overlapped with MGL after 2ms and 1slot from the end of the slot of the PDCCH when SSB is outside active BWP, this is only applicable to UE supporting inter-frequency L1 measurement with MG, so ‘FFS’ in the spec can be removed.

Proposal 1: RAN4 to remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

2.2 Cell switch delay requirements for Pcell/PSCell

LS from RAN2

In the last meeting, the LS on the fast RRC processing for LTM was sent from RAN2 [3], and there were three questions for LTM fast RRC processing capability, the questions are copied as follows:

RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

MaxNumberStoredConfigCells indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations.

MaxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133.

For question 1, we think that RAN2’s understanding is right, and RAN4 can confirm RAN2 understanding in the reply LS. For For question 3, in our view, the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations can exceeds the UE reported maxNumberStoredConfigCells, if it exceeds the UE reported maxNumberStoredConfigCells, then UE will select the maximum number of cells that can be stored to perform fast RRC processing.

Proposal 2: RAN4 to confirm the understanding from RAN2 in question 1.

Proposal 3: RAN4 to reply RAN2 that it is possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells.

2.3 Improvement on SCell/SCG setup delay

More test for Improvement on SCell/SCG setup delay

In RAN4 #112 meeting, the Way Forward on test cases for Improvement on SCell/SCG setup delay is copied as follows [1]:

< Way Forward > FFS the following option: - Proposal 1 (Nokia): - Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

In our view, we can understand the motivation of proposal 1. Rel-16 EMR test case only can verify the capability of reporting valid report if T331 is running, but a new validity condition including time restriction is introduced in Rel-18, so Rel-16 EMR test case can’t verify the impact of new X seconds, it can only verify the UE behaviour of reporting measurement results that meet the valid conditions in R16 EMR. If only the R16 test case to verify R18 EMR UE behaviour, UE may only report the result before T331 expiring, so the capability to report the measurement during the X seconds may not be verified.

In current R18 test cases design, only a test case of verifying the EMR UE behaviour of not reporting the invalid measurement before X seconds. Considering that UE has already passed the test for R16 EMR, which indicates that not reporting any results is not caused by UE issues, it is a normal working device and has the capability to report measurement results. The measurements before X seconds are not reported only because the results did not meet the valid conditions defined in R18.

Observation 1: In current R18 test cases design, only a test case of verifying the EMR UE behaviour of not reporting the invalid measurement before X seconds.

Observation 2: If only the R16 test case to verify R18 EMR UE behaviour, UE may only report the result before T331 expiring, so the capability to report the measurement during the X seconds may not be verified.

Therefore, according to the above, adding a new test case just extend T331 to longer than X seconds compared to legacy R16 EMR test case is ok for us. From another point of view, if the companies assume that no matter how the validity time of the measurement results is defined, the UE shall be able to complete a valid reporting because its reporting function has been verified to be good, then the current test cases design is also acceptable.

Proposal 4: RAN4 need to discuss if the understanding of ‘If a UE has already passed the test for R16 EMR and R18 EMR tests, no matter how the validity time of the measurement results is defined, the UE shall be able to complete valid reporting because its reporting function has been verified to be good’ is right.

Proposal 5: If the above understanding is assumed, then the current R18 test cases design is enough.

3. Conclusion

In this paper, we provide our views on RRM core requirements maintenance for further NR mobility enhancements. From this discussion we have derived the following observation and proposals:

Proposal 1: RAN4 to remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

Proposal 2: RAN4 to confirm the understanding from RAN2 in question 1.

Proposal 3: RAN4 to reply RAN2 that it is possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells.

Proposal 4: RAN4 need to discuss if the understanding of ‘If a UE has already passed the test for R16 EMR and R18 EMR tests, no matter how the validity time of the measurement results is defined, the UE shall be able to complete valid reporting because its reporting function has been verified to be good’ is right.

Proposal 5: If the above understanding is assumed, then the current R18 test cases design is enough.

4. Reference

- R4-2413960 WF on Further NR mobility enhancements, MediaTek Inc.

- TS 38.133 NR Requirements for support of radio resource management (V18.7.0)

- R4-2417509 LS on the fast RRC processing for LTM, RAN2