3GPP TSG-RAN WG4 Meeting #113	R4-2417715

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on less than 5MHz for NTN RRM requirement

Source: 	CATT

Agenda item:	7.8.3.4

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh was approved and many agreements were reached, but there still are some issues left [1].

This document will further discuss the remaining RRM issues about less than 5MHz for NTN, and give our understanding and proposals.

- Discussion

The feature of supporting less than 5MHz for NR will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. Therefore, as a baseline, RAN4 should define the related RRM requirement based on NTN requirements. This objective is to extend NR-NTN by considering support for dedicated spectrum less than 5 MHz for FR1.

2.2 Handover

### HO delay

In the last meeting, RAN4 agreed to extend T∆ in HO requirement for NTN in exiting TS 38.133, but the specific value was still in square brackets. The agreements are copied as below [1]:

Agreements - T∆ in HO requirement for NTN in exiting TS38.133 clause 6.1C.1.2. shall be extended to [3 Trs]

In our view, T∆ was extended to 3 Trs in TN with less than 5MHz, and the extension is due to the reduction of SSB bandwidth. Therefore, we believe that the extension shall not be changed by the scenario.

Proposal 1: T∆ in HO requirement for NTN in exiting TS 38.133 clause 6.1C.1.2. shall be extended to 3 Trs.

### CHO delay

In the last meeting, RAN4 discussed CHO delay and agreed to extend T∆ in CHO without L3 measurement requirement for NTN in exiting TS 38.133, but CHO with L3 measurement requirement were still FFS. The agreements and Way-forward are copied as follows [1]:

Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

Firstly, the specific value of extension for T∆ in CHO requirement was still in square brackets, according to the same reason in the above HO delay requirements, we prefer to reuse the legacy value  of extension for T∆ in TN with less than 5MHz, that is 3 Trs.

Proposal 2: For CHO without and with L3 measurement, T∆ in CHO requirement for NTN in exiting TS 38.133 shall be extended to 3 Trs.

In addition, for CHO with L3 measurement, the extension for Tmeasure also should be considered. Since Tmeasure was related to Tidentify_intra/inter_with_index in NTN CHO requirements, so the extension for Tmeasure will refer to the extension defined for Tidentify_intra/inter_with_index for a target cell with 12 PRB SSB.

Proposal 3: For CHO with L3 measurement, Tmeasure n CHO requirement for NTN in exiting TS 38.133 will be extended and the extension will be the same as the extension in Tidentify_intra/inter_with_index.

### Satellite switch time requirement

In the last meeting, RAN4 agreed to extend T∆ in Tinterrupt for SAN satellite switch time requirements in exiting TS38.133, but the specific value was still in square brackets. The agreements are copied as below [1]:

Agreements - T∆ in Tinterrupt for SAN satellite switch time requirements in existing TS38.133 6.1C.3 can be extended to [3 Trs]

For satellite switch time requirements, according to the same reason in the above HO/CHO delay requirements, we prefer to reuse the legacy value  of extension for T∆ in Tinterrupt with less than 5MHz, that is 3 Trs.

Proposal 4: For satellite switch time requirements, T∆ in Tinterrupt for SAN satellite switch time requirements in existing TS 38.133 6.1C.3 can be extended to 3 Trs.

2.3 Measurement Procedure

### How to define the new requirements for SSB time index detection

In the last meeting, RAN4 agreed to extend the number of SSB samples for Rel-18 NTN measurement requirements for intra-frequency and inter-frequency cases respectively, but the values of  X and Y were still FFS. The Agreements and Wayforward are copied as follows [3]:

Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

For how to extend the number of SSB samples in Tidentify_intra/inter_with_index for a target cell with 12 PRB SSB for NTN, we believe that the same values for X and Y can be followed as TN with less than 5MHz. That is, Tidentify_intra_with_index in intra-frequency measurement requirements will be extended by 4 more SSB samples (X = 4), and Tidentify_inter_with_index in inter-frequency measurement requirements will be extended by 3 more SSB samples (Y = 3).

In TN with less than 5MHz, the extension on the number of samples is based on the simulation result, and it is just due to the reduction of SSB bandwidth not related to scenario. Therefore, we believe that the extension shall not be changed by the scenario. If the values change, more simulation will be needed, and we don’t expect more simulation.

In addition, some companies think that SSB periodicity can be further extended in Rel-19 RAN2-led NTN phase 3 WI, we think that the two WIs should not be discussed in parallel at present.

      - Proposal 5: The same values for X and Y can be followed as TN with less than 5MHz:

- For intra-frequency measurement, Tidentify_intra_with_index will be extended by 4 more SSB samples (X = 4).

- For inter-frequency measurement, Tidentify_inter_with_index will be extended by 3 more SSB samples (Y = 3).

Observation 1: If the extension of T∆ in HO requirement for NTN changes, the new simulation will be expected.

Proposal 6: Don’t support to discuss Rel-19 NTN with less than 5MHz WI and RAN2-led NTN phase 3 WI in parallel at present.

- Conclusion

This document discussed the issues for less than 5MHz for NR NTN RRM requirements and presented following observation and proposals:

Proposal 1: T∆ in HO requirement for NTN in exiting TS 38.133 clause 6.1C.1.2. shall be extended to 3 Trs.

Proposal 2: For CHO without and with L3 measurement, T∆ in CHO requirement for NTN in exiting TS 38.133 shall be extended to 3 Trs.

Proposal 3: For CHO with L3 measurement, Tmeasure n CHO requirement for NTN in exiting TS 38.133 will be extended and the extension will be the same as the extension in Tidentify_intra/inter_with_index.

Proposal 4: For satellite switch time requirements, T∆ in Tinterrupt for SAN satellite switch time requirements in existing TS 38.133 6.1C.3 can be extended to 3 Trs.

      - Proposal 5: The same values for X and Y can be followed as TN with less than 5MHz:

- For intra-frequency measurement, Tidentify_intra_with_index will be extended by 4 more SSB samples (X = 4).

- For inter-frequency measurement, Tidentify_inter_with_index will be extended by 3 more SSB samples (Y = 3).

Observation 1: If the extension of T∆ in HO requirement for NTN changes, the new simulation will be expected.

Proposal 6: Don’t support to discuss Rel-19 NTN with less than 5MHz WI and RAN2-led NTN phase 3 WI in parallel at present.

- Reference

[1]      R4-2416851, WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)

[2]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0