3GPP TSG-RAN WG4 Meeting #113	R4-2417717

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.24.2.1

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#105 [1].

This document will discuss the RRM issues about event triggered L1 measurement for LTM and give our understanding and proposals.

- Discussion

- 2.1 Requirements for reporting criteria per measurement category

In the last meeting, the candidate solutions for where to capture event triggered L1 report requirement are copied as below [3]:

- Candidate solutions: - Option 1: (MTK) - Define a separate capability for supported number of events triggered L1 measurement reporting. - Focus on SSB based L1-RSRP measurement for event triggered reporting at first. - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options: - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers. - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling. - Option 2: discuss and confirm in RAN4#112bis that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline. (Apple) Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells. Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5; 10; Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5; 1; Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5; 1; Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. DL RSTD Note 2,4,5; 1; DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34].. UE Rx-Tx Note 2,4,5; 1; UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34].. DL PRS-RSRP Note 2,4,5; 1; DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. DL PRS-RSRPP Note 2,4,5; 1; DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. SRS-RSRP Note 2,3,4,5; 1; SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16.. CLI-RSSI Note 2,3,4,5; 1; CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16.. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.. - Option 3: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” (OPPO)

Firstly, we support to introduce requirements of ‘Capabilities for Support of Event Triggering and Reporting Criteria’. Furthermore, for whether the requirements of reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell, we think that whether SS-RSRP and CSI-RSRP in current description include both L1-RSRP and L3 RSRP should be clarified. In our view, since SS-RSRP and CSI-RSRP do not emphasize specific measurement, we may not need to additionally emphasize and add L1-RSRP separately. From this point of view, the revisions in option 2 may be not needed for measurement category of intra-frequency and inter-frequency.

Proposal 1: RAN4 to introduce requirements of ‘Capabilities for Support of Event Triggering and Reporting Criteria’.

Proposal 2: For the requirements for reporting criteria per measurement category, whether ‘SS-RSRP and CSI-RSRP’ in current description include both L1-RSRP and L3 RSRP should be clarified.

Observation 1: SS-RSRP and CSI-RSRP in current description do not emphasize specific measurement.

Proposal 3: RAN4 may not need to additionally emphasize and add L1-RSRP separately for measurement category of intra-frequency and inter-frequency.

- 2.2 Filtering assumption event triggered L1 report

In RAN4#112 meeting, RAN4 discussed the RRM impacts caused by filtering assumption, the related discussion are under discussion in RAN1/2, and companies were still controversial about the type of filtering. The candidate solutions in the last meeting are copied as follows [3]:

- Candidate solutions: - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT). - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC) - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected. - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline. - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

As we know, the filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report not for event evaluation. However, there is still no conclusion on whether to introduce such a L1 filtering in RAN1.

Since the current measurement requirements defined in R18 LTM didn’t consider the any filtering, if L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Otherwise, there is no impact on RAN4.

Observation 2: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report.

Observation 3: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 4: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

- 2.3 Beam level or cell level measurement result for new LTM event evaluation

In the last meeting, the candidate solutions for beam level or cell level measurement result for new LTM event evaluation are copied as below [3]:

- Candidate solutions: - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

In RAN2 #127 meeting, it was agreed that beam level measurement result, not cell level measurement result, is used LTM event evaluation. Therefore, we think that RAN4 need to follow RAN2 agreements to only use beam level measurement result for LTM event evaluations.

Proposal 5: For LTM event triggered L1 reporting, beam level measurement result will be used for LTM event evaluation.

- 2.4 L1 event triggered reporting delay

In the last meeting, RAN4 agreed to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements. The candidate solutions for L1 event triggered reporting delay are copied as below [3]:

- Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define ; (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

In our view, the existing framework in L3 event triggered reporting RRM requirements can be as a starting point. In legacy requirements, for the event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify with index or T identify without index defined in clause 9.2.5.1 or clause 9.2.6.2, the Tidentify _with _index is the sum of TPSS/SSS_sync , T SSB_measurement_period  and TSSB_time_index, and the Tidentify _without_index is the sum of TPSS/SSS_sync and T SSB_measurement_period. For LTM, since only known cells are considered, so only TL1-RSRP_Measurement_Period_SSB and TSSB_time_index need to be considered.

Therefore, for SSB based event triggered L1 measurement reporting, it is proposed that the delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index, which are defined for Rel-18 LTM.

For CSI-RS based event triggered L1 measurement reporting, the delay requirements should be defined according to the gNB triggered CSI-RS based L1 measurement requirements.

Proposal 6: the existing framework in L3 event triggered reporting RRM requirements can be as a starting point.

Proposal 7: For SSB based event triggered L1 measurement reporting, it is proposed that the delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index for intra and inter frequency measurement, which are defined for Rel-18 LTM.

Proposal 8: For CSI-RS based event triggered L1 measurement reporting, the delay requirements should be defined according to the gNB triggered CSI-RS based L1 measurement requirements.

- 2.5 Where to capture event triggered L1 report requirement

In the last meeting, the candidate solutions for where to capture event triggered L1 report requirement are copied as below [3]:

- Candidate solutions: - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

In our view, according to the current framework of the spec for LTM in R18 [4], we agree that event triggered reporting for LTM L1 measurement requirements can to be defined in clause 9.14.3 and clause 9.15.3.

Proposal 9: The event triggered reporting for LTM L1 measurement requirements can to be defined in clause 9.14.3 and clause 9.15.3.

- Conclusion

This document discussed preliminary the issues for event triggered L1 measurement for LTM and the following proposals and observations are presented:

Proposal 1: RAN4 to introduce requirements of ‘Capabilities for Support of Event Triggering and Reporting Criteria’.

Proposal 2: For the requirements for reporting criteria per measurement category, whether ‘SS-RSRP and CSI-RSRP’ in current description include both L1-RSRP and L3 RSRP should be clarified.

Observation 1: SS-RSRP and CSI-RSRP in current description do not emphasize specific measurement.

Proposal 3: RAN4 may not need to additionally emphasize and add L1-RSRP separately for measurement category of intra-frequency and inter-frequency.

Observation 2: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report.

Observation 3: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 4: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

Proposal 5: For LTM event triggered L1 reporting, beam level measurement result will be used for LTM event evaluation.

Proposal 6: the existing framework in L3 event triggered reporting RRM requirements can be as a starting point.

Proposal 7: For SSB based event triggered L1 measurement reporting, it is proposed that the delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index for intra and inter frequency measurement, which are defined for Rel-18 LTM.

Proposal 8: For CSI-RS based event triggered L1 measurement reporting, the delay requirements should be defined according to the gNB triggered CSI-RS based L1 measurement requirements.

Proposal 9: The event triggered reporting for LTM L1 measurement requirements can to be defined in clause 9.14.3 and clause 9.15.3.

- Reference

[1]	RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]      RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[3]      R4-2416862 WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0