3GPP TSG-RAN WG4 Meeting #113	R4-2417718

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on CSI-RS based L1 measurement for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.24.2.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#105 [2].

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part1 was approved [3]. This document will discuss the RRM issues about CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

- 2.1 Known cell definition in CSI-RS based L1 measurement

In the last meeting, RAN4 discussed known cell definition in CSI-RS based L1 measurement, but there are some remaining issues that need further discussions. The agreements for known cell definition in CSI-RS based L1 measurement are copied as follows [3]:

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

For the first condition, since only SSB was considered in R18 LTM, so it is not emphasized that L3 measurement is based on SSB. However, both SSB and CSI-RS are considered for R19 LTM, so CSI-RS based L3 measurement will not be excluded.

For third condition, we think that the difference of the defination between detectable and measurable need to be further clarified. As far as we know, the defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Proposal 1: For the first condition, CSI-RS based L3 measurement will not be excluded.

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Proposal 2: For third condition, the difference of the defination between detectable and measurable need to be further clarified.

- 2.2 CSI-RS based L1-SINR measurement

In the last meeting, the candidate solutions for CSI-RS based L1-SINR measurement are copied as follows [3]:

- Candidate solutions: - Option 1: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement. (CATT) - RAN4 to wait for more RAN1 input. - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement (Apple, Nokia)

In RAN1#118-bis meeting, the measurement metrics for CSI-RS based L1 measurement has been discussed, and RAN1 had no consensus on the support L1-SINR measurement based on CSI-RS for candidate cells. The conclusions in RAN1 are copied as follows:

Conclusion (RAN1#118bis) - There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

According to the above, RAN4 also doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

Observation 2: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

Proposal 3: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

- 2.3 CSI-RS based L1 measurement outside active BWP or with measurement gap

In the last meeting, RAN4 discussed whether CSI-RS based L1 measurement can be outside active BWP, but there was no consensus among the companies. The candidate solutions are copied as below [3]:

- Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

As far as we know, the bandwidth of SSB is fixed, and the transmission bandwidth of the actual system may be much larger than that of SSB, and the measurements based on SSB may not be able to truly reflect the quality of the wireless link of DL outside the transmission bandwidth of SSB, which is one of the main reasons for the introduction of CSI-RS based L3 measurement.

For CSI-RS based L1 measurement, if RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited. Generally speaking, we think that CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario.

Observation 3: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Proposal 4: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

- 2.4 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

- 2.5 Category of CSI-RS based L1 RSRP measurement

In the last meeting, RAN4 discussed whether and how to have a definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, but there was no consensus among the companies. Some companies supported to reuse the definition of intra-frequency and inter-frequency for CSI-RS based L3 measurement as legacy, other companies believed that the center frequency of the CSI-RS resource of the serving cell in the legacy definition is not clear for CSI-RS based L1 measurement. The candidate solutions are copied as below [3]:

- Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

In our view, from the point of forward compatibility and backward compatibility, having a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement is better.

Proposal 5: From the point of forward compatibility and backward compatibility, suggest to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement.

In legacy, CSI-RS for mobility was supported for L3 measurement, and the CSI-RS resource is configured based on the frequency in Serving MO, so the centre frequency of the CSI-RS resource of the serving cell is clear. For CSI-RS based L1-RSRP measurement, there will be multiple CSI-RS resources in the serving cell and multiple centre frequencies of CSI-RS resources of the serving cell, so the reference centre frequency of the CSI-RS resource in the legacy definition of intra- and inter-frequency is not clear.

Observation 4: For CSI-RS based L1-RSRP measurement, there may be multiple CSI-RS resources of the serving cell and multiple centre frequencies of CSI-RS resources in the serving cell, so the reference centre frequency of the CSI-RS resource in the legacy definition of intra- and inter-frequency is not clear.

Therefore, the legacy definition of intra-frequency and inter-frequency for CSI-RS based L3 measurement may not be reused directly. RAN4 need to futher clarify the reference centre frequency of CSI-RS resources in the definition, and one possible way is to clarify that the reference centre frequency can be any centre frequency of the CSI-RS resources in the serving cell based on the legacy definition.

Proposal 6: The legacy definition of intra-frequency and inter-frequency for CSI-RS based L3 measurement may not be reused directly, and RAN4 need to futher clarify the reference centre frequency of CSI-RS resources in the definition.

- One possible way is to clarify that the reference centre frequency can be any centre frequency of the CSI-RS resources in the serving cell based on the legacy definition.

In addition, since definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement was still FFS in the last meeting, the category of CSI-RS based L1 RSRP measurement has not been determined. If the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement has been defined, it is naturally to categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency.

Proposal 7: If the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement has been defined, it is naturally to categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency.

- 2.6 L1 measurement on CSI-RS with repetition ON/OFF

In the last meeting, the candidate solutions for L1 measurement on CSI-RS with repetition ON and OFF are copied as below [3]:

L1 measurement on CSI-RS with repetition ON - Candidate solutions - Option 1: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. (MTK) - Option 1a: RAN4 to not consider P3-like early beam management in CSI-RS based L1-RSRP measurement requirement definition, i.e. no Rx beam sweeping factor, assuming the CSI-RS is not configured with ‘repetition = on.’ (QC) - P1 (wide gNB Tx & wide UE Rx): SSB based BM (implicit/explicit SSB-ID report to gNB) - P2 (narrow gNB Tx & wide UE Rx): On CSI-RS resources with ‘repetition = off’, UE applies a fixed Rx beam obtained from P1 and measures/reports CRI and L1-RSRP. - P3(narrow gNB Tx & narrow UE Rx): On CSI-RS resources with ‘repetition = on’, UE sweeps its Rx beam with a finer beamwidth, desirably confined within the P1/P2 Rx beamwidth. No report is required. - Option 2: If the repetition is ON, UE can receive the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping directly. (CTC, CATT) L1 measurement on CSI-RS with repetition OFF - Candidate solutions - Option 1: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK) - Option 1a: RAN4 to define the L1 measurement requirements for CSI-RS with repetition OFF in FR2 for the following procedure (CATT) - UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement - UE is capable of supporting R18 LTM. - Option 2: If the repetition is OFF, UE needs to perform measurement on the associated SSB firstly, but we do not capture the significant benefit of second situation. (CTC)

Firstly, we want to clarify that what do CSI-RS repetition ON and OFF mean. According to TS 38.214, this parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Specifically, if the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, and the gNB will use a wide beam.

Observation 5: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 6: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 7: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

In FR1, there is no need to consider beam sweeping. UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

Proposal 8: In FR1, UE shall firstly perform SSB based L3 measurement on candidate neighbour cells, then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

In FR2, three typical cases can be analyzed:

For P1 case, wide beam will be used for gNB Tx and wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, SSB based BM is used and rough beam is used for UE.

For P2 case, narrow beam will be used for gNB Tx and fixed wide beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘OFF’, then UE applies a fixed Rx beam obtained from SSB based L3 measurement.

For P3 case, fixed narrow beam will be used for gNB Tx and narrow beam will be used for UE Rx, and the CSI-RS repetition is configured with ‘ON’, then UE sweeps its Rx beam with a finer beamwidth, we can at least agree to not support this case for neighbor cell.

Therefore, RAN4 will not to define the requirements for CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell. Considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam as P2, so RAN4 can define the requirements for the following case in FR2: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 9: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 10: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

- 2.7 CSI-RS resource type

In the last meeting, the candidate solution for CSI-RS resource type is copied as below [3]:

- Candidate solutions: - Option 1: RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS. (vivo)

This issue is related to RAN1 progress, and the working assumption agreed in RAN1#118bis meeting is copied as follows:

Working Assumption (Agreement (RAN1#118bis) - In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). - Support of semi-persistent CSI-RS is subject to UE capability. - Final LS in R1-2409283.

According to the above, in addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability. Therefore, in addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells.

Observation 8: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 11: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells.

- 2.8 Measurement gap related issues

In Rel-18, RAN4 only define the requirement for inter-frequency L1-RSRP measurement with type 1 MG. In Rel-19, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement. In our view, at least the type 1 MG should be supported.

Proposal 12: In Rel-19 LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

In the last meeting, for the case of gap-based measurement, some companies were concerned about whether the existing gap can cover the CSI-RS configuration in the same frequency layer. In our view, the same problem exists in the legacy CSI-RS based L3 measurement, and the CSI-RS configuration is also flexible. RAN4 defined the requirements applicability for CSI-RS based L3 measurement and used time window to limit CSI-RS resourse transmission, the details in clause 9.10 of TS 38.133 are copied as follows [4]:

#### 9.10.3.2 Requirements applicability The associated SSB layer of the CSI-RS follows the same requirements as SSB based measurements defined in 9.3. The requirements in clause 9.10.3 apply, provided: - The associated SSB of the cell being identified or measured is detectable, and - All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and - The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and - CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

Therefore, at the very least, RAN4 can follow the same approach as CSI-RS based L3 measurement to address companies' concerns, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Observation 9: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 13: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Another feasible approach is to introduce related gap enhancements. In R18 LTM, RAN4 just supported R16 type 1 gap as a baseline. Considering that many gap enhancements have been specified in Gap WI, which also can be used for CSI-RS based L1 measurement in R19 LTM. If RAN4 to introduce the concurrent gap, the above issue can also be resolved.

In additon, according to RAN1 conclusions on the CSI-RS resource type, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability. If RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

Observation 10:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 14: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

- Conclusion

This document discussed preliminary the issues for CSI-RS based L1 measurement for LTM in Rel-19 and the following proposals and observations are presented:

Proposal 1: For the first condition, CSI-RS based L3 measurement will not be excluded.

Observation 1: The defination of ’detectable’ exists in current spec, but there is no defination of ’measurable’ defined in spec.

Proposal 2: For third condition, the difference of the defination between detectable and measurable need to be further clarified.

Observation 2: There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

Proposal 3: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement.

Observation 3: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Proposal 4: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM.

Proposal 5: From the point of forward compatibility and backward compatibility, suggest to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement.

Observation 4: For CSI-RS based L1-RSRP measurement, there may be multiple CSI-RS resources of the serving cell and multiple centre frequencies of CSI-RS resources in the serving cell, so the reference centre frequency of the CSI-RS resource in the legacy definition of intra- and inter-frequency is not clear.

Proposal 6: The legacy definition of intra-frequency and inter-frequency for CSI-RS based L3 measurement may not be reused directly, and RAN4 need to futher clarify the reference centre frequency of CSI-RS resources in the definition.

- One possible way is to clarify that the reference centre frequency can be any centre frequency of the CSI-RS resources in the serving cell based on the legacy definition.

Proposal 7: If the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement has been defined, it is naturally to categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency.

Observation 5: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side not from UE side.

Observation 6: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction.

Observation 7: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping.

Proposal 8: In FR1, UE shall firstly perform SSB based L3 measurement on candidate neighbour cells, then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

Proposal 9: In FR2, RAN4 will not to define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell.

Proposal 10: In FR2, RAN4 will define the requirements for the following case:

- CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB.

Observation 8: In addition to periodic CSI-RS, RAN1 has agreed to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting, and the semi-persistent CSI-RS is subject to UE capability.

Proposal 11: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells.

Proposal 12: In Rel-19 LTM, at least the type 1 MG should be supported for CSI-RS based L1 measurement.

Observation 9: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 13: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., requirements applicability for CSI-RS based L1 measurement will be defined to ensure existing gap can cover the CSI-RS configuration in the same frequency layer.

Observation 10:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 14: For CSI-RS based L1 with gap measurement, if RAN4 also agree to consider semi-persistent CSI-RS transmission and measurements, may be pre-MG can be introduced for semi-persistent CSI-RS measurements.

- Reference

[1]	RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]      RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[3]      R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.7.0